3GPP TSG-SA5 Meeting #126	S5-195381

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0027 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Analytical KPI of Traffic Load Level |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Traffic load level analytics is useful for network operators to monitor the load information considering both the user numbers and the data transmission of the active users for each cell. The capability of the gNB is also considered to evaluate whether the cell of the gNB is overload or not. This KPI is also helpful for network operators to configure the mobility parameters to perform the load balance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add analytical KPI of traffic load level. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The KPI of traffic load level will not be monitored. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, 6.x.1(new), A.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First of changes

4	End to end KPI concept and overview

The following KPI categories are included in the present document:

-	Accessibility (see the definition in [3]).

-	Integrity (see the definition in [3]).

-	Utilization

-	Analytics

For future update of the document it will also include:

-	Retainability (see the definition in [3]).

-	Availability.

-	Mobility.

Second of changes

6.X	Analytics KPI

6.X.1	Traffic Load Level

a)	The level of traffic load for NG-RAN.

b)	This KPI describes the load level of NG-RAN traffic. It is used to evaluate the comprehensive traffic load including not only the end user’ numbers but also the data transmission by active users and radio resources.

c)	This KPI is an integer which shows the comprehensive traffic load level. The KPI is derived from analyzing the measurements related with data volumes, active users, RRC connection numbers and resource utilizations. The range of the KPI values depends on the implementation.

d)	DRB.TrafficLoad= analytics (RRC.ConnMean, RRC.ConnMax, RRU.PrbTotDl, RRU.PrbTotUl, RRU.PrbTotDlDist.BinX, RRU.PrbTotUlDist.BinX, DRB.PdcpSduVolumeDL_Filter, DRB.PdcpSduVolumeUL_Filter)

e)	Measurement names used for traffic load analytics

| Performance Measurements Category | Input performance measurements | Source | Description |
| --- | --- | --- | --- |
| RRC connection number | RRC.ConnMean, RRC.ConnMax | PM data | The number of users in RRC connected mode, see clause 5.1.1.4 of TS 28.552. |
| Radio resource utilization | RRU.PrbTotDl, RRU.PrbTotUl, RRU.PrbTotDlDist.BinX, RRU.PrbTotUlDist.BinX | PM data | The usage of physical radio resource utilization of the radio network, see clause 5.1.1.2 of TS 28.552. |
| PDCP Data Volume | DRB.PdcpSduVolumeDL_Filter, DRB.PdcpSduVolumeUL_Filter | PM data | The number of PDCP data volume transmitted in downlink, see clause 5.1.2.1 and 5.1.3.6 of TS 28.552. |

f)	NG-RAN

g)	Analytics

h)	Time interval (microsecond)

i)	Mean

Third of changes

A.X	Use Case for Analytics KPI

Use case for Traffic Load Level

Traffic load level analytics is useful for network operators to monitor the load information considering both the user numbers and the data transmission of the active users for each cell. The capability of the gNB is also considered to evaluate whether the cell of the gNB is overload or not. This KPI is also helpful for network operators to configure the mobility parameters to perform the load balance.

End of changes