3GPP TSG-SA5 Meeting #126	S5-195396

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0051 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add RESTful HTTP-based solution set of file-based performance assurance for integration with ONAP VES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ONAP3GPP |  |  |  |  |  | Date: |  |  | 2019-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | ONAP DCAE VES Collector is the consumer of 3GPP perfoarmance data file report management service. It is required that a new RESTful HTTP-based solution set be specified for this purpose. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new RESTful HTTP-based solution set for enabling ONAP DCAE VES Collector to consume 3GPP performance data file report management service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.3.x (new), A.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

12.3.x RESTful HTTP-based solution set of file-based performance assurance for intergration with ONAP VES API

12.3.x.1 Mapping of operations

NOTE: no use case has been specified by ONAP. Therefore this mapping is not part of the present document.

12.3.x.2 Mapping of notifications

12.3.x.2.1 Introduction

12.3.X.2.1.1 General

The 3GPP IS notifications are mapped to SS equivalents according to table 12.3.x.2.1.1.-1.

Table 12.3.x.2.1.1-1: Mapping of 3GPP IS notifications to SS equivalents

| 3GPP IS notifications | HTTP Method | Resource URI | SQ |
| --- | --- | --- | --- |
| notifyFileReady | POST | /eventListener | M |
| notifyFilePreparationError | POST | /eventListener | M |

12.3.x.2.1.2 Notification parameter mapping principles

3GPP performance data file notification parameters are mapped to ONAP VES API file3gpp fields – see mapping tables in following clause 12.3.X.2.2 and 12.3.X.2.3.

12.3.x.2.2 Notificaiton notifyFileReady

The 3GPP IS notification parameters are mapped to SS equivalents according to table 12.3.x.2.1.2-1.

Table 12.3.x.2.2-1: Mapping of IS notification parameters to SS equivalents in ‘file3gppFields’

| 3GPP IS notification parameter name | SQ | SS parameter name in ‘file3gppFields’ – cf. A.x | SQ |
| --- | --- | --- | --- |
| objectClass, objectInstance | M | DN | M |
| notificationId | M | notificationId | M |
| eventTime | M | eventTime | M |
| notificationType | M | notificationType (value = ‘notifyFileReady’) | M |
| fileInfoList | M | fileInfoList | O |
| additionalText | O | additionalText | O |

12.3.x.2.3 Notification notifyFilePreparationError

The 3GPP IS notification parameters are mapped to SS equivalents according to table 12.3.x.2.1.2-1.

Table 12.3.x.2.3-1: Mapping of IS notification parameters to SS equivalents in ‘file3gppFields’

| 3GPP IS notification parameter name | SQ | SS parameter name in ‘file3gppFields’ – cf. A.x | SQ |
| --- | --- | --- | --- |
| objectClass, objectInstance | M | DN | M |
| notificationId | M | notificationId | M |
| eventTime | M | eventTime | M |
| notificationType | M | notificationType (value = ‘notifyFilePreparationError’) |  |
| fileInfoList | M | fileInfoList | M |
| reason | M | reason | M |
| additionalText | O | additionalText | M |

12.3.x.3 Resource

12.3.x.3.1 Resource structure

Figure 12.3.x.3.1-1 shows the resource structure of file-based perforamance assurance MnS in the context of its integration with ONAP DCAE VES Collector.

Figure 12.3.x.3.1-1: Resource URI structure of file-based perforamance assurance MnS for integration with ONAP DCAE VES

Table 12.3.x.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 12.3.x.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| eventListener | /eventListener | POST | Send notifications |

12.3.x.3.2 Resource definations

See [21]

12.3.x.4 Data type definitions

See clause 12.3.1.1.4.

2nd Change

A.3.x	JSON schema of ‘file3gppFields’ for integration with ONAP VES

{

"file3gppFields": {

"description": "fields specific to 3GPP performance data file report notifications for integration with ONAP VES",

"type": "object",

"properties": {

"file3gppFieldsVersion": {

"description": "version of the file3gppFields block",

"type": "string",

"enum": [

"16.0"

]

},

"performancedatafilevisionNotifications": {

"oneOf": [

{

"$ref": "#/definitions/notifyFileReady-NotifType"

},

{

"$ref": "#/definitions/notifyFilePreparationError-NotifType"

}

]

}

},

"definitions": {

"notifyFileReady-NotifType": {

"description": "fields specific to 3GPP notification NotifyFileReady",

"type": "object",

"properties": {

"dN": {

"$ref": "#/definitions/dN-Type"

},

"notificationId": {

"$ref": "#/definitions/notificationId-Type"

},

"eventTime": {

"$ref": "#/definitions/eventTime-Type"

},

"notificationType": {

"$ref": "#/definitions/notificationType-Type"

},

"fileInfoList": {

"$ref": "#/definitions/fileInfo-Type"

},

"additionalText": {

"$ref": "#/definitions/additionalText-Type"

}

}

},

"notifyFilePreparationError-NotifType": {

"description": "fields specific to 3GPP notification NotifyFilePreparationError",

"type": "object",

"properties": {

"dN": {

"$ref": "#/definitions/dN-Type"

},

"notificationId": {

"$ref": "#/definitions/notificationId-Type"

},

"eventTime": {

"$ref": "#/definitions/eventTime-Type"

},

"notificationType": {

"$ref": "#/definitions/notificationType-Type"

},

"fileInfoList": {

"$ref": "#/definitions/fileInfo-Type"

},

"reason": {

"$ref": "#/definitions/reason-Type"

},

"additionalText": {

"$ref": "#/definitions/additionalText-Type"

}

}

},

"dN-Type": {

"type": "string"

},

"notificationId": {

"type": "string"

},

"eventTime": {

"type": "string"

},

"notificationType-Type": {

"type": "string",

"enum": [

"notifyFileReady ",

"notifyFilePreparationError"

]

},

"fileInfo-Type": {

"type": "object",

"properties": {

"fileLocation": {

"$ref": "#/components/schemas/uri-Type"

},

"fileSize": {

"$ref": "#/components/schemas/long-Type"

},

"fileReadyTime": {

"$ref": "#/components/schemas/dateTime-Type"

},

"fileExpirationTime": {

"$ref": "#/components/schemas/dateTime-Type"

},

"fileCompression": {

"type": "string"

},

"fileFormat": {

"type": "string"

}

}

},

"reason-Type": {

"type": "string"

},

"additionalText-Type": {

"type": "string"

}

}

}

}

End of 2nd Change