3GPP TSG-SA5 Meeting #126 	S5-195422

# Bruges, Belgium 19-23 August 2019												Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0147 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | generate JSON definition for 5GC NRM based on new style guideline |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-5GNRM |  |  |  |  |  | Date: |  |  | 2019-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Align with JSON definition style guide and stage 2 definition |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change JSON definition for 5GC NRM |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The JSON definition is difficult to be implemented by operators and vendors |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, G.3, G.4.2, G.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Include nrNrm.json |  |  |  |  |  |  |  |  |

Start of 1st modification

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[6]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[7]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[8]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[9]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage 2".

[10]	3GPP TS 28.540: "Management and orchestration; 5G Network Resource Model (NRM);Stage 1".

[11]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

[12]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[13]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[14]	3GPP TS 36.410: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 general aspects and principles".

[15]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 application protocol".

[16]	3GPP TS 36.425: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 interface user plane protocol".

[17]	3GPP TS 28.625: "State Management Data Definition Integration Reference Point (IRP); Information Service (IS)".

[18]	ITU-T Recommendation X.731: "Information technology - Open Systems Interconnection - Systems Management: State management function".

[19]	3GPP TS 28.658: "Telecommunications management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[20]	3GPP TS 28.702: "Core Network (CN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[21]	3GPP TS 28.708: "Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[22]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[23]	3GPP TS 29.510: "5G system; Network Function Repository Services; Stage 3".

[24]	3GPP TS 29.531: "5G System; Network Slice Selection Services Stage 3".

[25]	Void.

[26]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[27]	3GPP TS 28.554: "Management and orchestration; 5G End to end Key Performance Indicators (KPI)".

[28]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[29]	ETSI GS NFV-IFA 013 V2.4.1 (2018-02) "Network Function Virtualisation (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[30]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[31]	Void.

[32]	3GPP TS 38.211: "NR; Physical channels and modulation".

[33]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[34]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[35]	3GPP TS 28.532: "Management and orchestration; Management services".

[36]	Void.

[37]	IETF RFC 791: "Internet Protocol".

[38]	IETF RFC 2373: "IP Version 6 Addressing Architecture".

[39]	IEEE 802.1Q: "Media Access Control Bridges and Virtual Bridged Local Area Networks".

[40]	ETSI GS NFV-IFA 015 (V2.4.1): "Network Function Virtualisation (NFV); Management and Orchestration; Report on NFV Information Model".

[41]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[42]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[43]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) model repertoire".

[x]	3GPP TS 32.160: "Management and orchestration; Management Service Template".

End of 1st modification

Start of 2nd modification

Annex G (normative):
JSON definitions for 5GC NRM

# G.1	General

This annex contains the JSON definitions for the 5GC NRM, in accordance with 5GC NRM information model definitions specified in clause 5.

# G.2	Architectural features

The overall architectural feature of 5GC NRM information model is specified in clause 5, this annex specifies features that are specific to the Schema definitions.

# G.3	Mapping

Refer to TS 32.160 [x].

# G.4	Solution Set definitions

## G.4.1	JSON definition structure

JSON is used as resource representations format carried in the HTTP request and HTTP response message bodies. The properties (key-value pairs) on an object are defined using the properties keyword.

## G.4.2	Graphical representation

None.

## G.4.3	JSON schema "ngcNrm.json"

{

"openapi": "3.0.1",

"info": {

"title": "3GPP 5GC NRM",

"version": "15.3.0",

"description": "OAS 3.0.1 specification compatible schema for 3GPP 5GC NRM"

},

"paths": {},

"components": {

"schemas": {

"AmfIdentifier": {

"type": "object",

"description": "AmfIdentifier comprise of amfRegionId, amfSetId and amfPointer",

"properties": {

"amfRegionId": {

"$ref": "#/components/schemas/AmfRegionId"

},

"amfSetId": {

"$ref": "#/components/schemas/AmfSetId"

},

"amfPointer": {

"$ref": "#/components/schemas/AmfPointer"

}

}

},

"AmfRegionId": {

"type": "integer",

"description": "AmfRegionId is defined in TS 23.003",

"maximum": 255

},

"AmfSetId": {

"type": "string",

"description": "AmfSetId is defined in TS 23.003",

"maximum": 1023

},

"AmfPointer": {

"type": "integer",

"description": "AmfPointer is defined in TS 23.003",

"maximum": 63

},

"IpEndPoint": {

"type": "object",

"properties": {

"ipv4Address": {

"$ref": "genericNrm.json#/components/schemas/Ipv4Addr"

},

"ipv6Address": {

"$ref": "genericNrm.json#/components/schemas/Ipv6Addr"

},

"ipv6Prefix": {

"$ref": "genericNrm.json#/components/schemas/Ipv6Prefix"

},

"transport": {

"$ref": "genericNrm.json#/components/schemas/TransportProtocol"

},

"port": {

"type": "integer"

}

}

},

"NFProfileList": {

"type": "array",

"description": "List of NF profile",

"items": {

"$ref": "#/components/schemas/NFProfile"

}

},

"NFProfile": {

"type": "object",

"description": "NF profile stored in NRF, defined in TS 29.510",

"properties": {

"nFInstanceId": {

"type": "string",

"description": "uuid of NF instance"

},

"nFType": {

"$ref": "#/components/schemas/NFType"

},

"nFStatus": {

"$ref": "#/components/schemas/NFStatus"

},

"plmn": {

"$ref": "nrNrm.json#/components/schemas/PlmnId"

},

"sNssais": {

"$ref": "nrNrm.json#/components/schemas/Snssai"

},

"fqdn": {

"$ref": "#/components/schemas/Fqdn"

},

"interPlmnFqdn": {

"$ref": "#/components/schemas/Fqdn"

},

"nfServices": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NFService"

}

}

}

},

"NFService": {

"type": "object",

"description": "NF Service is defined in TS 29.510",

"properties": {

"serviceInstanceId": {

"type": "string"

},

"serviceName": {

"type": "string"

},

"version": {

"type": "string"

},

"schema": {

"type": "string"

},

"fqdn": {

"$ref": "#/components/schemas/Fqdn"

},

"interPlmnFqdn": {

"$ref": "#/components/schemas/Fqdn"

},

"ipEndPoints": {

"type": "array",

"items": {

"$ref": "#/components/schemas/IpEndPoint"

}

},

"apiPrfix": {

"type": "string"

},

"allowedPlmns": {

"$ref": "nrNrm.json#/components/schemas/PlmnId"

},

"allowedNfTypes": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NFType"

}

},

"allowedNssais": {

"type": "array",

"items": {

"$ref": "nrNrm.json#/components/schemas/Snssai"

}

}

}

},

"NFType": {

"type": "string",

"description": " NF name defined in TS 23.501",

"enum": [

"NRF",

"UDM",

"AMF",

"SMF",

"AUSF",

"NEF",

"PCF",

"SMSF",

"NSSF",

"UDR",

"LMF",

"GMLC",

"5G_EIR",

"SEPP",

"UPF",

"N3IWF",

"AF",

"UDSF",

"DN"

]

},

"Fqdn": {

"type": "string"

},

"UdrInfo": {

"type": "object",

"properties": {

"supiRanges": {

"type": "array",

"items": {

"$ref": "#/components/schemas/SupiRange"

}

}

}

},

"SupiRange": {

"type": "object",

"properties": {

"start": {

"type": "string"

},

"end": {

"type": "string"

},

"pattern": {

"type": "string"

}

}

},

"NFStatus": {

"type": "string",

"description": "any of enumrated value",

"enum": [

"REGISTERED",

"SUSPENDED"

]

},

"NFRegistrationData": {

"type": "object",

"properties": {

"heartBeatTimer": {

"type": "integer"

},

"nfProfile": {

"$ref": "#/components/schemas/NFProfile"

}

}

},

"NSIIdList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NSIId"

}

},

"NSIId": {

"type": "string",

"description": "NSI Id is defined in TS 29.531"

},

"NrCellId": {

"type": "string"

},

"SBIFqdn": {

"$ref": "#/components/schemas/Fqdn"

},

"SBIServiceList": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NFService"

}

},

"TACList": {

"type": "array",

"items": {

"$ref": "nrNrm.json#/components/schemas/NrTac"

}

},

"WeightFactor": {

"type": "integer"

},

"AmfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"amfIdentifier": {

"$ref": "#/components/schemas/AmfIdentifier"

},

"sBIFqdn": {

"type": "string"

},

"sBIServiceList": {

"$ref": "#/components/schemas/SBIServiceList"

},

"weightFactor": {

"$ref": "#/components/schemas/WeightFactor"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

},

"amfSet": {

"$ref": "genericNrm.json#/components/schemas/Dn"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N2": {

"$ref": "#/components/schemas/EP_N2"

},

"EP_N8": {

"$ref": "#/components/schemas/EP_N8"

},

"EP_N11": {

"$ref": "#/components/schemas/EP_N11"

},

"EP_N12": {

"$ref": "#/components/schemas/EP_N12"

},

"EP_N14": {

"$ref": "#/components/schemas/EP_N14"

},

"EP_N15": {

"$ref": "#/components/schemas/EP_N15"

},

"EP_N17": {

"$ref": "#/components/schemas/EP_N17"

},

"EP_N20": {

"$ref": "#/components/schemas/EP_N20"

},

"EP_N22": {

"$ref": "#/components/schemas/EP_N22"

},

"EP_N26": {

"$ref": "#/components/schemas/EP_N26"

},

"EP_NLS": {

"$ref": "#/components/schemas/EP_NLS"

},

"EP_NLG": {

"$ref": "#/components/schemas/EP_NLG"

}

}

}

]

},

"AmfSet": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"nRTACList": {

"$ref": "#/components/schemas/TACList"

},

"amfSetId": {

"$ref": "#/components/schemas/AmfSetId"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

}

}

}

]

}

}

}

]

},

"AmfRegion": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"nRTACList": {

"$ref": "#/components/schemas/TACList"

},

"amfRegionId": {

"$ref": "#/components/schemas/AmfRegionId"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

}

}

}

]

}

}

}

]

},

"SmfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"nRTACList": {

"$ref": "#/components/schemas/TACList"

},

"sBIFqdn": {

"type": "string"

},

"sBIServiceList": {

"$ref": "#/components/schemas/SBIServiceList"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N4": {

"$ref": "#/components/schemas/EP_N4"

},

"EP_N7": {

"$ref": "#/components/schemas/EP_N7"

},

"EP_N10": {

"$ref": "#/components/schemas/EP_N10"

},

"EP_N11": {

"$ref": "#/components/schemas/EP_N11"

},

"EP_N16": {

"$ref": "#/components/schemas/EP_N16"

},

"EP_S5C": {

"$ref": "#/components/schemas/EP_S5C"

}

}

}

]

},

"UpfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"nRTACList": {

"$ref": "#/components/schemas/TACList"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N3": {

"$ref": "#/components/schemas/EP_N3"

},

"EP_N4": {

"$ref": "#/components/schemas/EP_N4"

},

"EP_N6": {

"$ref": "#/components/schemas/EP_N6"

},

"EP_N9": {

"$ref": "#/components/schemas/EP_N9"

},

"EP_S5U": {

"$ref": "#/components/schemas/EP_S5U"

}

}

}

]

},

"N3iwfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N3": {

"$ref": "#/components/schemas/EP_N3"

},

"EP_N4": {

"$ref": "#/components/schemas/EP_N4"

}

}

}

]

},

"PcfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBIServiceList": {

"$ref": "#/components/schemas/SBIServiceList"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N5": {

"$ref": "#/components/schemas/EP_N5"

},

"EP_N7": {

"$ref": "#/components/schemas/EP_N7"

},

"EP_N15": {

"$ref": "#/components/schemas/EP_N15"

},

"EP_N16": {

"$ref": "#/components/schemas/EP_N16"

},

"EP_Rx": {

"$ref": "#/components/schemas/EP_Rx"

}

}

}

]

},

"AusfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBIServiceList": {

"$ref": "#/components/schemas/SBIServiceList"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N12": {

"$ref": "#/components/schemas/EP_N12"

},

"EP_N13": {

"$ref": "#/components/schemas/EP_N13"

}

}

}

]

},

"UdmFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBIServiceList": {

"$ref": "#/components/schemas/SBIServiceList"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N8": {

"$ref": "#/components/schemas/EP_N8"

},

"EP_N10": {

"$ref": "#/components/schemas/EP_N10"

},

"EP_N13": {

"$ref": "#/components/schemas/EP_N13"

}

}

}

]

},

"UdrFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBIServiceList": {

"$ref": "#/components/schemas/SBIServiceList"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

}

}

}

]

}

}

}

]

},

"UdsfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBIServiceList": {

"$ref": "#/components/schemas/SBIServiceList"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

}

}

}

]

}

}

}

]

},

"NrfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"nSIIdList": {

"$ref": "#/components/schemas/NSIIdList"

},

"nFProfileList": {

"$ref": "#/components/schemas/NFProfileList"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N27": {

"$ref": "#/components/schemas/EP_N27"

}

}

}

]

},

"NssfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"nSIIdList": {

"$ref": "#/components/schemas/NSIIdList"

},

"nFProfileList": {

"$ref": "#/components/schemas/NFProfileList"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N22": {

"$ref": "#/components/schemas/EP_N22"

},

"EP_N31": {

"$ref": "#/components/schemas/EP_N31"

}

}

}

]

},

"SmsfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBIServiceList": {

"$ref": "#/components/schemas/SBIServiceList"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N20": {

"$ref": "#/components/schemas/EP_N20"

},

"EP_N21": {

"$ref": "#/components/schemas/EP_N21"

},

"EP_MAP_SMSC": {

"$ref": "#/components/schemas/EP_MAP_SMSC"

}

}

}

]

},

"LmfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_NLS": {

"$ref": "#/components/schemas/EP_NLS"

}

}

}

]

},

"NgeirFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBIServiceList": {

"$ref": "#/components/schemas/SBIServiceList"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N17": {

"$ref": "#/components/schemas/EP_N17"

}

}

}

]

},

"SeppFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

}

}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-ContainingObjects"

},

{

"type": "object",

"properties": {

"EP_N32": {

"$ref": "#/components/schemas/EP_N32"

}

}

}

]

},

"NwdafFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"sBIFqdn": {

"type": "string"

},

"sBIServiceList": {

"$ref": "#/components/schemas/SBIServiceList"

},

"snssaiList": {

"$ref": "nrNrm.json#/components/schemas/SnssaiList"

}

}

}

]

}

}

}

]

},

"ExternalAmfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

},

"amfIdentifier": {

"$ref": "#/components/schemas/AmfIdentifier"

}

}

}

]

}

}

}

]

},

"ExternalNrfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

}

}

}

]

}

}

}

]

},

"ExternalNssfFunction": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedFunction-Attributes"

},

{

"type": "object",

"properties": {

"plmnIdList": {

"$ref": "nrNrm.json#/components/schemas/PlmnIdList"

}

}

}

]

}

}

}

]

},

"ManagedElement-Single": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/ManagedElement-Attributes"

},

{

"type": "object",

"properties": {}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/ManagedElement-ContainingObjects"

},

{

"type": "object",

"properties": {

"AmfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/AmfFunction"

}

},

"SmfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/SmfFunction"

}

},

"UpfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/UpfFunction"

}

},

"N3iwfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/N3iwfFunction"

}

},

"PcfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/PcfFunction"

}

},

"AusfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/AusfFunction"

}

},

"UdmFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/UdmFunction"

}

},

"UdrFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/UdrFunction"

}

},

"UdsfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/UdsfFunction"

}

},

"NrfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NrfFunction"

}

},

"NssfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NssfFunction"

}

},

"SmsfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/SmsfFunction"

}

},

"LmfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/LmfFunction"

}

},

"NgeirFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NgeirFunction"

}

},

"SeppFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/SeppFunction"

}

},

"NwdafFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/NwdafFunction"

}

}

}

}

]

},

"ManagedElement-Multiple": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ManagedElement-Single"

}

},

"SubNetwork-Single": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/Top-Attributes"

},

{

"type": "object",

"properties": {

"attributes": {

"allOf": [

{

"$ref": "genericNrm.json#/components/schemas/SubNetwork-Attributes"

},

{

"type": "object",

"properties": {}

}

]

}

}

},

{

"$ref": "genericNrm.json#/components/schemas/SubNetwork-ContainingObjects"

},

{

"type": "object",

"properties": {

"SubNetwork": {

"$ref": "#/components/schemas/SubNetwork-Multiple"

},

"ManagedElement": {

"$ref": "#/components/schemas/ManagedElement-Multiple"

},

"ExternalAmfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ExternalAmfFunction"

}

},

"ExternalNrfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ExternalNrfFunction"

}

},

"ExternalNssfFunction": {

"type": "array",

"items": {

"$ref": "#/components/schemas/ExternalNssfFunction"

}

},

"AmfSet": {

"type": "array",

"items": {

"$ref": "#/components/schemas/AmfSet"

}

},

"AmfRegion": {

"type": "array",

"items": {

"$ref": "#/components/schemas/AmfRegion"

}

}

}

}

]

},

"SubNetwork-Multiple": {

"type": "array",

"items": {

"$ref": "#/components/schemas/SubNetwork-Single"

}

},

"EP_N2": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N3": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N4": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N5": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N6": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N7": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N8": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N9": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N10": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N11": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N12": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N13": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N14": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N15": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N16": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N17": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N20": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N21": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N22": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N26": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N27": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N31": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_N32": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_SBI_X": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_SBI_IPX": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_S5C": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_S5U": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_Rx": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_MAP_SMSC": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_NLS": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

},

"EP_NLG": {

"$ref": "nrNrm.json#/components/schemas/EP_RP"

}

}

}

}

End of 2nd modification