3GPP TSG-SA5 Meeting #126	S5-195589

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.275 | CR | 0070 | rev | 2 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of applicable Triggers for converged charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSIMSCH |  |  |  |  |  | Date: |  |  | 2019-08-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding of convergent charging to MMTEL scenarios |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new clause for convergent charging scenarios |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MMTel cannot support convergent charging |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x.1.2 (new after 5.3) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-194202 initial CR S5-195255 resubmitted without changes |  |  |  |  |  |  |  |  |

First change

#### 5.x.1.2	Applicable Triggers for MMTel

##### 5.x.1.2.1	General

When a charging event is issued towards the CHF, it includes details such as Subscriber identifier (e.g. IMPI).

Each trigger condition (i.e. chargeable event) defined for the MMTel converged charging functionality, is specified with the associated behaviour when they are met.

Table 5.x.1.2.1 summarizes the set of default trigger conditions, including their category and if they are possible to change, which shall be supported by the MMTel AS. For "immediate report" category, the table also provides the corresponding Charging Data Request [Initial, Udate, Termination] message sent towards the CHF.

Table 5.4.1.2.1: Default Trigger conditions for MMTel

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Originating Identification Presentation (OIP) |  |  |  |  |  |
| Invite OK | - | Immediate | Not Applicable | Not Applicable |  |
| Originating Identification Restriction (OIR) |  |  |  |  |  |
| Invite OK | - | Immediate | Not Applicable | Not Applicable |  |
| Terminating Identification Presentation (TIP) |  |  |  |  |  |
| Invite OK | - | Immediate | Not Applicable | Not Applicable |  |
| Terminating Identification Restriction (TIR) |  |  |  |  |  |
| Invite OK | - | Immediate | Not Applicable | Not Applicable |  |
| Communication Hold (HOLD) |  |  |  |  |  |
| Update OK | - | Immediate | Not Applicable | Not Applicable |  |
| Communication Barring (CB) |  |  |  |  |  |
| Invite | - | Immediate | Not Applicable | Not Applicable |  |
| Message Waiting Indication (MWI) |  |  |  |  |  |
| Notify | - | Immediate | Not Applicable | Not Applicable |  |
| Conference (CONF) |  |  |  |  |  |
| Invite | - | Immediate | Not Applicable | Not Applicable |  |
| Created | - | Immediate | Not Applicable | Not Applicable |  |
| User joining | - | Immediate | Not Applicable | Not Applicable |  |
| User leaving | - | Immediate | Not Applicable | Not Applicable |  |
| Bye | - | Immediate | Not Applicable | Not Applicable |  |
| Completion of Communication sessions to Busy Subscriber (CCBS) |  |  |  |  |  |
| Notify | - | Immediate | Not Applicable | Not Applicable |  |
| Completion of Communications by No Reply (CCNR) |  |  |  |  |  |
| Notify | - | Immediate | Not Applicable | Not Applicable |  |
| Communications Diversion (CDIV) |  |  |  |  |  |
| Invite (OK) | - | Immediate | Not Applicable | Not Applicable |  |
| Answer | - | Immediate | Not Applicable | Not Applicable |  |
| Bye | - | Immediate | Not Applicable | Not Applicable |  |
| Communication Waiting (CW) |  |  |  |  |  |
| Invite OK | - | Immediate | Not Applicable | Not Applicable |  |
| Explicit Communication Transfer (ECT) |  |  |  |  |  |
| Refer | - | Immediate | Not Applicable | Not Applicable |  |
| Invite | - | Immediate | Not Applicable | Not Applicable |  |
| Answer | - | Immediate | Not Applicable | Not Applicable |  |
| Release | - | Immediate | Not Applicable | Not Applicable |  |
| Flexible Alerting (FA) |  |  |  |  |  |
| Invite | - | Immediate | Not Applicable | Not Applicable |  |
| Answer/Cancel | - | Immediate | Not Applicable | Not Applicable |  |
| Bye | - | Immediate | Not Applicable | Not Applicable |  |
| Malicious Communication Identification (MCID) |  |  |  |  |  |
| Store | - | Immediate | Not Applicable | Not Applicable |  |
| Customized Alerting Tone (CAT) |  |  |  |  |  |
| Stop | - | Immediate | Not Applicable | Not Applicable |  |
| Closed User Group (CUG) |  |  |  |  |  |
| Invite | - | Immediate | Not Applicable | Not Applicable |  |
| Answer | - | Immediate | Not Applicable | Not Applicable |  |
| Bye | - | Immediate | Not Applicable | Not Applicable |  |
| Personal Network Management (PNM) |  |  |  |  |  |
| Invite | - | Immediate | Not Applicable | Not Applicable |  |
| Answer | - | Immediate | Not Applicable | Not Applicable |  |
| Bye | - | Immediate | Not Applicable | Not Applicable |  |
| Customized Ringing Signal (CRS) |  |  |  |  |  |
| Stop | - | Immediate | Not Applicable | Not Applicable |  |

Editor's note:	The use of IEC, PEC, ECUR and SCUR is FFS.

The CDR generation mechanism processed by the CHF upon receiving Charging Data Request [Event, Initial, Update, Termination] issued by the MMTel for these chargeable events, is specified in clause 5.x.3.

End of change