3GPP TSG-SA5 Meeting #126	S5-195590

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.275 | CR | 0072 | rev | 2 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of converged charging CDR handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSIMSCH |  |  |  |  |  | Date: |  |  | 2019-08-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding of convergent charging to MMTEL CDR handling |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new clauses for convergent charging CDR handling |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MMTel cannot support convergent charging |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x.3, 5.x.4, 5.x.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This should be in the chapter introduced by S5-195588 CR 0069 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-194204 Initial CR S5-195257 resubmitted without changes |  |  |  |  |  |  |  |  |

First change

### 5.x.3	CDR generation

#### 5.x.3.1	Introduction

The CHF CDRs for MMTel charging are generated by the CHF to collect charging information that they subsequently transfer to the Charging Gateway Function (CGF).

The following clauses describe in details the conditions for generating, opening, updating and closing the CHF CDR, which shall be supported by the CHF.

#### 5.x.3.2	Triggers for CHF CDR

##### 5.x.3.2.1	General

A MMTel charging CHF CDR is used to collect charging information related to MMTel chargeable events. The details of the triggers are specified in TS 32.260 [20].

Second change

### 5.x.4	Ga record transfer flows

Details of the Ga protocol application are specified in TS 32.295 [54].

Third change

### 5.x.5	Bi CDR file transfer

Details of the Bi protocol application are specified in TS 32.297 [52].

End of change