# 3GPP TSG SA WG5 (Telecom Management) Meeting #126                     	   S5-195599
19-23 August 2019, Bruges, Belgium  	 Revision of S5-195143

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Use of MnS for performance data and architecture implications

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

This is a pCR to TR 32.845 to clarify the use of MnS for performance data and architecture implications

# 2	References

[1]	TR 32.845 "Study on Charging Aspects of Network Slicing".

[2]	3GPP TR 32.899: " Study on charging aspects of 5G system architecture phase 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 28.530: " Management and orchestration; Concepts, use cases and requirements".

[6]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[7]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[8]	3GPP TS 28.532: "Management and orchestration; Generic management services".

# 3	Rationale

This pCR proposes to clarify the use of MnS for performance data and architecture implications.

# 4	Detailed proposal

The following changes are proposed to be incorporated into TR 32.845 [1]

First change

###### 5.1.4.1.2.1	Performance based Network Slice Instance Charging Function

To leverage the capabilities of network slice management system, the enhancement of Converged Charging System for Network slice performance based charging is proposed:

Figure 5.1.4.1.2.1-1: Enhancement of Converged Charging System for Network slice

The NSI charging information provider is the service producer; the Performance based Network Slice Instances Charging Function in CCS is the service consumer. The Performance based Network Slice Instances Charging Function takes charge of requesting performance data based on agreement and creates the performance data CDR. The Performance Management services specified in TS 28.550 [230] can be used for acquiring the performance data.

This is based on management architecture for 3GPP 5G system specified in TS 28.533[206]: the Management Function is a MnF playing the role of MnS producer and the Performance based Network Slice Instances Charging Function is the MnS consumer.

Online charging is not applied for performance based charging of network slice instances.

###### 5.1.4.1.2.2	Management Function - NSMF

The following figure 5.1.4.1.2.2-1 shows Performance based Network Slice Instances Charging Function consumer of MnS service in the TS 28.533[206] management architecture,:

Figure 5.1.4.1.2.2-1: Performance based Network Slice Instances Charging Function consumer of MnS service

The NSMF is the Producer of Network Slice Performance Management Service.

##### 5.1.4.1.3	Flow Description

The figure 5.1.4.1.3-1 describes the high level charging procedure for network slice instance in solution 5.1.4.1.2.1.

Figure 5.1.4.1.3-1: Charging procedure for NSI

1.   The Performance based Network Slice Instance Charging Function decides the charging start based on subscription.

2.	The Performance based Network Slice Instance Charging Function sends the subscription request to Management Function based on agreement.

3.	The Management Function response with the successful subscription.

4.	Management Function sends the notify request to Performance based Network Slice Instance Charging Function.

5.	Performance based Network Slice Instance Charging Function acknowledges the notification.

6.	Performance based Network Slice Instance Charging Function unsubscribes the subscription.

7.	Management Function responses the successful un-subscription.

The procedure uses the "Subscribe-notify" paradigm of management framework for 3GPP 5G system which is specified in TS 28.533[206] in clause 5.1.2.

Next change

##### 5.1.4.4.2	Architecture Description

###### 5.1.4.4.2.1	CHF consumer on MnS

To leverage the capabilities of network slice management system, the CHF is proposed to be a consumer of the interface provided by the 3GPP Management Function.

Figure 5.1.4.4.2-1: CHF as producer of Nchf and consumer of the Management Service (MnS)

The Management Function is the producer of the MnS which is consumed by the CHF in order to do charging based on performance and configuration of a Network Slice. Since there is currently no granting of units available on the MnS it means that no quota management can be performed for the network slice in this solution, it would instead rely on a subscription and notification for the collection of the relevant charging information. The Performance Management services specified in TS 28.550 [230] can be used for acquiring the performance data.

This is based on management architecture for 3GPP 5G system specified in TS 28.533[206]: the Management Function is a MnF playing the role of MnS producer and the CHF is the MnS consumer.

###### 5.1.4.4.2.2	Management Function - NSMF

The following figure 5.1.4.4.2-1 shows CHF consumer of MnS service in the TS 28.533[206] management architecture, with a layered approach:

Figure 5.1.4.4.2.2-1: CHF consumer of MnS service

The NSMF is the Producer of Network Slice Performance Management Service.

Next change

#### 5.1.4.x	Solution #1.x: Network Slice Instance Performance based charging for Key issues #1.1

##### 5.1.4.x.1	Consideration for Network Slice Instance Performance Based Charging

See clause 5.1.4.1.1.

##### 5.1.4.x.2	Architecture Description

###### 5.1.4.x.2.1	Network Slice Charging Trigger Function

This solution is based on the location of a CTF within a new Network Slice Charging Trigger Function.

The following figure 5.1.4.x.2-1 shows the Network Slice Charging Trigger Function and the CHF in the TS 28.533[206] management architecture:

Figure 5.1.4.x.2-1: Network Slice Charging Trigger Function using Nchf interface for performance charging.

A charging service is exposed on Nchf by CHF and consumed by the Network Slice Charging Trigger Function.

The NSMF is the Producer of Network Slice Performance Management Service.

The Network Slice Charging Trigger Function is the MnS consumer of Network Slice Performance Management Service.

##### 5.1.4.x.3	Flow Description

The flow is the same as in clause 5.1.4.2.3 where the SMF has been replaced by the Network Slice Charging Trigger Function.

##### 5.1.4.x.4	Solution evaluation

For this solution the advantages are:

-	Different customers can have flexible and personalized network slice instance charging.

-	The CHF don’t need to be a consumer as well as a producer of services, except for the services provided by NRF.

-	The CHF can directly influence the provided service, i.e. it can grant or deny service access.

-	No major impact on the Converged Charging System architecture i.e. no new NF or service is added

And the disadvantages are:

-	Personalized network slice instance performance indicators for other management cannot be reused by charging.

-	Superfluous information may be sent to CHF in order to provide the needed flexibility and personalization possibilities.

-	A new Network Slice Charging Trigger Function needs to be defined to support consumption of PM service as well the Nchf service.

End of change