3GPP TSG-SA5 Meeting #126	S5-195701

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0112 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Number of PDU session creation in HR roaming scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2019-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The PDU sessions are created in two scerarios; Non-roaming/LBO-roaming and HR-roaming, and are created by (V-)SMF and H-SMF respectively. Currently the measurement of number of PDU session creation in HR roaming scenarios is not available in TS 28.552. Therefore it is added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Number of PDU session creation in HR roaming scenarios: - Number of PDU session creation requests in HR roaming scenarios - Number of successful PDU session creations in HR roaming scenarios - Number of failed PDU session creations in HR roaming scenarios Update PDU session establishment related measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The number of PDU session creation in HR roaming scenario cannot be measured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1.x, 5.3.1.y, 5.3.1.z (new), A.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.3.1.x	Number of PDU session creation requests in HR roaming scenario

a)	This measurement provides the number of PDU sessions requested to be created by the H-SMF in Home-Routed roaming scenario.

b)	CC

c)	On receipt by the H-SMF from V-SMF of Nsmf_PDUSession_Create Request (see 3GPP TS 23.502 [7]). Each PDU session requested to be created is added to the relevant subcounter per NSI identifier (S-NSSAI) and the relevant subcounter per request type.

d)	Each subcounter is an integer value

e)	SM.PduSessionCreationHRroam.SNSSAI

Where SNSSAI identifies the NSI;

SM.PduSessionCreationHRroam.ReqType

Where ReqType indicates the request type (e.g., initial request, initial emergency request) for the PDU session.

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

5.3.1.y	Number of successful PDU session creations in HR roaming scenario

a)	This measurement provides the number of PDU sessions successfully created by the H-SMF in Home-Routed roaming scenario.

b)	CC

c)	On transmission by the H-SMF to V-SMF of Nsmf_PDUSession_Create Response that indicates a successful PDU session creation (see 3GPP TS 23.502 [7]). Each PDU session successfully created is added to the relevant subcounter per NSI identifier (S-NSSAI) and the relevant subcounter per request type.

d)	Each subcounter is an integer value

e)	SM.PduSessionCreationHRroamSucc.SNSSAI

Where SNSSAI identifies the NSI;

SM.PduSessionCreationHRroamSucc.ReqType

Where ReqType indicates the request type (e.g., initial request, initial emergency request) for the PDU session.

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

5.3.1.z	Number of failed PDU session creations in HR roaming scenario

a)	This measurement provides the number of PDU sessions failed to be created by the H-SMF in Home-Routed roaming scenario.

b)	CC

c)	On transmission by the H-SMF to V-SMF of Nsmf_PDUSession_Create Response that indicates a rejected PDU session creation (see 3GPP TS 23.502 [7]). Each PDU session rejected to be created is added to the relevant subcounter per rejection cause.

d)	Each subcounter is an integer value

e)	SM.PduSessionCreationHRroamFail.cause

Where cause indicates the rejection cause for the PDU session.

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

2nd Change

A.14	PDU session establishment related measurements

The PDU session establishment is one of essential procedures for 5G network. The performance of PDU session establishment directly impacts the QoS of the network and the QoE of the end users. Therefore, the performance measurements are needed to reflect the performance of the PDU session establishment.

The PDU sessions are created in two scerarios; Non-roaming/LBO-roaming and HR-roaming, and are created by (V-)SMF and H-SMF respectively.

On receipt by the (V-)SMF from AMF of Nsmf_PDUSession_CreateSMContex Request, the PDU session is created in non-roaming/LBO-roaming scenario and HR-roaming scenario.

On receipt by the H-SMF from V-SMF of Nsmf_PDUSession_Create Request, the PDU session is created in HR-roaming scenario.

The number and success rate of PDU session creations, the number of PDU sessions running on the SMF are some of the basic performance measurements to monitor the performance of the PDU session establishment. And the performance measurements of failed PDU session creations are helpful to solve the network issues in case the performance is below the expectation.

End of Changes