3GPP TSG-SA5 Meeting #126	S5-195730

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019							Revision of S5-195096

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0074 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR 32.255 Correction of Serving Node change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile Com. Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-DCH |  |  |  |  |  | Date: |  |  | 2019-08-21 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS 32.255, Table 5.2.1.4.1 and Table 5.2.1.4.2 define Serving Node change as one of ‘Change of Charging conditions’ in FBC scenario. However, Table 5.2.1.6.1, Table 5.2.1.6.2, Table 5.2.3.2.2.1 and Table 5.2.3.3.2.1 wrongly use AMF change. In the EPS and 5GS interworking, serving node can be AMF or SGW. AMF change is an incomplete representation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use ‘Serving Node change’ instead of ‘AMF change’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Related Charging implementation may be confusing. Charging for interworking is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

Table 5.2.1.6.1: Default Chargeable events in SMF for QBC

| Chargeable event | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Start of PDU session | PDU session | Immediate | Not Applicable | Not Applicable | Charging Data Request [Initial] |
| Start of a QoS Flow | QoS Flow | Deferred | Not Applicable | Not Applicable | Charging Data Request [Update] |
| Change of Charging conditions |  |  |  |  |  |
| QoS change | QoS Flow | Deferred | Yes | Yes |  |
| User Location change | PDU session | Deferred | Yes | Yes |  |
| Serving Node change | PDU session | Deferred | Yes | Yes |  |
| Change of UE presence in Presence Reporting Area(s) | PDU session | Deferred | Yes | Yes |  |
| Change of 3GPP PS Data off Status | PDU session | Deferred | Yes | Yes |  |
| Tariff time change | PDU session | Deferred | No | No |  |
| UE time zone change | PDU session | Immediate | Yes | Yes |  |
| PLMN change | PDU session | Immediate | Yes | Yes |  |
| RAT type change | PDU session | Immediate | Yes | Yes |  |
| Session-AMBR change | PDU session | Immediate | Yes | Yes |  |
| Addition of UPF | PDU session | Immediate | Yes | Yes |  |
| Removal of UPF | PDU session | Immediate | Yes | Yes |  |
| Handover cancel | PDU session | Immediate | Yes | Yes |  |
| Handover start | PDU session | Immediate | Yes | Yes |  |
| Handover complete | PDU session | Immediate | Yes | Yes |  |
| Limit per PDU session |  |  |  |  |  |
| Expiry of data time limit per PDU session | PDU session | Immediate | Yes | Yes |  |
| Expiry of data volume limit per PDU session | PDU session | Immediate | Yes | Yes |  |
| Expiry of data event limit per PDU session | PDU session | Immediate | Yes | Yes |  |
| Expiry of limit of number of charging condition changes | PDU session | Immediate | Yes | Yes |  |
| Limit per QoS Flow |  |  |  |  |  |
| Expiry of data time limit per QoS Flow | QoS Flow | Deferred | Yes | Yes |  |
| Expiry of data volume limit per QoS Flow | QoS Flow | Deferred | Yes | Yes |  |
| Others |  |  |  |  |  |
| End of QoS Flow | QoS Flow | Deferred | Yes | Yes |  |
| Management intervention | PDU session | Immediate | No | No |  |
| End of PDU session | PDU session | Immediate | No | No | Charging Data Request [Termination] |
| Abort request is received from the CHF | PDU session | Immediate | No | No |  |

Second change

Table 5.2.1.6.2: Chargeable events and their related actions in SMF for QBC

| Chargeable event | Conditions | SMF action |
| --- | --- | --- |
| Start of PDU session |  | Charging Data Request [Initial] |
| Start of a QoS Flow | Start of the QoS Flow associated with the default QoS rule | Charging Data Request [Update]. |
|  | Start of a QoS Flow | Start new counts with time stamps. |
| End of a QoS Flow |  | Close the counts with time stamps for the QoS flows |
| End of PDU session |  | Charging Data Request [Termination] Close the counts with time stamps |
| Change of charging condition in the SMF (e.g. QoS change, Session-AMBR change, user location change, Radio access type change, PLMN change, Serving Node change, UE Timezone change, change of UE presence in Presence Reporting Area(s), change of 3GPP PS Data Off status, handover cancel) | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps for all active QoS flows. |
|  | If the corresponding trigger is enabled and the category is set to "immediate reporting" | Charging Data Request [Update] |
| Handover start | If the corresponding trigger is enabled | Start new counts with time stamps. |
|  | If the corresponding trigger is enabled and the category is set to "immediate reporting" | Charging Data Request [Update] |
| Handover complete | If the corresponding trigger is enabled | Close the counts with time stamps for all active QoS flows. |
|  | If the corresponding trigger is enabled and the category is set to "immediate reporting" | Charging Data Request [Update] |
| Addition of UPF |  | Charging Data Request [Update]. |
| Removal of UPF |  | Close the counts with time stamps for the removed UPF |
| Expiry of time limit per QoS Flow | If the corresponding trigger is enabled | Close the counts with time stamps. |
|  | If the category is set to "immediate reporting" | Charging Data Request [Update] |
|  | If the QoS Flow is still active | Start new counts with time stamps |
| Expiry of data volume limit per QoS Flow | If the corresponding trigger is enabled | Close the counts with time stamps |
|  | If the category is set to "immediate reporting" | Charging Data Request [Update] |
|  | If the QoS Flow is still active | Start new counts with time stamps |
| Expiry of time limit per PDU session | If the corresponding trigger is enabled | Close the counts with time stamps for all QoS flows. |
|  | If the category is set to "immediate reporting" | Charging Data Request [Update] |
|  | If the PDU session is still active | Start new counts with time stamps for all active QoS flows |
| Expiry of data volume limit per PDU session | If the corresponding trigger is enabled | Close the counts with time stamps for all QoS flows. |
|  | If the category is set to "immediate reporting" | Charging Data Request [Update] |
|  | If the PDU session is still active | Start new counts with time stamps for all active QoS flows |
| Expiry of a limit of number of charging condition changes per PDU session | If the corresponding trigger is enabled | Close the counts with time stamps for all QoS flows. |
|  | If the category is set to "immediate reporting" | Charging Data Request [Update] |
|  | If the PDU session is still active | Start new counts with time stamps for all active QoS flows |
| Management intervention |  | Charging Data Request [Update] Close the counts with time stamps for all QoS Flows |
|  | If the PDU session is still active | Start new counts with time stamps |
| Abort |  | Charging Data Request [Termination] Close the counts with time stamps |

Third change

Table 5.2.3.2.2.1: Triggers for CHF CDR charging information addition

| Trigger Conditions | Applicable for converged charging | Applicable for offline only charging |
| --- | --- | --- |
| Change of Charging conditions |  |  |
| QoS change | Yes | Yes |
| User Location change | Yes | Yes |
| Serving Node change | Yes | Yes |
| Change of UE presence in Presence Reporting Area(s) | Yes | Yes |
| Change of 3GPP PS Data off Status | Yes | Yes |
| Limit per Rating Group |  |  |
| Expiry of data time limit per RG | Yes | Yes |
| Expiry of data volume limit per RG | Yes | Yes |
| Expiry of data event limit per RG | Yes | Yes |
| Quota management triggers |  |  |
| Time threshold reached | Yes | No |
| Volume threshold reached | Yes | No |
| Unit threshold reached | Yes | No |
| Time quota exhausted | Yes | No |
| Volume quota exhausted | Yes | No |
| Unit quota exhausted | Yes | No |
| Expiry of quota validity time | Yes | No |
| Re-authorization request by CHF | Yes | No |

Fourth change

Table 5.2.3.3.2.1: Triggers for CHF CDR charging information addition for roaming QBC

Trigger Conditions

Change of Charging conditions

QoS change

User Location change

Serving Node change

Change of UE presence in Presence Reporting Area(s)

Change of 3GPP PS Data off Status

Limit per QoS Flow

Expiry of data time limit per QoS Flow

Expiry of data volume limit per QoS Flow

Others

End of QoS Flow

End of change