3GPP TSG-SA5 Meeting #126	S5-195762

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0092 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of multipleQuotaInformation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SBI_CH |  |  |  |  |  | Date: |  |  | 2019-08-22 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Naming of multipleQuotaInformation is inconsistent with the naming in TS 32.290 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change of MultipleQuotaInformation to MultipleUnitInformation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies between specs may lead to inconsistent implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.2.4, 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-195237 initial CR |  |  |  |  |  |  |  |  |

First change

###### 6.1.6.2.2.4	Type MultipleUnitInformation

This clause is additional attributes of the type MultipleUnitInformationdefined in clause 6.1.6.2.1.8 for 5G data connectivity charging described in 3GPP TS 32.255[30].

Table 6.1.6.2.2.4-1: 5G Data Connectivity Specified attribute of type MultipleUnitInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| uPFID | NfInstanceId | Oc | 0..1 | UPF id |  |

Second change

## 7.1	Bindings of common CDR field, Information Element and Resource Attribute

Table 7.1-1: Bindings of common CDR field, Information Element and Resource Attribute

| Information Element | CDR Field | Resource Attribute |
| --- | --- | --- |
| Session Identifier | Charging Session Identifier | /{ ChargingDataRef }/ |
|  |  | ChargingDataRequest |
| Subscriber Identifier | Subscriber Identifier | /subscriberIdentifier |
| Invocation Timestamp | - | /invocationTimeStamp |
| Invocation Sequence Number | - | /invocationSequenceNumber |
| NF Consumer Identification | NF Information | /nfConsumerIdentification |
| NF Name | Recording Network Function ID | /nfConsumerIdentification/nFName |
| NF Address | SMF Address | /nfConsumerIdentification/nFIPv4Address /nfConsumerIdentification/nFIPv6Address |
| NF PLMN ID | SMF PLMN ID | /nfConsumerIdentification/nFPLMNID |
| NF Functionality | Record Type | /nfConsumerIdentification/nodeFunctionality |
| Notify URI |  | /notifyUri |
| Multiple Unit Usage | List of Multiple Unit Usage | /multipleUnitUsage |
| Rating Group | Rating Group | /multipleUnitUsage/ratingGroup |
| Requested Unit | - | /multipleUnitUsage/requestedUnit |
| Time | - | /multipleUnitUsage/requestedUnit/time |
| Total Volume | - | /multipleUnitUsage/requestedUnit/totalVolume |
| Uplink Volume | - | /multipleUnitUsage/requestedUnit/uplinkVolume |
| Downlink Volume | - | /multipleUnitUsage/requestedUnit/downlinkVolume |
| Service Specific Units | - | /multipleUnitUsage/requestedUnit/serviceSpecificUnits |
| Used Unit Container | Used Unit Container | /multipleUnitUsage/usedUnitContainer |
| Service Identifier | Service Identifier | /multipleUnitUsage/usedUnitContainer/serviceId |
| Quota management Indicator | Quota management Indicator | /multipleUnitUsage/usedUnitContainer/quotaManagementIndicator |
| Triggers | Triggers | /multipleUnitUsage/usedUnitContainer/triggers |
| Trigger Timestamp | Trigger Timestamp | /multipleUnitUsage/usedUnitContainer/triggerTimestamp |
| Time | Time | /multipleUnitUsage/usedUnitContainer/time |
| Total Volume | Total Volume | /multipleUnitUsage/usedUnitContainer/totalVolume |
| Uplink Volume | Uplink Volume | /multipleUnitUsage/usedUnitContainer/uplinkVolume |
| Downlink Volume | Downlink Volume | /multipleUnitUsage/usedUnitContainer/downlinkVolume |
| Service Specific Unit | Service Specific Unit | /multipleUnitUsage/usedUnitContainer/serviceSpecificUnits |
| Event Time Stamps | Event Time Stamps | /multipleUnitUsage/usedUnitContainer/eventTimeStamps |
| Local Sequence Number | Local Sequence Number | /multipleUnitUsage/usedUnitContainer/localSequenceNumber |
| Triggers | Service Condition Change | /triggers |
|  |  | ChargingDataResponse |
| Invocation Timestamp |  | /invocationTimeStamp |
| Invocation Sequence Number |  | /invocationSequenceNumber |
| Session Failover | - | /sessionFailover |
| Triggers | - | /triggers |
| Multiple Unit Information | - | /multipleUnitInformation |
| Result Code | - | /multipleUnitInformation/resultCode |
| Rating Group | - | /multipleUnitInformation/ratingGroup |
| Granted Unit | - | /multipleUnitInformation/grantedUnit |
| Tariff Time Change | - | /multipleUnitInformation/grantedUnit/tariffTimeChange |
| Time | - | /multipleUnitInformation/grantedUnit/time |
| Total Volume | - | /multipleUnitInformation/grantedUnit/totalVolume |
| Uplink Volume | - | /multipleUnitInformation/grantedUnit/uplinkVolume |
| Downlink Volume | - | /multipleUnitInformation/grantedUnit/downlinkVolume |
| Service Specific Units | - | /multipleUnitInformation/grantedUnit/serviceSpecificUnits |
| Triggers | - | /multipleUnitInformation/triggers |
| Validity Time | - | /multipleUnitInformation/validityTime |
| Quota Holding Time | - | /multipleUnitInformation/quotaHoldingTime |
| Final Unit Indication | - | /multipleUnitInformation/finalUnitIndication |
| Time Quota Threshold | - | /multipleUnitInformation/timeQuotaThreshold |
| Volume Quota Threshold | - | /multipleUnitInformation/volumeQuotaThreshold |
| Unit Quota Threshold | - | /multipleUnitInformation/unitQuotaThreshold |
| Invocation Result | - | /invocationResult |
| Result code | - | /invocationResult/error |
| Failed parameter | - | /invocationResult/error |
| Failure Handling | - | /invocationResult/failureHandling |

End of changes