3GPP TSG-SA5 Meeting #126	S5-195778

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.533 | CR | 0032 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR TS 28.533 Add management service discovery solution concept |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GDMS |  |  |  |  |  | Date: |  |  | 2019-08-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | One of the objectives of 5GDMS WI is to enchance and extend the existing management service discovery in release 15, currently captured in TS 28.533. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding management service discovery solution concept |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 4.7, 4.7.1 and 4.7.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of 1st modified section in TS 28.533

## 4.7			Management service discovery

### 4.7.1	Introduction

The MnS consumer need to discover MnS instances provided by MnS producers. In order for MnS Producer to be discovered by MnS consumer, the access information on MnS Producer needs to be registered in the DNS. The registration process is not subject of MnS standardisation.

Figure 4.7.x-1 below shows interactions of MnS Consumer with DNS and MnS Producer:

- The MnS Producer 1 provides MnS Consumer with a name

- MnS Consumer obtains MnS Producer 1 IP address and port

- MnS Consumer queries MnS Producer 1 for the information about MnS capability

Figure 4.7.x-1: Example of management service discovery procedure

Note: The use of DNS is not mandatory, e.g. the access information (e.g. IP address and port) for MnS Producer may be given to MnS Consumer in non-standard way.

End of modification in TS 28.533