3GPP TSG-SA5 Meeting #126	S5-195779

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0050 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR TS 28.532 Add stage 2 and stage 3 definition for MnSCapabilityQuery operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GDMS |  |  |  |  |  | Date: |  |  | 2019-08-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is a coresponding stage 1 definition of MnS capability query use case and requirement in TS 28.530. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding stage 2 and stage 3 definition for MnS Capability Query operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MnS Capability Query operation will be not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X(new), Y(new), Z(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# X	MnS discovery

## X.1	Operations and notifications

### X.1.1	MnS Capability Query operation

#### X.1.1.1	Description

The MnS consumer uses getMOIAttributes operation defined in TS 28.532 to obtain information about MnS capability

## X.2	Managed Information

### X.2.1	MnSInformation

#### X.2.1.1	Definition

The IOC contains information about capabilities of MnS instance.

#### X.2.1.2	Attribute

| Attribute name | Support Qualifier |
| --- | --- |
| MnStype | M |
| additionalText | O |
| MnSProducer | M |

### X.2.2	Information attribute definition

#### X.2.2.1	Definition and legal values

| Name | Definition | Legal Values |
| --- | --- | --- |
| MnSType | It indicates the type of the MnS instance | All types of MnS (e.g. provisioning MnS, Fault Supervision MnS, Performance Assurance MnS) |
| additionalText | It carries semantics that is outside the scope of this management service specification. It can contain further information on the MnS instance. |  |
| MnSProducer | It carries the information about the MnSProducer who provide the MnS instance |  |

Second change

# Y	RESTful HTTP-based solution set of MnS Discovery

## Y.1	Mapping of operations

### Y.1.1	Introduction

The IS operations are mapped to SS equivalents according to chapter 8.1.3 Operation "getMOIAttributes defined in TS 28.532".

The message flow is as follows:

- The Service Consumer sends a HTTP GET request to the Service Provider.

The URI identifies the "…/MnSs" collection resource.

The query part may contain optional parameter "filter". Absence of the query part means all MnSs shall be returned.

The request message body shall be empty.

- The Service Provider sends a HTTP GET response to the Service Consumer.

On success "200 OK" shall be returned. The response message body shall carry the returned MnSs. The response format is defined by ResponseType.

On failure, an appropriate error code shall be returned. The response message body may carry an error object.

## Y.2	Mapping of notifications

N/A

## Y.3	Resources

### Y.3.1	Resource definitions

#### Y.3.1.1	Resource "/MnSs"

##### Y.3.1.1.1	Description

This resource represents a collection of MnSs.

##### Y.3.1.1.2	URI

Resource URI: {DN_prefix_authority_part}/{DN_prefix_remainder}/DiscoveryMnS/v1500/MnSs

The resource URI variables a defined in the following table.

Table Y.3.1.1.2-1: URI variables

| Name | Definition |
| --- | --- |
| DN_prefix_authority_part | See subclause 4.4 of TS 32.158 [15] |
| DN_prefix_remainder | See subclause 4.4 of TS 32.158 [15] |

Y.3.1.1.2.1	HTTP GET

This method shall support the URI query parameters specified in the following table.

Table Y.3.1.1.2.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | Qualifier |
| --- | --- | --- | --- |
| filter | filter-QueryType | This parameter reduces the targeted set of resources by applying a filter to the scoped set of resource representations. Only resources representations for which the filter construct evaluates to "true" are targeted. No filter language is specified in the present release. | O |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table Y.3.1.1.2.1-2: Data structures supported by the GET Request Body on this resource

| Data type | Description | Qualifier |
| --- | --- | --- |
| n/a | n/a | n/a |

Table Y.3.1.1.2.1-3: Data structures supported by the GET Response Body on this resource

| Data type | Response codes | Description | Qualifier |
| --- | --- | --- | --- |
| MnSs-Type | 200 OK | The MnSCapability returned. | M |
| error-Type | 4xx/5xx | Returned in case of an error | O |

## Y.4	Data type definitions

### Y.4.1	General

Table Y.4.1-1: Data types defined in the present document

| Data type | Reference | Description |
| --- | --- | --- |
| General types |  |  |
| N/A |  |  |
| Types used in paths |  |  |
| N/A |  |  |
| Types in query parts |  |  |
| filter-QueryType | Y.4.4.2 | Used in the query part of HTTP GET on /MnSs to discriminate MnS instances to be returned. |
| Types used in request bodies |  |  |
| N/A |  |  |
| Types used in response bodies |  |  |
| MnSs-ResponseType | Y.4.2.1 | Used in the response body of HTTP GET on /alarms to return complete alarm information |
| Types used for resources |  |  |
| MnS-ResourceType | Y.4.2.2 | Representation of an MnS instance resource |
| Types used in notifications |  |  |
| N/A |  |  |
| Types referenced by the definitions above |  |  |
| additionalText-Type | Y.4.4.2 | Allows a free form text description to be reported. |
| MnSType-Type | Y.4.4.2 | Type of MnS instance |
| MnSProducer-Type | Y.4.4.2 | Information about the MnS Producer |

### Y.4.2	Query, message body and resource data types

#### Y.4.2.1	Type MnSs-ResponseType

Table Y.4.2.1-1: Definition of type MnSs-ResponseType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| data | array(MnS-ResourceType) | Used in the response body of HTTP GET on /MnSs to return complete MnS capability | M |

#### Y.4.2.2	Type MnS-ResponseType

Table Y.4.2.2-1: Definition of type MnS-ResponseType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| > MnSType | MnS-Type | Type of MnS instance | M |
| > additionalText | additionalText-Type | Further information on the MnS instance | O |
| >MnSProducer | MnSProducer-Type | Information about the MnS Producer who provide the MnS instance. | M |

### Y.4.3	Referenced structured data types

N/A

### Y.4.4	Simple data types and enumerations

#### Y.4.4.1	General

This subclause defines simple data types and enumerations that are used by the data structures defined in the previous subclauses.

#### Y.4.4.2	Simple data types

Table Y.443.2-1: Simple data types

| Type name | Type definition | Description |
| --- | --- | --- |
| additionalText-Type | string | Allows a free form text description. |
| filter-QueryType | string | Used in the query part of HTTP GET on /MnSs to discriminate MnS capabilites to be returned |
| MnSProducer-Type | string | Information about the MnSProducer who provide the MnS instance. |

#### Y.4.4.3	Enumeration MnSType-Type

Table Y.4.4.3-1: Enumeration MnSType-Type

| Enumeration value | Description |
| --- | --- |
| ProvisionMnS | A MnS instance of this type is Provivisioning MnS defined in Clause 5. |
| FaultSupervisionMnS | A MnS instance of this type is FaultSupervision MnS defined in Clause 6. |
| PerformanceAssuranceMnS | A MnS instance of this type is PerformanceAssurance MnS defined in Clause 7. |

Third change

## Annex Z (normative): OpenAPI specification for MnS discovery

This clause describes the capabilities of the MnS in the structure of the OpenAPI Specification Version 3.0.1 [A9]. The OpenAPI document is represented in the JSON format option.

{

"openapi": "3.0.0",

"info": {

"title": "TS 28.532 MnS Discovery",

"version": "15.0.0",

"description": "OAS 3.0.0 specification for the MnS Discovery (MnSDiscovery)"

},

"servers": [

{

"url": "http://{DN_prefix_authority_part}/{DN_prefix_remainder}/MnSDiscovery/v1500",

"variables": {

"DN_prefix_authority_part": {

"description": "See subclause 4.4 of TS 32.158",

"default": "example.com"

},

"DN_prefix_remainder": {

"description": "See subclause 4.4 of TS 32.158",

"default": ""

}

}

}

],

"paths": {

"/MnSs": {

"get": {

"summary": "Retrieve MnSs",

"description": "Retrieve the MnS capabilites identified by a filter.",

"parameters": [

{

"name": "filter",

"in": "query",

"required": false,

"description": "This parameter reduces the targeted set of resources by applying a filter to the set of resource representations.Using logical operators AND, OR and NOT, filter constraint can be used as logical expression.If the filter is not present, all of the MnS capabilites included by the scope are selected. ",

"schema": {

"$ref": "#/components/schemas/filter-QueryType"

}

}

],

"responses": {

"200": {

"description": "Success case (\"200 OK\"). Returns the MnS capabilites identified in the request.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/MnSs-ResponseType"

}

}

}

},

"default": {

"description": "Response in case of error.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

}

}

},

"components": {

"schemas": {

"uri-Type": {

"type": "string"

},

"filter-QueryType": {

"type": "string"

},

"MnSs-ResponseType": {

"type": "object",

"properties": {

"data": {

"type": "array",

"items": {

"$ref": "#/components/schemas/MnS-ResourceType"

}

}

}

},

"error-ResponseType": {

"type": "object",

"properties": {

"error": {

"type": "object",

"properties": {

"errorInfo": {

"type": "string"

}

}

}

}

},

"MnS-ResourceType": {

"type": "object",

"properties": {

"href": {

"$ref": "#/components/schemas/uri-Type"

},

"MnSProducer": {

"$ref": "#/components/schemas/MnSProducer-Type"

},

"MnSType": {

"$ref": "#/components/schemas/MnSType-Type"

},

"additionalText": {

"$ref": "#/components/schemas/additionalText-Type"

}

}

},

"additionalText-Type": {

"type": "string"

},

"MnSProducer-Type": {

"type": "string"

},

"MnSType-Type": {

"type": "string",

"enum": [

"Provisioning MnS",

"Fault Supervision MnS",

"Performance Assurance MnS"

]

}

}

}

}

End