3GPP TSG-SA5 Meeting #126	S5-195903

# Bruges, Belgium, 19th Aug 2019 - 23rd Aug 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.622 | CR | 0037 | rev | 2 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update class definition with inheritance information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-5GNRM |  |  |  |  |  | Date: |  |  | 2019-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As endorsed discussion paper S5-194539 proposed, add description information indicating the inheritance information in class definition clause to avoid the issued caused by missing inherited attributes in attribute table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description information indicating the inheritance information in class definition for Generic NRM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The missing inherited attributes in attribute table in class definition clause will make some misunderstanding and complexity for the TS stage 2 Reader to get all the attribute for the class. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.2.2, 4.3.3.2, 4.3.4.2, 4.3.5.2, 4.3.6.2, 4.3.7.2, 4.3.8.2, 4.3.9.2, 4.3.10.2, 4.3.11.2, 4.3.12.2, 4.3.13.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

4.3	Class definitions

4.3.1	Any

4.3.1.1	Definition

This class represents the classes (e.g. IOC) that are not defined in this specification but are or will be defined in other IRP specification(s).

4.3.1.2	Attributes

None

4.3.1.3	Attribute constraints

None

4.3.1.4	Notifications

This class does not support any notification.

4.3.2	IRPAgent

4.3.2.1	Definition

This IOC represents the functionality of an IRPAgent. It shall be present. For a definition of IRPAgent, see 3GPP TS 32.102 [2].

The IRPAgent will be contained under an IOC as follows (only one of the options shall be used):

1)	ManagementNode, if the configuration contains a ManagementNode;

2)	SubNetwork, if the configuration contains a SubNetwork and no ManagementNode;

3)	ManagedElement, if the configuration contains no ManagementNode or SubNetwork.

4.3.2.2	Attributes

The IRPAgent IOC includes the attributes inherited from Top_ IOC (defined in TS 28.620 [9]), attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| systemDN | C | M | - | - | M |

4.3.2.3	Attribute constraints

None

4.3.2.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions

4.3.3	ManagedElement

4.3.3.1	Definition

This IOC represents telecommunications equipment or TMN entities within the telecommunications network providing support and/or service to the subscriber. 
An ME communicates with a manager (directly or indirectly) over one or more management interfaces for the purpose of being monitored and/or controlled. MEs may or may not additionally perform element management functionality. 
An ME contains equipment that may or may not be geographically distributed. An ME is often referred to as a "Network Element".

A telecommunication equipment has software and hardware components. The IOC described above represents the case when the software component is designed to run on dedicated hardware component. In the case when the software is designed to run on ETSI NFV defined NFVI [15], the IOC description would exclude the NFVI component supporting the above mentioned subject software. A ManagedElement may be contained in either a SubNetwork or in a MeContext instance. A single ManagedElement may also exist stand-alone with no parent at all.

The ManagedElement IOC may be used to represent combined ME functionality (as indicated by the managedElementType attribute and the contained instances of different functional IOCs).

Single function ManagedElement IOC instances will have a 1..1 containment relationship to a function IOC instance (in this context a function IOC instance is an instance of an IOC derived from the ManagedFunction IOC). Multiple function ManagedElement instances will have a 1..N containment relationship to function IOC instances.

NOTE:	For some specific functional IOCs a 1..N containment relationship is permitted.  The specific functional entities are identified in the NRMs that define subclasses of ManagedFunction.

4.3.3.2	Attributes

The ManagedElement IOC includes the attributes inherited from ManagedElement_ IOC (defined in TS 28.620 [9]), attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| id | M | M | - | M | - |
| vendorName | M | M | - | - | M |
| userDefinedState | M | M | M | - | M |
| swVersion | M | M | - | - | M |
| priorityLabel | O | T | F | T | F |

4.3.3.3	Attribute constraints

Attribute constrains for dnPrefix: The attribute dnPrefix shall be supported if an instance of ManagedElement is the local root instance of the MIB. Otherwise the attribute shall be absent or carry no information.

4.3.3.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC. In addition, the following set of notifications is also valid.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyDownloadNESwStatusChanged | See Software Management IRP (3GPP TS 32.532 [6]) |  |
| notifyInstallNESwStatusChanged | See Software Management IRP (3GPP TS 32.532 [6]) |  |
| notifyActivateNESwStatusChanged | See Software Management IRP (3GPP TS 32.532 [6]) |  |

4.3.4	ManagedFunction

4.3.4.1	Definition

This IOC is provided for sub-classing only. It provides attribute(s) that are common to functional IOCs. Note that a ManagedElement may contain several managed functions. The ManagedFunction may be extended in the future if more common characteristics to functional objects are identified.

This IOC can represent a telecommunication function either realized by software running on dedicated hardware or realized by software running on NFVI. Each ManagedFunction instance communicates with a manager (directly or indirectly) over one or more management interfaces exposed via its containing ME instance.

4.3.4.2	Attributes

The ManagedFunction IOC includes the attributes inherited from Function_ IOC (defined in TS 28.620 [9]), attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| id | M | M | - | M | - |
| vnfParametersList | CM | M | M | - | M |
| peeParametersList | CM | M | M | - | M |
| priorityLabel | O | T | F | T | F |

4.3.4.3	Attribute constraints

| Name | Definition |  |
| --- | --- | --- |
| vnfParametersList | It shall be supported if the ManagedFunction instance is realized by one or more VNF instance(s). Otherwise this attribute shall be absent. |  |
| peeParametersList |  | It shall be supported if the control and monitoring of PEE parameters is supported by the ManagedFunction or sub-class instance. |

4.3.4.4	Notifications

There is no notification defined.

4.3.5	ManagementNode

4.3.5.1	Definition

This IOC represents a telecommunications management system (EM) within the TMN that contains functionality for managing a number of ManagedElements (MEs). The management system communicates with the MEs directly or indirectly over one or more interfaces for the purpose of monitoring and/or controlling these MEs.

This class has similar characteristics as the ManagedElement. The main difference between these two classes is that the ManagementNode has a special association to the managed elements that it is responsible for managing.

4.3.5.2	Attributes

The ManagementNode IOC includes the attributes inherited from ManagementSystem_ IOC (defined in TS 28.620 [9]), attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| id | M | M | - | M | - |
| vendorName | M | M | - | - | M |
| userDefinedState | M | M | M | - | M |
| locationName | M | M | - | - | M |
| swVersion | M | M | - | - | M |

4.3.5.3	Attribute constraints

None

4.3.5.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

4.3.6	MeContext

4.3.6.1	Definition

This IOC is introduced for naming purposes. It may support creation of unique DNs in scenarios when some MEs have the same RDNs due to the fact that they have been manufacturer pre-configured. 
If some MEs have the same RDNs (for the above mentioned reason) and they are contained in the same SubNetwork instance, some measure shall be taken in order to assure the global uniqueness of DNs for all IOC instances under those MEs. One way could be to set different dnPrefix for those NEs, but that would require either that:

a)	all LDNs or DNs are locally modified using the new dnPrefix for the upper portion of the DNs, or

b)	a mapping (translation) of the old LDNs or DNs to the new DNs every time they are used externally, e.g. in alarm notifications.

As both the two alternatives above may involve unacceptable drawbacks (as the old RDNs for the MEs then would have to be changed or mapped to new values), using MeContext offers a new alternative to resolve the DN creation. Using MeContext as part of the naming tree (and thus the DN) means that the dnPrefix, including a unique MeContext for each ME, may be directly concatenated with the LDNs, without any need to change or map the existing ME RDNs to new values.

MeContext have 0..N instances. It may exist even if no SubNetwork exists. Every instance of MeContext contains exactly one ManagedElement during steady-state operations.

4.3.6.2	Attributes

The MeContext IOC includes the attributes inherited from Top_ IOC (defined in TS 28.620 [9]), attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dnPrefix | CM | M | - | - | M |

4.3.6.3	Attribute constraints

| Name | Definition |
| --- | --- |
| dnPrefix | It shall be supported if an instance of MeContext is the local root instance of the MIB. Otherwise the attribute shall be absent or carry no information. |

4.3.6.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

4.3.7	SubNetwork

4.3.7.1	Definition

This IOC represents a set of managed entities.There may be zero or more instances of a SubNetwork. It shall be present if either a ManagementNode or multiple ManagedElements are present (i.e. ManagementNode and multiple ManagedElement instances shall have SubNetwork as parent).

The SubNetwork instance not contained in any other instance of SubNetwork is referred to as "the root SubNetwork instance".

4.3.7.2	Attributes

The SubNetwork IOC includes the attributes inherited from Domain_ IOC (defined in TS 28.620 [9]), attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| id | M | M | - | M | - |
| setOfMcc | CM | M | - | - | M |
| priorityLabel | O | T | F | T | F |

4.3.7.3	Attribute constraints

| Name | Definition |
| --- | --- |
| dnPrefix (inherited from Domain_) | It shall be supported if an instance of SubNetwork is the local root instance of the MIB. Otherwise the attribute shall be absent or carry no information. |
| setOfMcc | If there is more than one value in setOfMcc of the SubNetwork instance, the support of this attribute setOfMcc is mandatory. Otherwise it is optional. |

4.3.7.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions

4.3.8	Top

4.3.8.1	Definition

This IOC is provided for sub-classing only. All information object classes defined in all TS that claim to be conformant to 32.102 [2] shall inherit from Top.

4.3.8.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| objectClass | M | M | - | M | M |
| objectInstance | M | M | - | M | M |

4.3.8.3	Attribute constraints

None

4.3.8.4	Notifications

There is no notification defined.

4.3.9	VsDataContainer

4.3.9.1	Definition

The VsDataContainer managed object is a container for vendor specific data. The number of instances of the VsDataContainer can differ from vendor to vendor. This IOC shall only be used by the Bulk CM IRP for all the NRMs.

4.3.9.2	Attributes

The VsDataContainer IOC includes the attributes inherited from Top_ IOC (defined in TS 28.620 [9]), attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| vsDataType | M | M | - | - | O |
| vsData | M | M | O | - | O |
| vsDataFormatVersion | M | M | - | - | O |

4.3.9.3	Attribute constraints

None

4.3.9.4	Notifications

Support for notification on the change of attribute value is vendor-specific.

4.3.10	Link

4.3.10.1	Definition

This IOC is provided for sub-classing only. This IOC represents a communication link or reference point between two network entities. The Link IOC does not indicate whether the represented communication link or reference point is a physical or logical entity.

For the subclasses of Link, the following rules apply:

1)	The subclass names shall have the form “Link_<X>_<Y>”, where <X> is a string that represents the IOC at one end of the association related to the particular Link subclass, and <Y> is a string that represents the IOC at the other end of the association. For the order of the two strings, <X> shall come alphabetically before <Y>.

2)	In case <X> and <Y> are YyyFunction IOCs (inheriting from ManagedFunction and on first level below ManagedElement), the <X> and <Y> strings shall have the same form as the legal values of the managedElementType attribute (see clause 4.5.1), e.g. “Auc”.  Otherwise <X> and <Y> shall be the full IOC names.

Thus, two valid examples of Link subclass names would be: Link_As_Cscf and Link_Mrfc_Mrfp.

4.3.10.2	Attributes

The Link IOC includes the attributes inherited from TopologicalLink_ (defined in TS 28.620 [9]), attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| id | M | M | - | M | - |
| userLabel | M | M | M | - | M |
| linkType | O | M | - | - | M |
| protocolVersion | O | M | - | - | M |

4.3.10.3	Attribute constraints

| Name | Definition |
| --- | --- |
| aEnd and zEnd (inherited from TopologicalLink_) | The property multiplicity is 1. |

4.3.10.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions

4.3.11	EP_RP

4.3.11.1	Definition

This IOC is provided for sub-classing only. This IOC represents an end point of a link used across a reference point between two network entities.

The detailed subclassed IOC, e.g. EP_X2, will be defined in E-UTRAN NRM IRP, by inheriting from this EP_RP.

For naming the subclasses of EP_RP, the following rules shall apply:

-	The name of the subclassed IOC shall have the form “EP_<rp>”, where <rp> is a string that represents the name of the reference point.

Thus, two valid examples of EP_RP subclassed IOC names would be: EP_S1 and EP_X2.

4.3.11.2	Attributes

The EP_RP IOC includes the attributes inherited from Top_ (defined in TS 28.620 [9]), attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| farEndEntity | O | M | - | - | M |
| userLabel | O | M | M | - | M |

4.3.11.3	Attribute constraints

None

4.3.11.4	Notifications

This class does not support any notification.

4.3.12	MeasurementControl

4.3.12.1	Definition

This IOC represents the capabilities to produce and deliver Measurements identified by <<IOC>>MeasurementReader.

One instance of this IOC can be name-contained by one instance of <<IOC>>SubNetwork or one instance of <<IOC>>ManagedElement or one instance of <<IOC>>ManagedFunction. This IOC instance is responsible for the production and delivery of the Measurements of the (tree) nodes of a name-containment tree whose top (tree) node is the IOC instance name-containing the said <<IOC>>MeasurementControl instance.

Instance of this IOC shall not be created nor deleted by consumer. It shall be created and deleted by the producer.

This IOC identifies if stream-based or file-based delivery method is used. If fileLocation has valid information, the file-based delivery method is used and streamTarget information is ignored, otherwise, if streamTarget has valid information, the stream-based delivery method is used.

4.3.12.2	Attributes

The MeasurementControl IOC includes attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pMAdministrativeState | M | T | T | F | T |
| pMOperationalState | M | T | F | F | T |
| fileBasedGP | M | T | T | F | F |
| streamBasedGP | O | T | T | F | F |
| fileLocation | M | T | T | F | F |
| streamTarget | O | T | T | F | F |

4.3.12.3	Attribute constraints

None.

4.3.12.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions

4.3.13	MeasurementReader

4.3.13.1	Definition

This IOC identifies the entity (derivatives of <<IOC>>ManagedFunction) whose Measurements are required by consumer to be produced and captured. The types of Measurements required are identified by the attribute measurementNames.

This IOC identifies if stream-based or file-based delivery method is used. If fileLocation has valid information, the file-based delivery method is used and streamTarget information is ignored, otherwise, if streamTarget has valid information, the stream-based delivery method is used

In case the fileLocation contain no information, the associated <<IOC>>MeasurementControl fileLocation is effective. In case it contains information, the associated <<IOC>>MeasurementControl fileLocation is ignored.

In case the streamTarget contain no information, the associated <<IOC>>MeasurementControl streamTarget is effective. In case it contains information, the associated <<IOC>>MeasurementControl streamTarget is ignored.

This IOC uses an attribute managedObjectDNs and managedObjectDNsBasic to identify specific managed objects of the management containment tree.

4.3.13.2	Attributes

The MeasurementReader IOC includes attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| measurementTypes | M | T | T | F | F |
| fileBasedGP | O | T | T | F | F |
| streamBasedGP | O | T | T | F | F |
| fileLocation | O | T | T | F | F |
| streamTarget | O | T | T | F | F |
| managedObjectDNsBasic | M | T | T | F | F |
| managedObjectDNs | M | T | T | F | F |

4.3.13.3	Attribute constraints

None.

4.3.13.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions

4.3.14	Measurements<<datatype>>

4.3.14.1	Definition

This <<datatype>> captures the Measurement types supported by its associated <<IOC>>, i.e. <<IOC>>ManagedFunction or <<IOC>>ManagedElement or <<IOC>>EP_RP. It also captures the list of GPs supported, on per Measurement type. This <<datatype>> indicates a capability of the associated <<IOC>> and therefore its isWritable property is False and its isInvariant property is False (since vendor product upgrade can increase or decrease the number of supported Measurement types without the need to deleting the older-version instance and creating a newer-version instance.)

4.3.14.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| measurementTypeAndGPs | M | T | F | F | F |

4.3.14.3	Attribute constraints

None

4.3.14.4	Notifications

Not applicable.

End of Change