3GPP TSG-SA5 Meeting #128 	S5-197128

# Zhuhai,China, 18-22 November 2019

# Source:	Huawei

# Title:	Add solutions for ANR management

# Document for:	Approval

Agenda Item:	6.4.13

1	Decision/action requested

The group is asked to discuss and approve the proposals.

2	References

[1]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks v0.0.0".

[2]             3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage 2".

3	Rationale

ANR function resides in gNB is described in TS 38.300 [2] for both intra-system and inter-system scenairos. Also the corresponding use cases and requirements have been captured in TS 28.313[1].

This contribution proposes to add the solutions to support the management of ANR function.

4	Detailed proposal

1st modified section

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ANR	Automatic Neighbour Relation

NCR	Neighbour Cell Relation

NG-RAN	Next Generation Radio Access Network

NCRT	Neighbour Cell Relation Table

2nd modified section

7	Management services for SON

Editor’s note: this clause contains the management services for SON and definition of information, such as targets, control, parameters, and PM measurements that are needed to support SON functions.

7.1	Management services for D-SON management

7.1.X	ANR management

7.1.X.1	Information definition

7.1.X.1.1	Targets information

The targets of ANR are shown in the Table 7.1.X.1.1-1.

Table 7.1.X.1.1-1.  ANR management targets

| Target Name | Definition | Legal Values |
| --- | --- | --- |
| Total handover failure rate | (the number of failure events related to handover) / (the total number of handover events) | [0..100] in unit percentage |

7.1.X.1.2	Control information

The parameter is used to control the ANR function.

Table 7.1.X.1.2-1.  ANR management control information

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| Intra-system ANR control | This attribute allows the operator to enable/disable the intra-system ANR functionality. | Boolean On, off |
| Inter-system ANR control | This attribute allows the operator to enable/disable the inter-system ANR functionality | Boolean On, off |

7.1.X.1.3	Parameters to be updated

Table 7.1.X.1.3-1.  ANR management parameters

| Control parameters | Definition | Legal Values |
| --- | --- | --- |
| xnBlackList | Prohibited from sending Xn connection request to target node; Forced to tear down established Xn connection to target node; Not allowed to accept incoming Xn connection request from target node. | Integer 0,1 |
| xnWhiteList | Is allowed to request the establishment of Xn connection with the target node; Is not allowed to initiate the tear down of established Xn connection to target node. | Integer 0,1 |
| xnHOBlackList | Handover is prohibited from source node to target node using Xn connection. | Integer 0,1 |
| isHOAllowed | Indicates if handover from source cell to target cell is allowed or not. | Enumeration Yes, no |
| isRemoveAllowed | Indicates if certain neighbour cell relations can be removed or not. | Enumeration Yes, no |

7.1.X.1.4	Performance measurements

Performance measurements related to ANR management are captured in Table 7.1.X.1.4-1:

Table 7.1.X.1.4-1.  ANR management related performance measurements

| Performance measurements | Description | Related targets |
| --- | --- | --- |
| Number of handover events | Includes all successful and unsuccessful handover events (see clause 5.1.1.6 in TS 28.552 [5]). | Total handover failure rate |
| Number of handover failures | Includes unsuccessful handover events with failure causes (see clause 5.1.1.6 in TS 28.552 [5]). | Total handover failure rate |

End of modified section