3GPP TSG-SA5 Meeting #129-e	S5-201238

# Online, , 24th Feb 2020 - 4th Mar 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.290 | CR | 0107 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of quota handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, 5GS_Ph1-SBI_CH |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The Diameter Credit-Control Application RFC 8506 specified that “When the used units are reported to the credit-control server, the credit-control client will not have any units in its possession before new granted units are received from the credit-control server.”. A similar statement is missing in for the Nchf. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification of how the granted quota should be treated at reporting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CTF may have different interpretation of how the granted quota should be handled at reporting. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.3.1	Basic principles

When offline charging and online charging are both applicable to a service delivery, the charging information of both offline charging and online charging can be provided in one single command.

For converged charging the following applies:

-	a chargeable event may trigger charging information to be provided for both online and offline charging

-	request for quota and reporting of usage shall always done based on rating group

-	reporting of usage may be further divided on service identification

-	in the same request/report one rating group shall be either online or offline charged

-	the same units shall only to be reported once either as quota managed or not quota managed

-	in the case service delivery is continuing at the time of usage reporting, which is quota managed, the NF (CTF) shall request new quota independent if all the quota was used or not,

End of changes