3GPP TSG-SA5 Meeting #129-e	S5-201342

# Online, , 24th Feb 2020 - 4th Mar 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0185 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to tariffTimeChange with UTC format |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, 5GS_Ph1-SBI_CH |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 32.291 defined tariffTimeChange with data type of DateTime, refer to TS 29.571 which in turn refer to OpenAPI specification that follows ISO 8601. All the charging information and usage report related times has been defined with UTC time without offset in TS 29.244. This means that the TS 32.291 (using TS 29.571) defined DateTime that allows an UTC offset does not match TS 29.244 which doesn’t allow an UTC offset The Tariff-Time-Change AVP defined in TS 32.299, which refer to IETF RFC 8506 is specified as UTC time without offset |  |  |  |  |  |  |  |  |
| Summary of change: |  | All times: tariffTimeChange startTime, stopTime, timeofFirstUsage, timeofLastUsage, triggerTimestamp, reportTime, timeofFirstUsage and timeofLastUsage as UTC time with zero offset. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency between TS 32.291 and TS 29.244 is up for interpretation, and may lead to interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.1.11, 6.1.6.2.2.8, 6.1.6.2.2.9, 6.1.6.2.2.14,6.1.6.2.2.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

###### 6.1.6.2.1.11	Type GrantedUnit

Table 6.1.6.2.1.11-1: Definition of type GrantedUnit

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| tariffTimeChange | DateTime | OC | 0..1 | This field contains the switch time when the tariff will be changed, as UTC with zero offset. |  |
| time | Uint32 | OC | 0..1 | This field holds the amount of granted time. |  |
| totalVolume | Uint64 | OC | 0..1 | This field holds the amount of granted volume in both uplink and downlink directions. |  |
| uplinkVolume | Uint64 | OC | 0..1 | This field holds the amount of granted volume in uplink direction. |  |
| downlinkVolume | Uint64 | OC | 0..1 | This field holds the amount of granted volume in downlink direction. |  |
| serviceSpecificUnits | Uint64 | OC | 0..1 | This field holds the amount of granted requested service specific units. |  |

Second change

###### 6.1.6.2.2.8	Type PDUSessionInformation

Table 6.1.6.2.2.8-1: Definition of type PDUSessionInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| networkSlicingInfo | NetworkSlicingInfo | OM | 0..1 | information of network slice serving the PDU session |  |
| pduSessionID | PduSessionId | M | 1 |  |  |
| pduType | PduSessionType | OM | 0..1 | type of the PDU session |  |
| sscMode | SscMode | OC | 0..1 | information of SSC Mode type. |  |
| hPlmnId | PlmnId | OC | 0..1 | PLMN identifier of the home network |  |
| servingNetworkFunctionID | ServingNetworkFunctionID | Oc | 0..1 | This field holds serving Network Function identifier. |  |
| servingCNPlmnId | PlmnId | Oc | 0..1 | Serving Core Network Operator PLMN ID selected by the UE in shared networks. |  |
| ratType | RatType | OC | 0..1 | the RAT Type of the PDU session |  |
| dnnId | Dnn | M | 1 | a Data Network Name |  |
| dnnSelectionMode | DnnSelectionMode | OC | 0..1 | This field indicates how the DNN was selected. |  |
| chargingCharacteristics | string | OC | 0..1 | the Charging Characteristics for this PDU session. |  |
| chargingCharacteristicsSelectionMode | ChargingCharacteristicsSelectionMode | OC | 0..1 | information about how the "Charging Characteristics" was selected. |  |
| startTime | DateTime | OC | 0..1 | the time which represents the start of a PDU session at the SMF, as UTC with zero offset |  |
| stopTime | DateTime | OC | 0..1 | the time which represents the stop of a PDU session at the SMF, as UTC with zero offset |  |
| 3gppPSDataOffStatus | 3GPPPSDataOffStatus | OC | 0..1 | This field holds the 3GPP Data off Status when UE’s 3GPP Data Off status is Activated or Deactivated. |  |
| sessionStopIndicator | boolean | OC | 0..1 | This field indicates to the CHF that the PDU session has been terminated. |  |
| pduAddress | PDUAddress | OC | 0..1 | Group of user ip address/prefix |  |
| diagnostics | Diagnostics | OC | 0..1 | provides a more detailed cause value from SMF. |  |
| authorizedQoSInformation | AuthorizedDefaultQos | OC | 0..1 | This field holds the authorized QoS applied to PDU session. |  |
| subscribedQoSInformation | SubscribedDefaultQos | OC | 0..1 | This field holds the subscribed Default QoS |  |
| authorizedSessionAMBR | Ambr | OC | 0..1 | This field holds the authorized session-AMBR. |  |
| subscribedSessionAMBR | Ambr | OC | 0..1 | This field holds the subscribed session-AMBR. |  |

Third change

###### 6.1.6.2.2.9	Type PDUContainerInformation

Table 6.1.6.2.2.9-1: Definition of type PDUContainerInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| timeofFirstUsage | DateTime | OC | 0..1 | the time stamp for the first IP packet to be transmitted and mapped to the reporting used unit, as UTC with zero offset. |  |
| timeofLastUsage | DateTime | OC | 0..1 | the time stamp for the last IP packet to be transmitted and mapped to the reporting used unit, as UTC with zero offset. |  |
| qoSInformation | QoSData | OC | 0..1 | the QoS applied for the reporting used unit. In case gbrUl or gbrDl are present for GBR flow, the GBR targets are "GUARANTEED", otherwise, are " NOT_GUARANTEED". |  |
| qoSCharacteristics | QosCharacteristics | OC | 0..1 | Map of QoS characteristics for non standard 5QIs and non-preconfigured 5QIs. |  |
| afChargingIdentifier | ChargingId | OC | 0..1 | An identifier, provided from the AF, may be used to correlate the measurement for the Charging key/Service identifier values in this PCC rule with application level reports. |  |
| userLocationInformation | UserLocation | OC | 0..1 | provides information on the location |  |
| uetimeZone | TimeZone | OC | 0..1 | the UE Time Zone during the used unit container interval. |  |
| rATType | RatType | OC | 0..1 | the RAT Type of the used unit |  |
| servingNodeID | array(ServingNetworkFunctionID) | OC | 0..N | the list of serving node identifiers during the used unit container interval. |  |
| presenceReportingAreaInformation | map(PresenceInfo) | OC | 0..N | the Presence Reporting Area status of UE during the used unit container interval. |  |
| 3gppPSDataOffStatus | 3GPPPSDataOffStatus | OC | 0..1 | the 3GPP Data off Status during the used unit container interval. |  |
| sponsorIdentity | string | OC | 0..1 | an identifier of the sponsor. |  |
| applicationserviceProviderIdentity | string | OC | 0..1 | an identifier of the application service provider |  |
| chargingRuleBaseName | string | OC | 0..1 | the reference to group of PCC rules predefined at the SMF. |  |

Forth change

###### 6.1.6.2.2.14	Type MultipleQFIcontainer

Table 6.1.6.2.1.14-1: Definition of type MultipleQFIcontainer

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| triggers | array (Trigger) | Oc | 0..N | This field holds reason for closing the QFI unit container. |  |
| triggerTimestamp | DateTime | Oc | 0..1 | This field holds the timestamp when the reporting trigger occur, as UTC with zero offset. |  |
| time | Uint32 | OC | 0..1 | This field holds the amount of time. |  |
| totalVolume | Uint64 | OC | 0..1 | This field holds the amount of volume in both uplink and downlink directions. |  |
| uplinkVolume | Uint64 | OC | 0..1 | This field holds the amount of volume in uplink direction. |  |
| downlinkVolume | Uint64 | OC | 0..1 | This field holds the amount of volume in downlink direction. |  |
| localSequenceNumber | integer | M | 1 | QFI data container sequence number. It starts from 1 and increased by 1 for each container generation |  |
| qFIContainerInformation | QFIContainerInformation | OC | 0..1 | This field holds the QFI data container information |  |

Fifth change

###### 6.1.6.2.2.16	Type QFIContainerInformation

Table 6.1.6.2.1.16-1: Definition of type QFIContainerInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| qFI | Qfi | OM | 0..1 | QoS Flow Identifier (QFI) |  |
| reportTime | DateTime | M | 1 | the time stamp when the QFI data container was closed, as UTC with zero offset |  |
| timeofFirstUsage | DateTime | OC | 0..1 | the time stamp for the first IP packet to be transmitted and mapped to the QFI container, as UTC with zero offset |  |
| timeofLastUsage | DateTime | OC | 0..1 | the time stamp for the last IP packet to be transmitted and mapped to the QFI container, as UTC with zero offset. |  |
| qoSInformation | QoSData | OC | 0..1 | the QoS applied to QFI container. In case gbrUl or gbrDl are present for GBR QoS flow, the GBR targets are "GUARANTEED", otherwise, are " NOT_GUARANTEED". |  |
| qoSCharacteristics | QosCharacteristics | OC | 0..1 | Map of QoS characteristics for non standard 5QIs and non-preconfigured 5Qis. |  |
| userLocationInformation | UserLocation | OC | 0..1 | provides information on the location |  |
| uetimeZone | TimeZone | OC | 0..1 | UE Time Zone the UE is currently located |  |
| presenceReportingAreaInformation | map(PresenceInfo) | OC | 0..N | the Presence Reporting Area status of UE during the used unit container interval. |  |
| rATType | RatType | OC | 0..1 | the RAT Type of the used unit |  |
| servingNetworkFunctionID | array(ServingNetworkFunctionI) | OC | 0..N | the list of serving Node Identifiers during the used unit container interval. |  |
| 3gppPSDataOffStatus | 3GPPPSDataOffStatus | OC | 0..1 | the 3GPP Data off Status during the used unit container interval. |  |
| 3gppChargingId | ChargingId | OC | 0..1 | IP-CAN bearer Charging identifier used to identify this IP-CAN bearer in different records created by PGW-C+SMF. Charging Id is generated by P-GW at IP-CAN bearer activation and is included in all containers in order to identify the containers which pertain to the IP-CAN bearer. Only applicable for 5GS and EPS interworking. |  |
| diagnostics | Diagnostics | OC | 0..1 | provides a more detailed cause value for the release. Only applicable for 5GS and EPS interworking. |  |
| enhancedDiagnostics | array(string) | OC | 0..N | provides a set of causes for the release Only applicable for 5GS and EPS interworking. |  |

Editor’s Note: the diagnostics for interworking is ffs.

End of change