3GPP TSG-SA5 Meeting #130-e	S5-202111

- April 20 – 24, 2020								        											      		s5-2abcde

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0219 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add PCI related measurement of candidate cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SON_5G |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This measurement is needed to support the PCI configuration SON function to detect PCI collision or confusion issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the PCI related measurement of candidate cells. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The centralized PCI configuration SON function cannot be monitored. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x (new), A.58 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 5		Performance measurements for 5G Network Functions

## 5.x	Measurements related to UE Measurement Reports

### 5.x.1	PCI of candidate cells

a)	This measurement provides the PCI used by candidate cells. The measurement is derived from MeasResultListNR IE in measResultNeighCells (see clause 6.3.2 in TS 38.331 [20]) where it contains PhysCellId to indicate the PCI value(s) of the candidate cells, and contains subcounters per PCI combination to indicate the number of MeasResultListNR with such PCI value(s) being received.

b)	CC.

c)	This measurement is obtained by incrementing the subcounter, corresponding the PCI value(s) received in MeasResultListNR IE in measResultNeighCells. For example, MeasResultListNR contains one or multiple MeasResultNR with PCI value(s) physCellId PCI #m and PCI #n. Then, a subcounter identified by PCI #m and PCI#n will be incremented. The total number of subcounters equals to the number of PCI value combination received in a granularity interval.

d)	Each measurement is an integer value.

e)	MR.PCIsOfCandidateCell.Combo
Where Combo identifies the subcounter associated with the PCI value combination.

NOTE: To reduce the mumber of suncounters, a subcounter with superset of PCI values will be incremented for each MeasResultListNR with its subset. For example, a subcounter with PCI (1, 3, 6, 8, 10), will be incremented for MeasResultListNR with PCI (1, 3), PCI (1, 6), PCI (1, 3, 6), PCI (1, 3, 6, 8), …

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support centralized SON function of PCI re-configuration (see TS 28.313 [30]).

Next Modified Section

# A.58	Monitoring of PCI to detect PCI collision or confusion

Each NR cell is assigned a PCI that enables UE to uniquely identify the cell. PCI values need to be reused, as there are only 1008 PCI values. Typically, operators use network planning tool to assign PCIs to cells when the network is deployed to insure all neighbouring cells have different PCIs. However, due to the addition of new cells or changes of neighbour relations from ANR functions, issues can arise, such as PCI collision, PCI confusion, where

- PCI collision: Two neighbouring cells have the same PCIs;

- PCI confusion: A cell has the same PCI value as the neiighbour of its neiighbour.

NOTE: PCI Confusion or Conflict will only happen in case of cell having same PCI and same Frequency.

Figure A.58-1 shows an example about PCI confusion detection.

Figure A.58-1: PCI confusion example

A measurement can be created with subcounters to count the number of occurance of PCI value combination for Cell-IV:

Cell-IV measurement:

- Subcounter #1 (PCI 7, 9, 13): 1 (found by UE-A)

- Subcounter #2 (PCI 7, 9): 1 (found by UE-B)

- Subcounter #3 (PCI 10): 2 (found by UE-C and UE-E)

- Subcounter #4 (PCI 7, 14, 15): 1 (found by UE-D)

The centralized PCI configuration SON function can analize the measurement to detect a potential PCI issue where the same PCI value “7” is found in subcounter #1 with PCI 7, 9, 13 and subcounter #4 with PCI 7, 14, 15. It indicates that both Cell-IV’s neighboring cells are using the same PCI value “7’. The PCI configuration SON function can uses the NCR tables NRCellRelation [3] to confirm that it is the PCI confusion issue, as both Cell-IV’s neighboring cells – cell-III and cell-VI are using the same PCI value “7’.

Figure A.58-2 shows an example about PCI conflict detection.

Figure A.58-2: PCI conflict example

A measurement can be created with subcounters to count the number of occurance of PCI value combination for Cell-III:

Cell-III measurement

- Subcounter #1 (PCI 10, 11): 1 (found by UE-J)

- Subcounter #2 (PCI 10): 3 (found by UE-I, UE-F and UE-G)

- Subcounter #3 (PCI 10, 2): 1 (found by UE-H)

The centralized PCI configuration SON function can analize the measurement to detect a potential PCI issue where the same PCI value “10” is found in subcounter #1 with PCI 10 and 11 and subcounter #3 with PCI 10 and 2. It indicates that both Cell-III’s neighboring cells are using the same PCI value “10’.

The PCI configuration SON function then uses the NCR tables to confirm that it is the PCI conflict, since both Cell-IV and Cell-VIII are neighboring cells and are using the same PCI value “10”.

End of Modified Sections