3GPP TSG-SA5 Meeting #130e 	S5-202302

- e-meeting 20-28 April 2020

- Source:	Ericsson

- Title:	pCR R16 28308-030 QMC operations

- Document for:	Approval

Agenda Item:	6.4.1

# 1	Decision/action requested

Approve the proposal.

# 2	References

-

# 3	Rationale

This is the start of specifying the operations etc. for QMC.

# 4	Detailed proposal

First change.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.150: "Telecommunication management; Integration Reference Point (IRP) Concept and definitions".

[3]	3GPP TS 28.307: " Telecommunication management; Quality of Experience (QoE) measurement collection Integration Reference Point (IRP); Information Service (IS)".

[4]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[5]	3GPP TS 28.622: "Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[6]	3GPP TS 32.302: "Configuration Management (CM); Notification Integration Reference Point (IRP); Information Service (IS)".

[7]	3GPP TS 32.342: "File Transfer (FT) Integration Reference Point (IRP); Information Service (IS)".

[8]	3GPP TS 32.602: "Configuration Management (CM); Basic CM Integration Reference Point (IRP); Information Service (IS)".

[9]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[10]	3GPP TS 28.404: "Quality of Experience (QoE) measurement collection; Concepts, use cases and requirements"

[11]	3GPP TS 28.307:"Management of Quality of Experience (QoE) measurement collection Integration Reference Point (IRP); Requirements"

[x]	3GPP TS 32.312:"Generic Integration Reference Point (IRP) management; Information Service (IS)"

Next change.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1], 3GPP TS 32.150 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1], 3GPP TS 32.150 [2].

Next change.

#### 5.3.1.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serviceType | M | T | T | T | F |
| Area | M | T | T | T | F |
| qoECollectionEntityAddress | M | T | T | T | F |
| pLMNTarget | M | T | T | T | F |
| qMCTarget | M | T | T | T | F |
| qoEReference | M | T | T | T | F |
| qMCConfigurationFile | M | T | T | T | F |
| Attribute related to role |  |  |  |  |  |

Next change.

### 5.3.x	QMCRecord

### 5.3.x.1	Definition

TQMCRecord is the representation of the files containing the information from the network request session.

#### 5.3.x.2	Attributes

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| fileName | + | M | M | - |

### 5.3.y	QMCIRP

#### 5.3.y.1	Definition

QMCIRP is the representation of the QMC management capabilities specified by the present document. This IOC inherits from ManagedGenericIRP IOC specified in 3GPP TS 32.312 [x].

### 5.3.z	ManagedEntity

#### 5.3.z.1	Definition

For area based QMC the ManagedEntity represents the role that can be played by an instance of the following IOCs:

-	UtranCell

-	E-UtranCell

## 5.x	Information relationship definitions

### 5.x.1	relation-qMCIRP-qMCJob (M)

#### 5.x.1.1	Definition

This represents the relationship between QMCIRP and the QMCJob.

#### 5.x.1.2	Roles

| Name | Definition |
| --- | --- |
| theQMCIRP | It represents the QMCIRP |
| theQMCJobList | It represents the QMCJobList |

### 5.x.2	relation-qMCJob-managedEntity (M)

#### 5.x.2.1	Definition

This represents the relationship between QMCJob and the ManagedEntity.

#### 5.x.2.2	Roles

| Name | Definition |
| --- | --- |
| theManagedEntity | The ManagedEntity, when playing this role, represents the actual network resource instance, where a network request session is activated. |
| theQMCJob | It represents the QMCJob |

### 5.x.3	relation-qMCJob-qMCRecord (M)

#### 5.x.3.1	Definition

This represents the relationship between QMCJob and the QMCRecord.

#### 5.x.3.2	Roles

| Name | Definition |
| --- | --- |
| theQMCJob | It represents the QMCJob |
| theQMCRecord | It represnts the QMCRecord. |

Next change.

## 5.y	Attribute definitions

### 5.y.1		Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| serviceType | It identifies an individual UE for a specified end user service/end user service type. allowedValues: DASH (0), MTSI (1) | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| area | The area scope parameter defines the area in terms of cells or Tracking Area/Routing Area/Location Area where the QMC shall take place. allowedValues: The area scope parameter in UMTS is either: - List of cells, identified by CGI. Maximum 32 CGI can be defined. - List of Routing Area, identified by RAI. Maximum of 8 RAIs can be defined. - List of Location Area, identified by LAI. Maximum of 8 LAIs can be defined. The area scope parameter in LTE is either: - list of cells, identified by E-UTRAN-CGI. Maximum 32 CGI can be defined. - List of Tracking Area, identified by TAC. Maximum of 8 TAC can be defined. | type: String multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| qoECollectionEntityAddress | It indicates the IP address to which the QMC records shall be transferred. Ipv4 or Ipv6 address(es) may be used. allowedValues: N/A | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pLMNTarget | This parameter defines the PLMN for which sessions shall be selected in the network request session in case of area based QMC when several PLMNs are supported in the RAN (this means that shared cells and not shared cells are allowed for the specified PLMN. Furthermore, several PLMNs can be used for not shared RAN cases as well as for shared RAN cases.). Only the sessions may be selected where the PLMN that the UE reports as selected PLMN is the same as the PLMN Target. Note that the PLMN Target might differ from the PLMN specified in the Network Request Session Id, as that specifies the PLMN that is containing the management system requesting the Trace Session from the NE. | type: PLMNId multiplicity: 1 isOrdered: N/A isUnique: …N/A defaultValue: None isNullable: False |
| qMCTarget | The QMC target parameter specifies it the QMC is area based or individual UE based. - Area based QMC (0) - NOTE: Individual UE based QMC is not supported in this release. I allowedValues: … | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qoEReference | The QoE reference parameter specify the network request session. The QoE reference shall be globally unique therefore it is composed as follows: MCC+MNC+QMC ID, where the MCC and MNC are coming with the trace activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3 byte Octet String. The QMC ID is generated by the management system or the operator. It is used to identify the QoE measurement collection job in the traffic nodes and in the measurement collection centre. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| qMCConfigurationFile | The QMC configuration file is a container that is specified in 3GPP TS 26.247 and 3GPP TS 26.114 allowedValues: N/A | type: xxx multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

te related to role". See example below.

Next change.

### 5.y.2		Constraints

This clause indicates whether there are any constraints affecting attributes. Each constraint is defined by a triplet (propertyName, affectedAttributes, propertyDefinition). PropertyDefinitions are expressed in natural language.

An example is given here below:

| Name | Affected attribute(s) | Definition |
| --- | --- | --- |
| inv_TimerConstraints | ntfTimeTickTimer | The ntfTimeTickTimer is lower than or equal to ntfTimeTick. |

This clause shall state "None." if there is no constraint.

Next change.

## 5.z	Common Notifications

This clause presents notifications that can be referred to by any class defined in the specification. This information is provided in tables.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAttributeValueChange | O | Example common notification |
| notifyObjectCreation | O | Example |
| notifyObjectDeletion | O | Example |

This clause shall state "None." if there are no common notifications.

Next change.

### 5.z.1		Alarm notifications

The following quoted text shall be copied as the only paragraph of this clause.

"This clause presents a list of notifications, defined in [x], that IRPManager can receive. The notification header attribute objectClass/objectInstance, defined in [y], shall capture the DN of an instance of a class defined in this specification."

The information is provided in a table. The following is an example.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyNewAlarm | M | -- |

Next change.

### 5.z.2		Configuration notifications

The following quoted text shall be copied as the only paragraph of this clause.

"This clause presents a list of notifications, defined in [x], that IRPManager can receive. The notification header attribute objectClass/objectInstance, defined in [z], shall capture the DN of an instance of a class defined in this specification."

The information is provided in a table. The following is an example.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAttributeValueChange | O | -- |
| notifyObjectCreation | O | -- |
| notifyObjectDeletion | O | -- |

Next change.

### 6.3.1		Operation activateAreaQoEJob (M)

#### 6.3.1.1	Definition

This operation support IRPManager’s request to create an AreaQoEJob through Itf-N.

Once the AreaQoEJob has been created, the attributes of the AreaQoEJob will not be modified during the lifetime of the AreaQoEJob.

One AreaQoEJob can manage QoE Collection Sessions in one or more ManagedEntity.

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 28.307 [3] | REQ-EUSPC-FUN-1 |  |

#### 6.3.1.2	Input parameters

| Parameter Name | Qualifier | Information type | Comment |
| --- | --- | --- | --- |
| iOCInstance | M | ManagedEntity.objectInstance | It specifies the DN of ManagedEntity instance where QMC session is to be activated. |
| qoEReference | M | QMCJob.qoeReference | It identifies the QMC Session. |
| qMCTarget | M | QMCJob.traceTarget | It specifies if the QMC is areabased or individual UE based. In the present release only areabased is supported. |
| qoECollectionEntityAddress | M | QMCJob.qoECollectionEntityAddress | It specifies the address to the QoE Collection Entity that is associated to the QoEJob. See 3GPP TS 32.422 [9]. |
| serviceType | M | QMCJob.serviceType | It specifies which service is to record information. |
| areaScope | M | QMCJob.areaScope | It specifies MDT area (Cells/TA/RA/LA) where the Logged MDT measurements or Logged MBSFN MDT shall be collected. It specifies one or list of eNBs where the RLF or RCEF reports shall be collected. It specifies MDT area (list of cells) where the Immediate MDT measurements shall be collected. |
| pLMNTarget | CM | QMCJob.pLMNTarget | It specifies which PLMN that the subscriber of the session to be recorded uses as selected PLMN. |
| qMCConfigurationFile | M | QMCJob.qMCConfiguration.File | It specifies the configuration of the QoE measurement to be recorded. See TS 26.247 [4] for DASH and TS 26.114 [9] for MTSI. |

ow:

| Parameter Name |  |  |  |
| --- | --- | --- | --- |

6.3.1.3	Output parameters

| Parameter Name | Qualifier | Matching Information | Comment |
| --- | --- | --- | --- |
| status | M | ENUM (Success, Failure, PartialSuccess) |  |
| unsupportedList | M | List of <ManagedEntity, qMCTarget, areaScope, serviceType, pLMNTarget) | It specifies what attributes are not supported when a QMC session is activated. The list can contain one or all of the elements and relevant only for error cases. |

ow:

End of changes.