3GPP TSG-SA5 Meeting #131e 	S5-203142rev3

- e-meeting 25th May-3rd June 2020

- Source:	Huawei

- Title:	Add use case for 5GC service assurance

- Document for:	Approval

Agenda Item:	6.4.7

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	3GPP TS 28.535: Management and orchestration; Management Services for Communication Service Assurance; Requirements

# 3	Rationale

This contribution is to add the use case of 5G core for communication service assurance

# 4	Detailed proposal

First change

### 6.1.x		5G Core assisted SLS communication service Assurance

The goal of this use case is to describe 5G Core management to assure compliance to SLS (Service Level Specifications) for a particular communication service in 3GPP management system.The pre-condition of this management to 5GC to ensure SLS is that the configurations to the control plan functions and resource configurations related to 5GC is available for use of SLS communication service assurance..

3GPP management system receives the SLS requirements that required by CSP or NOP. 3GPP management system is capable to translate e2e SLS goal and set the 5GC goal(s) of SLS related to 5GC and activate control loop for particular service assurance goal(s). To fulfill the SLS requirements, 3GPP management system is capable to configure the management resource and 5GC network functions (e.g. AMF, SMF, NWDAF) to monitor particular measurements and fault alarms that are relevant to the SLS. Since, for example, an network slice for eMBB can provide multiple communications services, one or multiple control loop for service assurance goals are set, and the network resource and performance measurements which are relevant to the SLS.

During the process of service assurance of 5GC, the 5GC domain MDAS provider can be used to provide analysis of 5GC related network resource, virtual resource and performance assurance related to  SLS in 5GC. The 5GC domain analysis report may be provided to 3GPP management system as part of the analysis result(s) of 5GC SLS.

Together with the report from NWDAF, performance measurements and fault alarms related to 5GC NFs are also available for analysis of any potential service degradation.

Second change

## 6.2	Requirements

REQ-CSA-CON-01 The 3GPP management system shall have the capability to take actions for a set of communication services serving certain group of UEs based on the target SLS.

REQ-CSA-CON-02 The 3GPP management system shall have the capability to collect service experience information.

REQ-CSA-CON-03 The 3GPP management system shall have the capability to analyse the performance information related to the set of communication services serving certain group of UEs.

REQ-CSA-CON-04 The 3GPP management system shall have the capability to modify the configuration parameters related to the set of communication services serving certain group of UEs.

REQ-CSA-CON-05	The 3GPP management system shall have the capability to collect NSI related QoE data from NWDAF.

REQ-CSA-CON-06	The 3GPP management system shall have the capability to derive CSI related QoE data from the collected NSI related QoE data.

REQ-CSA-CON-07	The 3GPP management system shall have the capability to ascertain SLS breach.

REQ-CSA-CON-08	The 3GPP management system shall have the capability to perform the root cause analysis (e.g., identifying the underlying reason) for an SLS breach.

REQ-CSA-CON-09	The 3GPP management system shall have the capability to take corrective actions against the root cause identified.

REQ-CSA-CON-x The 3GPP management system shall have the capability to translate communicate service requirements to cross domain SLS goal and single domain SLS goal.

REQ-CSA-CON-y The 3GPP management system shall have the capability to collect single domain SLS analysis as input to cross domain SLS analysis.

NOTE: The management system refers to the producer of management service for SLS assurance.

End of change