3GPP TSG-SA5 Meeting #132e 	S5-204057rev1

- e-meeting 17th 28th August 2020

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Update the Charging Architecture

- Document for:	Approval

Agenda Item:	7.4.2

# 1	Decision/action requested

This is a pCR to TS 28.202 to Update Charging Architecture

# 2	References

[1]	3GPP TS 28.202 "Charging management; Network slice management charging in the 5G System (5GS); Stage 2".

[2]	S5-204081 DP on Common Charging architecture

# 3	Rationale

This pCR proposes to Update Charging Architecture based on conclusion from DP S5-204081 [2]

Change Bnsm to Bns.

# 4	Detailed proposal

The following changes are proposed to be incorporated into TS 28.202 [1]

First change

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Bns	Reference point for the CDR file transfer from the Network slice  CGF to the BD.

Ga	Reference point for CDR transfer between a CDF and the CGF.

Nchf	Service based interface exhibited by CHF.

Next change

### 4.2.2	Converged charging architecture

Network Slice Management converged charging can be achieved under two alternatives:

-	Charging Trigger Function (CTF) based, as depicted in figure 4.2.2.1.

-	Charging Proxy Function (CPF) based, depicted in figure 4.2.2.2.

Figure 4.2.2.1: Network Slice Management converged charging architecture (CTF)

Figure 4.2.2.2: Network Slice Management converged charging architecture (CPF)

The Charging Trigger Function (CTF) is specified in TS 32.240 [1].

The Charging Proxy Function (CPF) is defined in TS 28.201 [70]:  charging information addressed by the CPF in the present document, are related to provisioning for network slices.

The MnS producer is defined in TS 28.533 [250]: the MnS producer in the present document is the producer of provisioning MnS.

For network slice management converged charging the CPF is a consumer of both:

-	provisioning service (MnS) for network slice exposed by the MnS Producer, specified in TS 28.532 [253];

-	charging (Nchf) service.

Details on the interfaces and functions can be found in TS 32.240 [1] for the general architecture components. Ga is described in clause 5.2.4 and Bns in clause 5.2.5 of the present document, and Nchf is described in TS 32.290 [50].

Next change

### 5.2.5	Bns CDR file transfer

Details of the Bns protocol application are specified in TS 32.297 [56].

Next change

### 6.1.2	CDR description on the Bns interface

End of change