- 3GPP TSG SA WG5 Meeting #132-e	S5-204109

- August 17 – 28, 2020	revision of S5-204xxx

- Source:	Intel, Samsung

- Title:	pCR 28.814 use cases and requirements for ECS deployment

- Document for:	Approval

Agenda Item:	6.6.6

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TR 28.814 “Study on enhancements of edge computing management”

[2] 	3GPP TS 23.558: "Architecture for enabling Edge Applications;"

# 3	Rationale

This contribution proposes to add the use cases and requirements for ECS deployment to TR 28.814 [1]. The deployment of ECS is defined in Figure 1 in TS 23.558 [2].

Figure 1: Architecture for enabling edge applications

# 4	Detailed proposal

1st modified section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 23.558: "Architecture for enabling Edge Applications;".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

Next modified section

# 5	Use cases

## 5.1	Business level use cases

## 5.1.x	The deployment of edge configuration server(s)

#### 5.1.x.1	Goal

The goal is to enable the instantiation of one or more ECS in PLMN operator (see clause 6.5.2.3 in TS 23.558 [2]).

##### 5.1.x.1.2	Description

###### 5.1.x.1.2.1	ECS instantiation

1.	The PLMN operator receives a request to instantiate one or more ECS.

2.	 The PLMN operator instantiates the ECS.

3.	 The PLMN operator sends a notification to indicate that the ECS has been instantiateed successfully.

###### 5.1.x.1.2.2	ECS termination

1.	The PLMN operator receives a request to termination one or more ECS with the ECS identifier.

2.	The PLMN operator terminates the ECS based on the ECS identifier.

3.	 The PLMN operator sends a notification to indicate that the ECS has been terminated successfully.

#### Note: The above use case also applies when the ECS is to be deployed by ECSP.

## 5.2	Specification level use cases

Editor’s note: this clause is to describe the use cases associated with the management services to support orchestration and management of edge computing.

### 5.2.x	ECS Instantiation

#### 5.2.x.1	Goal

The goal is to enable the instantiation of one or more ECS in PLMN operator (see clasue 6.5.2.3 in TS 23.558 [2]).

##### 5.2.x.2	Description

To support deploted EDN, PMN operator will deploy ECS serving one or multiple EES.

1.	A consumer request for EES(s) instantiation providing EES deployment requirements.

2.	The PLMN management system performs the following operations to instantiate the ECS:

-	Instantiate the ECS VNF.

-	Connect the ECS to the 5GC network functions.

Note: Note: Connecting ECS with 5GC network function will enable information exchange over EDGE-8.

3.	The PLMN management system sends a notification to indicate that the ECS has been instantiateed successfully.

##### 5.2.x.2	Requirements

REQ-ECS-DEPLOY-FUN-1 3GPP management service producer shall have the capability to instantiate the ECS, as per request from consumers.

REQ-ECS-DEPLOY-FUN-32 3GPP management service producer shall have the capability to send the notification indicating the status of ECS Instantiation.

REQ-ECS-DEPLOY-FUN-3: The 3GPP management producer system shall have the capability to relate instantiated ECS with one or multiple EES which it will be serving.

### 5.2.y	ECS Termination

##### 5.2.x.1	Goal

The goal is to enable the termination of one or more ECS in PLMN operator (see clasue 6.5.2.3 in TS 23.558 [2]).

##### 5.2.x.2	Description

1. A consumer consumes the management service of PLMN management system to terminate the ECS with the ECS identifier.

2. The PLMN management system terminates the ECS VNF based on the ECS identifier, and disconnects the ECS from the 5GC network functions.

3. The PLMN management system sends a notification to indicate that the ECS has been terminated successfully.

##### 5.2.x.23	Requirements

REQ-ECS-DEPLOY-FUN-1 3GPP management service producer shall have the capability to terminate the ECS with the ECS identifier, as per request from consumers.

REQ-ECS-DEPLOY-FUN-2 3GPP management service producer shall have the capability to send the notification indicating the status of EAS termination.

End of modified section