- 3GPP TSG SA WG5 Meeting #132-e	S5-204110

- August 17 – 28, 2020	revision of S5-20abcd

- Source:	Intel

- Title:	pCR 28.814 add solutions for EDN deployment

- Document for:	Approval

Agenda Item:	6.6.6

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TR 28.814 v0.0.0 “Study on enhancements of edge computing management”

# 3	Rationale

This contribution proposes to add the solutions for the EDN deployment to TR 28.814 [1].

# 4	Detailed proposal

1st modified section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 23.558: "Architecture for enabling Edge Applications;".

[y]	ETSI GS NFV-IFA 013 V3.4.1 “Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Os-Ma-nfvo reference point -Interface and Information Model Specification”.

[w]	ETSI GS NFV-IFA 030 V3.4.1 “Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Multiple Administrative Domain Aspect Interfaces Specification”.

[v]	ETSI GS NFV-IFA 011 V3.3.1 “Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; VNF Descriptor and Packaging Specification”.

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

Next modified section

# 6	Potential solutions

Editor’s note: this clause is to describe the potential solution to support orchestration and management of edge computing.

## 6.x	The deployment of UPFs and EDN NFs

### 6.x.1	General

Figure 6.x.1-1 shows an example of implementing the edge computing network as described in Figure 4.x-2, in the virtualization environment, based on ETSI NFV MANO [y]. While it is possible to implement the entire edge computing networks in a NS (Network Service) in NFV, this approach may not be scalable, as the tightly coupled networks prevent the EDN from scaling. For example, an EDN may want to adjust the virtualized resources of the EDN according to the demand (e.g. the number of EAS running on an EDN) that is independent of the mobile networks. Therefore, separate NS(s) are used to allow different service providers to deploy EDNs independently.

Figure 6.x.1-1 shows that the edge computing networks is realized by the composite NS #1 that contains nested NS #2, NS #3, and NS #4 to implement the mobile networks, EDN #1 and EDN #2, respectively. The VNFs in a NS is connected by the virtual links in the NS. The VNFs in different NS(s) are connected by the NS virtual links.

Figure 6.x.1-1: The realization of edge computing networks in network services

It is assumed that NS #2, based on NSD #2, has been instantiated to provide typical mobile services to UEs. Therefore, the solutins described in the following subclauses focus on the LCM of EDN VNFs (e.g. EAS, EES, ECS) and UPFs in 5GC. It covers the deployment senarios for one or multiple service provider(s).

### 6.x.2	LCM by a service provider

The deployment of mobile networks and EDN may be done by a service provider, as the ECSP and the PLMN operator can be part of the same organization (see Annex B in TS 23.558 [x]). Figure 7.x.2-1 depicts a scenario where a PLMN/EC service provider contains the The management system of PLMN/EC operator and ETSI NFV MANO to perform LCM operations to create NS instances, as described in Figur 7.x,1-1.

Figure 6.x.2-1: Management systems for a single service provider

It is assumed that NS #2 identified by nsInstanceId #2 has been instantiated, and the management system of PLMN/EC operator is the master to create the composite NS #1.

The management system of PLMN/EC operator has received a request from a consumer instantiate NS #3 and NS #4 for EDN #1 and EDN #2 respectively.

The management system of PLMN/EC operator requests the NFVO in ETSI NFV MANO to instantiate NS #3, containing VNFs of EAS #1-1, EAS #1-2, EES #1 for EDN#1.

The management system of PLMN/EC operator requests the NFVO in ETSI NFV MANO to instantiate NS #4, containing VNFs of EAS #2-1, EAS #2-2, EES #2 for EDN#2.

The management system of PLMN/EC operator requests the NFVO in ETSI NFV MANO to instantiate NS #1 that contains nested NS #2, NS #3, and NS #4 for mobile networks, EDN #1, and EDN #2, respectively.

### 6.x.3	LCM by multiple service providers with an ETSI NFV MANO

Figure 7.x.3-2 addresses a scenario where one PLMN provider and two ECSP contains separate management system and one ETSI NFV MANO to perform LCM operations to create NS instances, as described in Figur 7.x.1-1. This common ETSI NFV MANO may reside in a cloud service provider (e.g. AWS).

Figure 6.x.3-1: Management systems for multiple service providers with an ETSI NFV MANO

It is assumed that NS #2, identified by nsInstanceId #2 has been instantiated, and PLMN management system is the master to create the composite NS #1.

The instantiation of NS #3 for EDN #1

The management system of ECSP #1 receives a request from a consumer to instantiate NS #3 for EDN #1.

The management system of ECSP #1 operator requests NFVO in ETSI NFV MANO to instantiate NS #3, containing VNFs of EAS #1-1, EAS #1-2, EES #1 for EDN#1.

The management system of ECSP #1 consumes the management service of the management system of PLMN operator to connect EDN #1 with the mobile network (e.g. EAS to UPF, EES to PCF), based on the QoS information of N6 interface.

The management system of PLMN operator requests the NFVO in ETSI NFV MANO to connect EAS to UPF, and EES to PCF, based on NS virtual link,

The management system of PLMN operator requests the NFVO in ETSI NFV MANO to instantiate NS #1 that contains nested NS #2 and NS #3 for mobile networks and EDN #1.

The instantiation of NS #4 for EDN #2

The management system of ECSP #2 receives a request from a consumer to instantiate NS #4 for EDN #2.

The management system of ECSP #2 operator requests NFVO in ETSI NFV MANO to instantiate NS #4, containing VNFs of EAS #2-1, EAS #2-2, EES #2 for EDN#2.

The management system of ECSP #2 consumes the management service of the management system of PLMN operator to connect EDN #2 with the mobile network (e.g. EAS to UPF, EES to PCF), based on the QoS information of N6 interface.

The management system of PLMN operator requests the NFVO in ETSI NFV MANO to connect EAS to UPF, and EES to PCF, based on NS virtual link,

The management system of PLMN operator requests the NFVO in ETSI NFV MANO to update NS #1 to contain nested NS #2, NS #3, and NS #4 for mobile networks, EDN #1, and EDN #2, respectively.

NOTE: whether the NFVO can support multi-teancy is FFS.

### 6.x.4	LCM by multiple service providers with dedicated ETSI NFV MANO

Figure 7.x.4-2 addresses a scenario where one PLMN provider and two ECSP contains separate management system with a dedicated ETSI NFV MANO to perform LCM operations to create NS instances, as described in Figur 7.x.1-1.

Figure 6.x.4-1: Management systems for multiple service providers with dedicated ETSI NFV MANO

It is assumed that NS #2, identified by nsInstanceId #2 has been instantiated, and PLMN management system is the master to create the composite NS #1.

The instantiation of NS #3 for EDN #1

The management system of ECSP #1 receives a request from a consumer to instantiate NS #3 for EDN #1.

The management system of ECSP #1 operator requests NFVO in ETSI NFV MANO of ECSP #1 to instantiate NS #3, containing VNFs of EAS #1-1, EAS #1-2, EES #1 for EDN#1.

The management system of ECSP #1 consumes the management service of the management system of PLMN operator to connect EDN #1 with the mobile network (e.g. EAS to UPF, EES to PCF), based on the QoS information of N6 interface.

The management system of PLMN operator requests the NFVO in ETSI NFV MANO of PLMN operator to connect EAS to UPF, and EES to PCF, based on NS virtual link,

NFVO-C in ETSI NFV MANO of PLMN operator request NFVO-N in ETSI NFV MANO of ECSP #1 to connect connect EAS to UPF, and EES to PCF, based on NS virtual link [w].

NOTE: whether a composite NS can contain two NS instances that reside in different service providers with its own NFV MANO is FFS.

The instantiation of NS #3 for EDN #2

The management system of ECSP #2 receives a request from a consumer to instantiate NS #4 for EDN #2.

The management system of ECSP #2 operator requests NFVO in ETSI NFV MANO of ECSP #2 to instantiate NS #4, containing VNFs of EAS #2-1, EAS #2-2, EES #2 for EDN#2.

The management system of ECSP #2 consumes the management service of the management system of PLMN operator to connect EDN #2 with the mobile network (e.g. EAS to UPF, EES to PCF), based on the QoS information of N6 interface.

The management system of PLMN operator requests the NFVO in ETSI NFV MANO of PLMN operator to connect EAS to UPF, and EES to PCF, based on NS virtual link,

NFVO-C in ETSI NFV MANO of PLMN operator request NFVO-N in ETSI NFV MANO of ECSP #2 to connect connect EAS to UPF, and EES to PCF, based on NS virtual link.

End of modified section