- 3GPP TSG SA WG5 Meeting #133-e	S5-205035

- October 12 – 21, 2020

- Source:	Samsung, Intel

- Title:	pCR 28.814 solution for EAS deployment

- Document for:	Approval

Agenda Item:	6.5.3

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TR 28.814 “Study on enhancements of edge computing management”

[2] 	3GPP TS 23.558: "Architecture for enabling Edge Applications;"

# 3	Rationale

# 4	Detailed proposal

1st modified section

# 7. Potential solutions

Editor’s note: this clause is to describe the potential solutions to support orchestration and management of edge computing.

7.x 		EAS Deployment using provisioning MnS

### 7.x.1	General

Figure 7.x.4-1 shows that the ASP requests the EAS instantiation via the provisioning MnS.

Figure 7.x.1-1: EAS lifecycle management via provisioning MnS

To support EAS deployment via provisioning MnS, the following IOCs should be defined:

-  EASLcm IOC that contains the requirement for EAS deployement:

- Location: Target location for the EAS to be available at: for example geolocation (longitude, latitude); access network defined (e.g. Cell IDs, Tracking Areas); network defined (e.g. a specific EDN(s)), civic address.

- Latency: The required service latency to be achieved by EAS (e.g. round trip latency between potential application clients and the EAS).

- Maximum cost: Maximum cost for getting the EAS on the edge. Cost can be based on different criteria e.g cost per unit of time, number of application consumer.

- Affinity/Anti-affinity: The affinity and ant-affinity requirements for the EAS with other existing EAS on the target EDN.

- Service requirements, internal to the EDN (e.g. from the Edge Enabler Server (EES)) or externally (e.g. from the 3GPP 5GC).

- 3GPP network capability exposure: Whether 3GPP network capabilities (e.g. access to traffic influence, UE location capabilities) are required from the EDN.

- Service continuity support: Whether the service continuity (i.e. user context transfer for stateful applications) is supported among different instances of EAS deployed in the same or different EDNs.

- Virtual Resource requirements: Requirements for virtual resource e.g. CPU, memory, storage.

-  EASLcm IOC ID: the identifier of the EASLcm IOC.

-  Service area: the geographical service area that the EAS serves (see clasue 8.2.4 in [2]), for example geolocation (longitude, latitude); access network defined (e.g. Cell IDs, Tracking Areas); network defined (e.g. a specific EDN(s)), civic address.

-  EAS VNF information: The information needed to instantiate the EAS VNF.

-  Software image information incuding location, minimum RAM and disk requirements for the software image. (see clause 7.1.6.5 in in ETSI NFV IFA-011 [x])

-  Virtual compute resources including virtual CPU, virtual memory and virtual disk

-  QoS requirements: e.g. bandwidth.

-

- Affinity/Anti-affinity: The affinity and ant-affinity requirements for the EAS with other existing EAS on the target EDN.

-

- 3GPP network capability exposure: Whether 3GPP network capabilities (e.g. access to traffic influence, UE location capabilities) are required from the EDN.

- Service continuity support: Whether the service continuity (i.e. user context transfer for stateful applications) is supported among different instances of EAS deployed in the same or different EDNs.

-  EASFunction IOC that contains the following, but not limited to, attributes:

-  EASLcm IOC ID: the identifier of EASLcm IOC.

-  EASFunction IOC ID: the identifier of EAS VNF instance

-  EAS profile information elements (see clasue 8.2.4 in [2]).

### 7.x.2	EAS instantiation

The following procedures creates/instantiate EAS including selecting appropriate EDN to be used. The procedure is used by ASP (Application Service Provider, see TS 23.558) to host their application on an available EDN.

Figure 7.x.1-1: EAS lifecycle management via provisioning MnS

Note: The Provisioning MnS is exposed by the management system of entity responsible for EAS lifecycle management e.g ECSP management system.

- ASP consumes the provisioning MnS and invoke operation createMOI for EAS?????Lcm IOC to request the provisioning MnS Producer to instantiate the EAS.

- Provisioning MnS Producer selects the appropriate EDN (or set of EDNs) based on the EAS requirements provided in the request. The EDN can be selected either by considering the individual requirement or by grouping the multiple requirement as single selection criteria. When the grouping is used all the requirement specified as part of a group should be satisfied. Following are examples of various grouped selection criteria:

  - Latency and Cost can be combined as one selection criteria influencing the selection as in what latency can be assured in what cost.

  - Geo-location and Latency can be combined as one selection criteria influencing the selection as in at what location the requested latency can be provided depending on the target geographical area.

  - Affinity/Anti-affinity and Cost can be combined as one selection criteria influencing the selection as in what cost will be incurred to satisfy Affinity/Anti-affinity with a particular existing Edge Application.

- Provisioning MnS Producer selects the appropriate EES if available in the selected EDN based on the information provided in the request.

- ECSP Management System derives the requirements for VNF instance based on the EAS requirements.

- ECSP Management System requests NFVO via the Os-Ma-nfvo interface to instantiate EAS VNF instances.

- Provisioning MnS Producer creates the MOI for EASFunction IOC. Function

- Provisioning MnS Producer sends notifyMOICreation to notify ASP about the newly created MOIs for EASLcm IOC and EASFunction IOC.

- Provisioning MnS Producer sends notifyMOICreation to notify the consumer (ECSP) that MOIs for EASLcm IOC and EASFunction IOC have been created.

NOTE: It is assumed that ASP and ECSP consumer have subcribed to receive notification from ECSP management system.

### 7.x.3	EAS termination

ASP consumes the provisioning MnS with operation deleteMOI to request the Provisioning MnS Producer to terminate the EAS instance(s), including (but not limited to) the following attributes:

-  EAS VNF instance ID: the identifier of MOIs for EASFunction IOC indicating the EAS VNF instance(s) to be terminated.

ECSP management system will perform the following;

-  requests NFVO to terminate the VNF instances based on the EAS VNF instance ID.

-  deletes MOIs for EASFunction IOC.

- sends notifyMOIDeletion to notify the ASP that MOI for EASFunction IOC has been deleted.

- sends notifyMOIDeletion to notify the consumer (ECSP) that MOI for EASFunction IOC has been deleted.

NOTE: It is assumed that ASP and ECSP consumer have subcribed to receive notification from ECSP management system.

End of modified section