3GPP TSG-SA5 Meeting #134e	S5-206078

November 16 – 25, 2020, E-meeting 	s5-2abcde

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | 0008 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct the PCI notifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SON_5G |  |  |  |  |  | Date: |  |  | 2020-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The PCI collision and PCI confusion alarms in clause 7.1.3.3.1 have been changed to PCI collision and PCI confusion notifications. But, the requirements, use case, and solution for PCI configuration are still based on fault supervision. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change faulty supervision MnS to provisioning MnS in the requirements, use case, and solution for PCI configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification has inconsistency errors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.4, 6.4.1.4.2, 7.1.3.3, 7.2.1.3, and 8.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Modified Sections

#### 6.1.1.4	PCI configuration and re-configuration

REQ-DPCI-CONFIG-FUN-1 producer of provisioning MnS should have a capability allowing an authorized consumer to set or update the list(s) of PCI value(s) for NR cell(s).

REQ-DPCI-CONFIG-FUN-2 producer of provisioning MnS should have a capability allowing an authorized consumer to enable or disable the PCI configuration function.

REQ-DPCI-CONFIG-FUN-3 producer of provisioning MnS should have a capability to notify the authorized consumer with the PCI value(s) being selected for NR cell(s).

REQ-DPCI-CONFIG-FUN-4 producer of provisioning MnS should have a capability to notify the authorized consumer about the detection or resolution of PCI collision or PCI confusion problems for NR cells.

REQ-DPCI-CONFIG-FUN-5 producer of provisioning MnS should have a capability allowing an authorized consumer to configure or re-configure the PCI list at the PCI configuration function.

Next Modified Sections

##### 6.4.1.4.2	PCI re-configuration

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To automatically re-configure the PCIs of NR, due to the PCI collision or PCI confusion problems. |  |
| Actors and Roles | D-SON management function to support PCI re-configuration. |  |
| Telecom resources | - gNB; - The producer of provisioning MnS - The producer of fault supervision MnS |  |
| Assumptions | N/A |  |
| Pre-conditions | - 5G NR cell(s) have being assigned with PCI value(s). - The PCI configuration function is in operation, and enabled. |  |
| Begins when | PCI configuration function has detected the PCI problem of PCI collision or PCI confusion for NR cell(s). |  |
| Step 1 (M) | The D-SON management function receives a notification from the producer of provisioning MnS indicating the PCI collision or PCI confusion problems being detected for an NR cell(s). |  |
| Step 2 (M) | The D-SON management function requests the producer of provisioning MnS to re-configure the PCI list at the PCI configuration function. |  |
| Step 3 (M) | The PCI configuration function selects PCI value(s) from the PCI list. |  |
| Step 4 (M) | The producer of provisioning MnS notifies the consumer with the PCI value(s) being assigned for the NR cell(s). |  |
| Step 5 (M) | The D-SON management function receives a notification from the producer of provisioning MnS indicating the PCI collision or PCI confusion problems being resolved. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The PCI collision or PCI confusion have been resolved. |  |
| Traceability | REQ-DPCI-CONFIG-FUN-3, REQ-DPCI-CONFIG-FUN-4, REQ-DPCI-CONFIG-FUN-5 |  |

Next Modified Sections

#### 7.1.3.3	MnS Component Type C definition

##### 7.1.3.3.1	Notification information

Table 7.1.3.3-1: PCI notifications

| Notifications | Description | Note |
| --- | --- | --- |
| PCI collision notification | The collision notification is used to indicate two neighbouring cells of a serving cell are using the same PCIs. |  |
| PCI Confusion notification | The confusion notification is used to indicate that a serving cell has 2 neighbouring cells that are using the same PCI value. |  |

Next Modified Sections

#### 7.2.1.3	MnS Component Type C definition

##### 7.2.1.3.1	Notifications information

The table below lists the notifications related to PCI configuration are generated from the NR cells.

Table 7.2.1.3.1-1: PCI notificationa

| Notifications | Description | Note |
| --- | --- | --- |
| PCI collision notification | The collision notification is used to indicate two neighbouring cells of a serving cell are using the same PCIs. |  |
| PCI Confusion notification | The confusion notification is used to indicate that a serving cell has 2 neighbouring cells that are using the same PCI value. |  |

Next Modified Sections

#### 8.2.3.2	PCI re-configuration

Figure 8.2.3.2-1 depicts a procedure that describes how D-SON management function can re-configure the PCI list for NR cell(s) when PCI collision or PCI confusion issues were detected.

Figure 8.2.3.2-1: PCI re-configuration procedure

1. The PCI configuration (D-SON) function detects and reports the PCI collision or PCI confusion problems for NR cell(s) to MnS of fault supervision (NOTE).

2. The producer of provisioning MnS sends a notification notifyMOIAttributeValueChange to D-SON management function to report the PCI collision or PCI confusion problems detected on NR cell(s).

3. The D-SON management function consumes the MnS of NF provisioning with modifyMOIAttributes operation to re-configure the PCI list for NR cell(s).

3.a The MnS of NF provisioning re-configures the PCI list for NR cell(s) (NOTE).

4. The PCI configuration (D-SON) function selects PCI value(s) from the updated PCI list.

5. The PCI configuration (D-SON) function reports the PCI value(s) being assigned to the MnS of NF provisioning.

6. The MnS of NF provisioning sends a notification notifyMOIAttributeValueChange to D-SON management function to indicate the PCI value(s) being assigned to NR cell(s).

7. The PCI configuration (D-SON) function notifies MnS of fault supervision that the PCI collision or PCI confusion problems have been resolved (NOTE).

8. The producer of provisioning MnS sends a notification notifyMOIAttributeValueChange to D-SON management function to report the PCI collision or PCI confusion problems being resolved.

NOTE: 	The interface between MnS of NF provisioning and PCI configuration (D-SON) function is not subject to standardization.

End of Modified Sections