3GPP TSG-SA5 Meeting #137-e	S5-213099

- Online, , 10th May 2021 - 19th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0172 | rev | - | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR 28.532 Clarifying A B and C |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics France SA |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE |  |  |  |  |  | Date: |  |  | 2021-04-29 |  |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |  |
| Reason for change: |  | As per the discusson in #136e.1, an action item was created as below. C: model-driven approach should be decribed, contributions are welcome to 28.532/28.533. (Action for Deepanshu/Olaf to better describe model-driven approach). |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR tries address the above Action and introduce the table describing Component Type A, B and C for Generic performance assurance management service |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The model driven approach will remain not well documented. |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1, 11.3, 11.3.1, 11.4 |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |
| Other specs |  |  | ■ | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | ■ | Test specifications |  |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | ■ | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 11	Management services – Stage 2

## 11.1	Generic provisioning management service

11.1.x			Components

The components of generic provisioning management sevice are listed in table 11.1-1.

Table 11.1-1: Components of performance assurance management services

| Management service | Management service component type A | Management service component type B | Management service component type C |
| --- | --- | --- | --- |
| Generic provisioning management service | createMOI operation | IOCs defined in in TS 28.622 and 28.541 | NA |
|  | getMOIAttributes operation |  |  |
|  | modifyMOIAttributes operation |  |  |
|  | deleteMOI operation |  |  |

End of 1st Modified Section

2nd Modified Section

## 11.3	Generic performance assurance management service

11.3.x			Components

The components of performance assurance management service are listed in table 11.3-1.

Table 11.3-1: Components of performance assurance management services

| Management service | Management service component type A | Management service component type B | Management service component type C |
| --- | --- | --- | --- |
| Performance assurance management service | createMOI operation | <<IOC>> PerfMetricJob as defined in TS 28.622 | Performance measurements for 5G NFs, as defined in TS 28.552. |
|  | getMOIAttributes operation |  |  |
|  | modifyMOIAttributes operation |  |  |
|  | deleteMOI operation |  |  |
|  | notifyThresholdCrossing notification |  |  |
| Performance threshold monitoring service | createMOI operation | <<IOC>>ThresholdMonitor, as defined in TS 28.622 | Performance measurements for 5G NFs, as defined in TS 28.552. |
|  | getMOIAttributes operation |  |  |
|  | modifyMOIAttributes operation |  |  |
|  | deleteMOI operation |  |  |
|  | notifyThresholdCrossing notification |  |  |

### 11.3.1	Operations and notifications

#### 11.3.1.1	Void

#### 11.3.1.2	Operation and notification of performance threshold monitoring service

#### 11.3.1.2.1	Notification notifyThresholdCrossing (M)

###### 11.3.1.2.1.1	Definition

This notification supports the threshold monitoring notification target to be notified when the performance threshold is crossed or reached.

###### 11.3.1.2.1.2	Notification information

| Parameter Name | S | Information Type | Comment |
| --- | --- | --- | --- |
| objectClass | M | ManagedEntity.objectClass | Class of the managed object, where the threshold crossing occurred. |
| objectInstance | M | ManagedEntity.objectInstance | Instance of the managed object, where the threshold crossing occurred. |
| notificationId | M | -- |  |
| notificationType | M, Y | "notifyThresholdCrossing " |  |
| eventTime | M, Y | -- | Time when the threshold crossing occurred. |
| systemDN | M | -- |  |
| observedPerfMetricName | M | ThresholdMonitor.thresholdInfoList[x].\ performanceMetrics[y] | Name of the performance metric that has crossed the threshold |
| observedPerfMetricValue | M | -- | Value of the performance metric, that has crossed the threshold, when the threshold crossing was observed |
| observedPerfMetricDirection | M | -- | Direction ("UP" or "DOWN") of the performance metric, when the threshold crossing was observed |
| thresholdValue | M | ThresholdMonitor.thresholdInfoList[x].\ thresholdvalue | Threshold value of the triggered threshold |
| hysteresis | M | ThresholdMonitor.thresholdInfoList[x].\ hysteresis | Hysteresis of the triggered threshold |
| monitorGranularityPeriod | M | ThresholdMonitor.monitorGranularityPeriod | Granularity period of the threshold monitor |
| additionalText | O | -- | Vendor specific information |

End of 2nd Modified Section

3rd Modified Section

## 11.4	Heartbeat control management service

11.4.x			Components

The components of heartbeat control management service are listed in table 7.4-1.

Table 7.4-1: Components of heartbeat control management service

| Management service | Management service component type A | Management service component type B | Management service component type C |
| --- | --- | --- | --- |
| Heartbeat control management services | createMOI operation | <<IOC>> HeartbeatControl as defined in TS 28.622 | N.A |
|  | getMOIAttributes operation |  |  |
|  | modifyMOIAttributes operation |  |  |
|  | deleteMOI operation |  |  |
|  | notifyHeartbeat notification |  |  |

End of 3rd Modified Section