3GPP TSG SA WG5 Meeting 137-e		S5-213166

- electronic meeting, online, 10 - 19 May 2021

- Source:	Huawei, Chna Telecom, China Mobile

- Title:	Add concrete IntentExpectation for network optimization

- Document for:	Approval

Agenda Item:	6.4.10

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP draft TS 28.312: “Management and orchestration; Intent driven management services for mobile networks v0.4.0”.

# 3	Rationale

This contribution proposes to add concrete IntentExpectation for network optimization.

# 4	Detailed proposal

It proposes to make the following changes to TS 28.312[1].

1st Change

#### 6.2.1.2.X	NetworkOptExpectation <<IOC>>

EditorNote: the NetworkOptExpectation is an <<IOC>> or <<dataType>> needs FFS.

###### 6.2.1.2.X.1 	Definition

This IOC represents the properties of an IntentExpectation for network optimization.  The NetworkOptExpectation represent the MnS consumer’s expectation on optimizing the network in the specified area.

One NetworkOptExpectation instance can include one or multiple NetworknTargetsInfo(s) which can be standardized (e.g. RANUEThroughputTargetsInfo) or non-standardized (represented by ExtentionTargetInfo), depending on the MnS consumer’s expectation for the network in the specified areas to be optimized. Each NetworkTargetsInfo specified a set of optimization targets for certain purpose (e.g. coverage optimization, RANUEthroughput optimization). One NetworkOptExpectation instance can include only one NetworkTargetsInfo for one optimization purpose, for example, if the MnS consumer’s expectation is only for coverage optimization, then only CoverageTargetsInfo is included in the NetworkOptExpectation instance; One NetworkOptExpectation instance also can include multiple NetworkTargetsInfos for multiple optimization purposes, for example, if MnS consumer’s expectation is for both coverage and RANUEThroughput optimization, then both CoverageTargetsInfo and RANUEThroughputTargetsInfo should be included in the NetworkOpExpectation instance.

The attribute “optimizationAreas” is used to represent a list of coverage areas to be optimized. Coverage area is used as intent driven object for the intent expectation expressed by a MnS consumer. The intent attribute “optimizationAreas” can be implemented by multiple choices, e.g. CoverageAreaPolygon, TrackingAreaCode.

6.2.1.2.X.2	Attribute

The NetworkOpExpectation includes attributes inherited from Top_ <<IOC>> and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| optimizationRATs | O | T | T | F | T |
| optimizationTime | O | T | T | F | T |
| optimizationPeriod | O | T | T | F | T |
| networkTargetsInfos | M | T | T | F | T |
| optimizationScope | O | T | T | F | T |
| optimizationAreas | M | T | T | F | T |

###### 6.2.1.2.X.3	Attribute constraints

None

2nd Change

#### 6.2.1.3.X	CoverageTargetsInfo <<dataType>>

###### 6.2.1.3.X.1	Definition

This <<dataType>> represents the properties of coverage target to be optimized for the coverage optimization scenario.

###### 6.2.1.3.X.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| targetWeakRSRPRatioInfos | M | T | T | F | T |
| targetLowSINRRatioInfos | M | T | T | F | T |

###### 6.2.1.3.X.3	Attribute constraints

None

##### 6.2.1.3.Y	RANUEThroughputTargetsInfo <<dataType>>

###### 6.2.1.3.Y.1	Definition

This <<dataType>> represents the properties of RAN UE throughput target to be optimized for the RAN UE throughput optimization scenario.

6.2.1.3.Y.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| targetAveULRANUEThpt | M | T | T | F | T |
| targetAveDLRANUEThpt | M | T | T | F | T |
| targeLowULRANUEThptRatioInfos | M | T | T | F | T |
| targeLowDLRANUEThptRatioInfos | M | T | T | F | T |

###### 6.2.1.3.Y.3	Attribute constraints

None

##### 6.2.1.3.Z	ExtentionTargetsInfo <<dataType>>

6.2.1.3.Z.1	Definition

This <<dataType>> represents the properties of extention targets to be optimized for other optimization scenario.

6.2.1.3.Z.2	Attribute

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| targetName | M | T | T | F | T |
| targetValue | M | T | T | F | T |

###### 6.2.1.3.Z.3	Attribute constraints

None

###### 6.2.1.3.M	CoverageArea <<choice>>

6.2.1.3.M.1	Definition

The <<choice>> defines a coverage area for the network to be optimized.  Coverage area is used as intent driven object for the intent expectation expressed by a MnS consumer. The attribute “CoverageAreas” can be implemented by multiple choices, e.g. CoverageAreaPolygon, TrackingAreaCode.

6.2.1.3.M.2	Attribute

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| CHOICE_1 coverageAreaPolygon | CM | T | T | F | T |
| CHOICE_2 tAC | CM | T | T | F | T |

###### 6.2.1.3.M.3	Attribute constraints

| Name | Definition |
| --- | --- |
| CHOICE_1 coverageAreaPolygon | This attribute shall be supported, when the coverage area in the intent expectation is in the form of polygon. |
| CHOICE_2 tAC | This attribute shall be supported, when the coverage area in the intent expectation is in the form of tracking area code. |

3rd Change

#### 6.2.1.4	Attribute definition

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| userLabel | A user-friendly (and user assignable) name of the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: F isUnique: F defaultValue: None isNullable: False |
| intentExpectation | It indicates the expectations including requirements, goals and constraints given to a 3GPP system. The intentExpectation includes the intentAreas which describes a list of coverage areas for the intent to be applied. Editor’s Note: the detailed intentExpectation is for further discussion | type: FFS multiplicity: * isOrdered: F isUnique: F defaultValue: None isNullable: False |
| intentFulfilStatus | It describes the status of the intent fulfilment result, which is configured by MnS producer and can be read by MnS consumer. allowedValues: "FULFILLED", “NOT_FULFILLED Editor’s Note: whether other allowed values should be supported is FFS, and the name for the attribute intentFulfilStatus is FFS. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

| optimizationTime | It describes the time to optimize the network. optimizationTime including: - optimizationStartTime - optimizationEndTime(Optional) AllowedValues: Not Applicable | type: string multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| --- | --- | --- |
| optimizationPeriod | It describes the period to optimize the network. AllowedValues: Not Applicable | type: string multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| networkTargetsInfos | It describes a list of NetworkTargetsInfo(s) for the network in the specified areas for different optimization purposes. NetworkTargetsInfo can be CoverageTargetsInfo, RANUEThroughputTargetsInfo or ExtentionTargetsInfo or combination of different networkTargetInfos. AllowedValues: Not Applicable | type: NetworkTargetInfo multiplicity:1. * isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| optimizationScope | It describes the optimization scope for the network in the specified areas to be optimized. The optimizationScope can be specific service type, specific 5QI, specific UE group AllowedValues: Not Applicable | type: String multiplicity:1. * isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| targetWeakRSRPRatioInfos | It describes the list of target Weak RSRP Ratio for the network in the specified areas to be optimized as optimization target. Each TargetWeakRSRPRatioInfo includes following two items: -WeakRSRPThreshold -WeakRSRPRatio AllowedValues: Not Applicable | type: TargetWeakRSRPRatioInfo multiplicity: 1. * isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| targetLowSINRRatioInfos | It describes the list of target low SINR Ratio for the network in the specified areas to be optimized as optimization target. Each TargetLowSINRRatioInfo includes following two items: - LowSINRThreshold - LowSINRRatio AllowedValues: Not Applicable | type: TargetLowSINRRatioInfo multiplicity: 1. * isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| targetAveULRANUEThpt | It describes the target average uplink RAN UE throughput for the network in the specified areas to be optimized as optimization target. AllowedValues: Not Applicable | type: Integer multiplicity:1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| targetAveDLRANUEThpt | It describes the target average downlink RAN UE throughput for the network in the specified areas to be optimized as optimization target. AllowedValues: Not Applicable | type: Integer multiplicity:1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| targeLowULRANUEThptRatioInfos | It describes the list of target low uplink RAN UE throughput ratio for the network to be optimized in the specified area as optimization target. Each TargeLowULRANUEThptRatioInfo specifics the low UL RAN UE throughput for a specified LowULRANUEThptThreshold, which includes following two items: - LowULRANUEThptThreshold - LowULRANUEThptRatio AllowedValues: Not Applicable | type: TargeLowULRANUEThptRatioInfo multiplicity: 1. * isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| targeLowDLRANUEThptRatioInfos | It describes the list of target low downlink RAN UE throughput ratio for the network to be optimized as optimization target. Each TargeLowDLRANUEThptRatioInfo specifics the low DL RAN UE throughput for a specified LowDLRANUEThptThreshold, which includes following two items: - LowDLRANUEThptThreshold - LowDLRANUEThptRatio AllowedValues: Not Applicable | type: TargeLowDLRANUEThptRatioInfo multiplicity: 1. * isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| optimizationRATs | It describes the radio access technology of the network in the specified areas to be optimization. AllowedValues: EUTRAN, NR, EUTRAN_NR | type: string multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| optimizationAreas | It describes a list of coverage areas for the network to be optimized. Coverage are is used as intent driven object for the intent expectation expressed by a MnS consumer | type: CoverageArea multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| coverageAreaPolygon | It describes the coverage area in the form of polygon. | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |

End of changes