- 3GPP TSG-SA5 Meeting #137e	S5-213183rev1

- 10 May to 19 May 2021, E-meeting

- Source:	Intel, NEC, China Telecom, HUAWEI?

- Title:	pCR 28.104 Add common information elements of MDA reports

- Document for:	Approval

- Agenda Item:	6.4.18

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]     S5-213039 add structure of TS 28.104 Management Data Analytics (MDA).

# 3	Rationale

There are some information elements that are common for all MDA reports, i.e., these common information elements form a subset of all MDA reports.

It would be more organized and clearer if the common information elements can be defined commonly in a central place.

This pCR is to add common information of MDA reports to a dedicated clause in draft TS 28.104 aligning with the structure in S5-213039 [1].

# 4	Detailed proposal

1st Modified Section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[h]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[i]	3GPP TS 28.658: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[j]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM); Information Service (IS)".

[k]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[l]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) model repertoire".

Next Modified Section

## 7.2	Common information elements of MDA reports

There are some information elements that are common for all MDA reports, i.e., these common information elements form a subset of all MDA reports.

The common information elements of the MDA reports are defined in table 7.2-1.

Table 7.2-1: Common information elements of MDA reports

| Information element | Definition | Data type | Support qualifier | Remarks |
| --- | --- | --- | --- | --- |
| MDAReportId | The identifier that uniquely identifies the MDA report between the MDAS producer and consumer. | String | M |  |
| ReportGenerationTime | The time when the MDA report was generated. | String (in date-time format). | M |  |
| MDAType | It indicates the type of MDA capability for analysis of the reported issue(s) (Note 1). The allowed values are the MDA type names defined for each MDA capability respectively. | String (in format “CategoryName.CapabilityName”) | M | Each MDA type corresponds to an MDA capability. It allows to describe multiple issues for the same or different MDA types in one MDA report, in this case this information element may be provided multiple times in one MDA report. |
| IssueId | The identifier of the issue described in the MDA report (Note 1). | String | M | It may be used to correlate the MDA reports when they are related to the same issue, e.g., one MDA report provides the update of the issue reported before in another MDA report. It allows to describe multiple issues in one MDA report, in this case this information element may be provided multiple times in one MDA report. |
| Cause | Cause of the issue described in the MDA report. | Integer | O | The mapping of the cause values to actual causes is vendor specific. It allows to describe multiple issues in one MDA report, in this case this information element may be provided multiple times in one MDA report. |
| SeverityLevel | Severity level of the issue described in the MDA report. | Enumeration of { - Critical, - Major, - Minor, - Cleared, - Indeterminate, - Normal } | O | It allows to describe multiple issues in one MDA report, in this case this information element may be provided multiple times in one MDA report. |
| IssueStartTime | The time when the issue described in the MDA report started or is predicted to start. | DateTime (see 32.156 [l]) | O | It allows to describe multiple issues in one MDA report, in this case this information element may be provided multiple times in one MDA report. |
| IssueUpdateTime | The time (UTC) when the issue described in the MDA report was lately updated. | DateTime (see 32.156 [l]) | O | It allows to describe multiple issues in one MDA report, in this case this information element may be provided multiple times in one MDA report. |
| IssueStopTime | The time when the issue described in the MDA report stopped. | DateTime (see 32.156 [l]) | O | It allows to describe multiple issues in one MDA report, in this case this information element may be provided multiple times in one MDA report. |
| AffectedMOIs | The MOIs that are affected or potentially affected by the issue described in the MDA report. | List of DN (see TS 32.300 [k]). | M |  |
| RecommendedActions | The recommended actions to solve the issue described in the MDA report. | Structure of { - (optional) List of CreationOfMOI { - IOCName: string (Note 2), - SuperordinatedMOI: DN (see TS 32.300 [k]), - List of { - AttributeName: string (Note 3), - AttributeValue: any (Note 4) } }, - (optional) List of ModificationOfMOI { - MOI: DN (see TS 32.300 [k]), - List of { - AttributeName: string (Note 3), - ModifiedAttributeValue: any (Note 4) } }, - (optional) List of DeletionOfMOI { - MOI: DN (see TS 32.300 [k]) }, - (optional) List of Non3GPPOperation { - OperationName: string (Note 5), - List of { - ParameterName: string (Note 5), - ParameterValue: any (Note 5) } }, (optional) List of HumanReadableAction: (string) } | O | The recommended actions could be creating, modifying, and/or deleting 3GPP MOI(s), and/or invoking one or more non-3GPP (such as ETSI ISG NFV) operations, and/or human readable actions. It allows to describe multiple issues in one MDA report, in this case this information element may be provided multiple times in one MDA report. |
| NOTE 1: The issue may be an ongoing problem, a potential problem or a prodiction triggered by some reason. NOTE 2: The allowed values of the IOCName are the IOC names defined in the NRM TSs (e.g., TS 28.541 [h], 28.658 [i],28.622 [j]), plus vendor specific extensions. NOTE 3: The allowed values of the AttributeName are the attributes names defined for the subject IOC (identified by IOCName) in the NRM TSs (e.g., TS 28.541 [h], 28.658 [i],28.622 [j]), plus vendor specific extensions.ss NOTE 4: The allowed data types and values of “Any” for AttributeValue and ModifiedAttributeValue are the data types and allowed values of the subject attribute (identified by AttributeName) defined in the NRM TSs (e.g., TS 28.541 [h], 28.658 [i],28.622 [j]), plus vendor specific extensions. NOTE 5: The allowed non-3GPP operation names, parameter names and parameter values are up to the definitions in the non-3GPP specifications (e.g., ETSI ISG NFV GSs). |  |  |  |  |

End of Modified Sections