3GPP TSG SA WG5 Meeting 137-e		TDoc S5-213312

- electronic meeting, online, 10 - 19 May 2021

- Source:	Huawei

- Title:	pCR 28.816 add potential solution on north bound API charging

- Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	3GPP TS 28.816

# 3	Rationale

This contribution is to add potential solution to enhance north bound API charging to support control plane data transfer domain charging.

In this solution, the charging enhancement for north bound API charging is to add NEF configuration API as the trigger to send charging event, and also add message flow related to NEF and AF during NIDD.

# 4	Detailed proposal

(For pseudo CR, include the complete clause(s) or subclause(s) of the latest draft TS/TR to be modified, with clear clause and sub-clause headings included and all modifications shown with revision marks, unambiguously showing where the changes shall be made or inserted in the draft TS/TR. It is not sufficient to just state, for example, “add the following text to the draft TS/TR…”.)

First modification

### 5.x.1.2.2	Charging enhancement to north bound API charging for NEF based NIDD

5.x.1.2.2.1 Charging architecture in control plane data transfer domain charging for 5GS CIoT

The NEF embedding the CTF generates charging events to the CHF for the north bound API charging. Figure 5.x.1.2.1.2.1 shows the architectural options for the converged charging.

Figure 5.x.1.2.1.2.1: the converged charging arch NEF embedding the CTFitecture for control plane data transfer domain charging

5.x.1.2.2.2 Flow for north bound API charging in case of NIDD configuration

The NIDD configuration procedure is the configuration triggered by AF or NEF initiated.The NEF embedding CTF in north bound API charging may store and collect charging information of NIDD configuration.

The Figure 5.x.1.2.2.2.1 shows the message flow of charging enhancement in case of NIDD configuration.

Figure5.x.1.2.2.2.1: The message flow of charging enhancement for NIDD configuration

Step 1-2, the NEF initiated or AF triggered NIDD configuration messages exchange as described in clause 4.25.3 TS 23.502 [4]. The AF ID, and NEF ID are included and sent to AF for asking the NEF configuration.

Step 2ch-a, NEF is triggered to send Charging data request [Event] to CHF. The NEF ID, AF ID, the MTC provider information can be included as part of charging information.

Step 2ch-b, CHF opens CHF CDR.

Step 2ch-c, CHF sends Chaging data response to the NEF.

Step 3-7, the rest messages of NIDD configuration procedure as described in clause 4.25.3 TS 23.502 [4].

5.x.1.2.2.3 Flow for north bound API charging in case of NEF based NIDD

Non-IP Data Delivery (NIDD) it is a means for delivering data via a PDU Sessions of type "Unstructured". Converged charging and PEC based charging are used in this solution.

The Figure 5.x.1.2.1.2.1 shows the charging enhancements of SMF and CHF and procedure when UE performs the PDU Session establishment with PDU Sessions of type "Unstructured" toward NEF.

The Figure 5.x.1.2.2.3.1: Message flow of north bound API charging for NIDD

Step 1. The step 1 is described in clause 4.25.2 of TS 23.502 [4].

Step 2. As described in TS 23.502 [4], if the subscription information corresponding to DNN and S-NSSAI includes the "NEF Identity for NIDD" (NEF ID), the SMF should create a PDU Session towards the NEF. The SMF sends Nnef_SMContext_Create Request with 5GS CIoT optimization related indications (NIDD information, S-NSSAI, [Small Data Rate Control parameters], [Small Data Rate Control Status]) message towards the NEF.

Step 2ch-a. The NEF sends charging data request [Event] to CHF, with cooresponding charging information.

Step 2ch-b. CHF opens CHF CDR for the PDU Session of NIDD.

Step 2ch-c. CHF sends Charging data response [Event] to the SMF.

Step 3. The NEF creates an NEF PDU session Context and send invokes Nnef_SMContext_Create Response to the SMF

Step 4. The step 4 is to finish the PDU Session establishment as described in clause 4.25.2 of TS 23.502 [4].

Second modification