- 3GPP TSG-SA5 Meeting #137e	S5-213389

- 10 May to 19 May 2021, E-meeting

- Source:	Huawei

- Title:	pCR Add alarm incident analysis

- Document for:	Approval

- Agenda Item:	6.4.18

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	3GPP TS 28.104-000 “Management and orchestration; Management Data Analytics”

# 3	Rationale

In 5G system, millions of alarms are generated due to the more complex network with high density of network functions and end users. Huge amount of alarms brings difficulties in network operation and maintenance. Therefore, the alarms and deteriorated performance measurements of same root cause should be correlated and analysed to relieve the stess of manually alarm handling.

This contribution is proposed to add alarm incident analysis capability of MDAS in [2].

# 4	Detailed proposal

It is proposed to apply following changes in [2].

1st modified section

## 7.5	MDA assisted fault management

### 7.5.x	Alarm Incident Analysis

#### 7.5.x.1	Description

In 5G network, millions of alarms are generated due to the network complexity. Since the topological relations between different network elements and logical relations between different generated alarms, a series of alarms caused by a same root cause should be correlated with each other. In addition, the same root causes may give rise to the network performance deterioration. An incident is identified by numbers of alarms, performance measurements and etc. Therefore, correlations of alarms and deteriorated performance measurements should be analysed by MDA so that the output can be leveraged to relax the stess of manually alarm handling.

#### 7.5.x.2	Analytics output

The specific information elements of the analytics output for Alarm incident analysis, in addition to the common information elements of the MDA reports (see clause 7.x), are provided in the below tables.

Table 7.5.x.2.1-1: Analytics output for Alarm incident analysis for CN

| Information element | Definition | Data type | Support qualifier | Remarks |
| --- | --- | --- | --- | --- |
| Alarm Incident report Identifier | The identifier of the Alarm Incident analytics report | String | M |  |
| List of Correlated AlarmInfo | List of alarms correlated to this analytics report | correlatedAlarmList | M |  |
| > alarmId | Alarm identifier | key(AlarmId) | M |  |
| Affected objects and/or object relations | The affected objects (e.g., the MOIs of cells or subnetworks or network slices affected by the alarm incident) and the (list of) relationship(s) between the objects, | FFS | M | e.g., if two affected cells are connected with each other through Xn interface, the relationship between these two objects may be noted as neighbouring |
| Root alarm and cause | Root alarm identified or predicted by root cause decision model. | FFS | M | Allow multiple occurences in MDA report |
| > alarmId | Alarm identifier | key(AlarmId) | M |  |
| > Number of occurrences | Number of occurrences of the root alarm. | FFS | O |  |
| > associated ManagedElement | The assioicated managedElement | FFS | O |  |
| >> vendorName | The name of the vendor | FFS | O |  |
| Severity level | The severity level (e.g., critical, medium, not important) of the alarm incident | FFS | O |  |

Table 7.5.x.2.2-1: Analytics output for Alarm incident analysis for AN

| Information element | Definition | Data type | Support qualifier | Remarks |
| --- | --- | --- | --- | --- |
| Alarm Incident report Identifier | The identifier of the Alarm Incident analytics report | String | M |  |
| List of Correlated AlarmInfo | List of alarms correlated to this analytics report | correlatedAlarmList | M |  |
| > alarmId | Alarm identifier | key(AlarmId) | M |  |
| Root alarm and cause | Root alarm identified or predicted by root cause decision model. | FFS | M | Allow multiple occurences in MDA report |
| > alarmId | Alarm identifier | key(AlarmId) | M |  |
| Severity level | The severity level (e.g., critical, medium, not important) of the alarm incident | FFS | O |  |

Table 7.5.x.2.3-1: Analytics output for Alarm incident analysis for cross domain

| Information element | Definition | Data type | Support qualifier | Remarks |
| --- | --- | --- | --- | --- |
| Alarm Incident report Identifier | The identifier of the Alarm Incident analytics report | String | M |  |
| List of Correlated AlarmInfo | List of alarms correlated to this analytics report | correlatedAlarmList | M |  |
| > alarmId | Alarm identifier | key(AlarmId) | M |  |
| Affected objects and/or object relations | The affected objects (e.g., the MOIs of cells or subnetworks or network slices affected by the alarm incident) and the (list of) relationship(s) between the objects, | FFS | M | e.g., if two affected cells are connected with each other through Xn interface, the relationship between these two objects may be noted as neighbouring |
| Root alarm and cause | Root alarm identified or predicted by root cause decision model. | FFS | M | Allow multiple occurences in MDA report |
| > alarmId | Alarm identifier | key(AlarmId) | M |  |
| > Number of occurrences | Number of occurrences of the root alarm. | FFS | O |  |
| > associated ManagedElement | The assioicated managedElement | FFS | O |  |
| >> vendorName | The name of the vendor | FFS | O |  |
| Severity level | The severity level (e.g., critical, medium, not important) of the alarm incident | FFS | O |  |

2nd modified section

## 8.2	MDA report requesting and reporting

### 8.2.1	Description

### 8.2.2	Requirements

### REQ-ALARM_MDA-01:	The MDAS producer should have a capability to provide the analytics report describing the alarm incident analysis.

REQ-ALARM_MDA-02:	The analytics report for CN domain describing the alarm incident should include the following information:

-	Alarm incident Identifier

-	List of Correlated Alarms

-	The start time and stop time of the Alarm incident

-	The root cause or root alarm of the Alarm incident

-	Severity level

REQ-ALARM_MDA-03:	The analytics report for RAN domain describing the alarm incident should include the following information:

-	Alarm incident Identifier

-	List of Correlated Alarms

-	Affected objects and/or object relations

-	The start time and stop time of the Alarm incident

-	The root cause or root alarm of the Alarm incident

-	Severity level

REQ-ALARM_MDA-04:	The analytics report for cross domain describing the alarm incident should include the following information:

-	Alarm incident Identifier

-	List of Correlated Alarms

-	Affected objects and/or object relations

-	The start time and stop time of the Alarm incident

-	The root cause or root alarm of the Alarm incident

-	Severity level