3GPP TSG-SA5 Meeting #137-e	S5-213421

- Online, , 10th May 2021 - 19th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0511 | rev | 1 | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Usage of slice input parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | EMA5SLA |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Standards defines many different parameters for SliceProfile. The usage of these parameters should be clarified by the Standard. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Normative Annex clarifying a use of SliceProfile parameters, is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity on usage of SliceProfile parameters remains in the Standard |  |  |  |  |  |  |  |  |
| Clauses affected: |  | L.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## L.X	Usage of SliceProfile and xxxSliceSubnetProfile parameters

The present document defines a list of parameters contained in SliceProfile datatype  object including TopSliceSubnetProfile, CNSliceSubnetProfile and RANSliceSubnetProfile

The usage of the parameters contained in these objects can be as follows:

Dimensioning:	used for provisioning a slice, configuring a service to be supported by the slice

Monitoring:		used to monitor the service fulfillment by the slice

Enforcement:	used to keep utilization of the system resources within agreed limits

The usage of each individual parameter is clarified in the Table 1 (SliceProfile parameters) 
and Table 2 (CNSliceSubnetProfile, RANSliceSubnetProfile, and TopSliceSubnetProfile parameters) below

| Name of parameter | Usage Type | Comment |
| --- | --- | --- |
| sliceProfileId | - | To identify slice profile instance |
| sNSSAIList | dimensioning |  |
| pLMNIdList | dimensioning |  |
| perfReq | monitoring, enforcements, dimensioning |  |
| CNSliceSubnetProfile | container of parameters | Refer to table 2 below |
| RANSliceSubnetProfile | container of parameters | Refer to table 2 below |
| TopSliceSubnetProfile | container of parameters | Refer to table 2 below |

Table 1: Usage of SliceProfile parameters

| Name of parameter | Usage Type | Comment |
| --- | --- | --- |
| coverageArea | dimensioning |  |
| Latency | monitoring, enforcements, dimensioning |  |
| maxNumberofUEs | monitoring, enforcements, dimensioning |  |
| dLThptPerSliceSubnet | monitoring, enforcements |  |
| dLThptPerUEPerSubnet | monitoring, enforcements |  |
| uLThptPerSliceSubnet | monitoring, enforcements |  |
| uLThptPerUEPerSubnet | monitoring, enforcements |  |
| maxPktSize | monitoring, enforcements |  |
| maxNumberOfPDUSessions | monitoring, enforcements |  |
| sliceSimultaneousUse | monitoring, enforcements |  |
| delayTolerance | monitoring |  |
| termDensity | monitoring | Unit of measure: number/km2 |
| activityFactor | monitoring |  |
| coverageAreaTAList | Configuration; Provisioning |  |
| resourceSharingLevel | monitoring, enforcements, dimensioning |  |
| uEMobilityLevel | monitoring |  |
| uESpeed | monitoring |  |
| Reliability | monitoring, dimensioning |  |
| serviceType | dimensioning |  |

Table 2: Usage of parameters contained within CNSliceSubnetProfile, RANSliceSubnetProfile and TopSliceSubnetProfile

End of changes