3GPP TSG-SA5 Meeting #139-e	S5-215061

- October 11 – 20, 2021, e-Meeting								       		 		   Revision of s5-abcde

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | 0033 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add RRM related measurements information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSON_5G |  |  |  |  |  | Date: |  |  | 2021-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Table 7.2.2.3.1-1 is missing some RRM related measurments and the references to measurements in TS 28.552. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the missing RRM related measurments information and references in Table 7.2.2.3.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RRM related measurements are not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 7.2.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

#### 7.2.2.3	MnS Component Type C definition

##### 7.2.2.3.1	Performance measurements

RRM related performance measurements are captured in Table 7.2.2.3.1-1:

Table 7.2.2.3.1-1. RRM related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| Mean DL PRB used for data traffic | Provides the mean number of PRBs used in downlink for data traffic in the NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.2.5 in TS 28.552 [5]). |  |
| Peak DL PRB used for data traffic | Provides the peak number of PRBs used in downlink for data traffic in the NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.2.9 in TS 28.552 [5]). |  |
| Mean UL PRB used for data traffic | Provides the mean number of PRBs used in uplink for data traffic in the NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.2.7 in TS 28.552 [5]). |  |
| Peak UL PRB used for data traffic | Provides the peak number of PRBs used in uplink for data traffic in the NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.2.10 in TS 28.552 [5]). |  |
| Average DL UE throughput in gNB | Provides the average UE throughput in downlink in the NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.3.1 in TS 28.552 [5]). |  |
| Distribution of DL UE throughput in gNB | Provides the distribution of the UE throughput in downlink in the NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.3.2 in TS 28.552 [5]). |  |
| Average UL UE throughput in gNB | Provides the average UE throughput in uplink in the NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.3.3 in TS 28.552 [5]). |  |
| Distribution of UL UE throughput in gNB | Provides the average UE throughput in uplink in the NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.3.4 in TS 28.552 [5]). |  |
| Mean number of Active UEs in the DL per cell | Provides the mean number of active UEs in downlink in an NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.23.1 in TS 28.552 [5]). |  |
| Maximum number of Active UEs in the DL per cell | Provides the maximum number of active UEs in downlink in an NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.23.2 in TS 28.552 [5]). |  |
| Mean number of Active UEs in the UL per cell | Provides the mean number of active UEs in uplink in an NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.23.3 in TS 28.552 [5]). |  |
| Maximum number of Active UEs in the UL per cell | Provides the maximum number of active UEs in uplink in an NRCellDU, with subcounters per sNSSAI (see clause 5.1.1.23.4 in TS 28.552 [5]). |  |
| Mean number of DRBs being allocated | Provides the mean number of DRBs being allocated in the PDU sessions in the NRCellCU, with subcounters per sNSSAI (see clause 5.1.1.10.9 in TS 28.552 [5]). |  |
| Peak number of DRBs being allocated | Provides the peak number of DRBs being allocated in the PDU sessions in the NRCellCU, with subcounters per sNSSAI (see clause 5.1.1.10.10 in TS 28.552 [5]). |  |
| Mean number of PDU Sessions being allocated | Provides the mean number of PDU Sessions being allocated in the NRCellCU, with subcounters per sNSSAI (see clause 5.1.1.5.4 in TS 28.552 [5]). |  |
| Peak number of PDU Sessions being allocated | Provides the peak number of PDU Sessions being allocated in the NRCellCU, with subcounters per sNSSAI (see clause 5.1.1.5.5 in TS 28.552 [5]). |  |
| Mean number of RRC connections | Provides the mean number of RRC connections in the NRCellCU, with subcounters per sNSSAI (see clause 5.1.1.4.1 in TS 28.552 [5]). |  |
| Maximum number of RRC connections | Provides the maximum number of RRC connections in the NRCellCU, with subcounters per sNSSAI (see clause 5.1.1.4.2 in TS 28.552 [5]). |  |

End of Modified Sections