- 3GPP TSG SA WG5 Meeting #139e	S5-215091

- Online, , 11 Oct 2021- 20 Oct 2021

- Source:	Samsung

- Title:	pCR 28.819 solution for Dynamic software update and live testing

- Document for:	Approval

Agenda Item:	6.5.7

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

None

# 3	Rationale

This contribution provides the solution for dynamic software update and live testing use case. This solution proposes to use the slice framework for testing the upgrades in the live network. The use of slicing is justified as follows:

- For testing in the live network we need to isolate the component under test and restrict it to certain UEs, so that any test failure does not affect the other users. Slicing framework is already available which will cater to this requirement.

- If the test involves multiple NFs, the chaining/graphs of the NFs can be done using slicing.

- The provisioning/configuration of all the target NFs can be done by a single provisioning procedures of a slice.

# 4	Detailed proposal

1st modified section

6.2	 Operational testing

6.2.1 Description

The Network Slicing feature in 5G provides telecom operators the capability to support multiple logical networks on the common physical network infrastructure. Each of these Logical networks will cater to different services and might have different operational requirements spanning across domains. Therefore, in addition to the 5G feature, it is important for operators to have the flexibility to manage these logical networks independently. Cloud Native based platform and tools which are widely used in the IT world has inbuilt frameworks for agile and flexible deployment. These platforms are increasingly becoming the de-facto choice for 5G Telecom Products. This highlights the importance of bringing many Cloud Native benefits like CI-CD that are essential for automation and flexibility of 5G networks. CI-CD automates and integrates the development phase, testing phase and deployment phase to improve feature velocity. Enabling CI-CD in 5G networks which includes the most transaction-intensive and time-sensitive RAN network functions is still a challenge. Dynamic software upgrades and live testing of 5G software components in production environment without service interruptions remains the main challenge that need to be solved for enabling CI-CD in 5G networks.

The ability to test the upgraded network nodes in the operational network will increase the agility of 5G networks. Slicing and virtualizationcan be used to assist in live testing. A test network slice instance or a test NF can be created/deployed, using the upgraded network functions, for testing purpose. The basic principles of the operational testing are:

- Test network slice instance or test NF creation with appropriate information e.g. test cases, test duration, target network nodes etc.

- Automatic UE selection: The set of UEs to be assigned to the test network slice instance or the test NF instance can be selected  and updated over time to minimize the adverse effects of testing. The UEs can be selected based on any network or UE characteristics, such as current load of the network, location of the network nodes,UE mobility or dual connectivity etc.

- Deployment location selection for the upgraded node: The location of the upgraded node can be selected to minimize the adverse impact of testing. The location of the Networks nodes that needs to be upgraded and tested can be selected based on load conditions of the Network nodes at a given location. The location of the node can also be based on the availability of UEs for testing in a given location.

6.2.2 Requirements

REQ-CICD-FUN-1 The 3GPP Management system shall be able to support testing the upgraded software in the live network.

REQ-CICD-FUN-1 The 3GPP Management system shall be able to support creating/deploying a slice for testing the upgraded network functions in the live network.

REQ-CICD-FUN-1 The 3GPP Management system shall be able to support selecting appropriate UE(s) to be assigned to the test slice minimizing the service impacts, if any.

REQ-CICD-FUN-1 The 3GPP Management system shall be able to support selecting appropriate location for the upgrade network functions deployment minimizing the service impacts, if any.

6.2.3 	Solution Alternate 1 – Test NSI(s)

6.2.3.1 	Overview

The solution involves creating/instantiating a new network slice exclusively, or re-using an existing network slice, for testing purpose triggred by a software upgrade procedure. The slice will include information related with the testing including Managed Function to be tested, test duration, test case etc.

The appropriate UE(s) are assigned to this test slice minimizing the service impacts. UEs can be selected based on the location of the network nodes where the upgraded software is to be deployed. UEs can also be selected based on the UE charecteristics like UE mobility and UE capabilities like dual connectivity or can just be percentage of avalible UEs

The location of the networks nodes that needs to be upgraded and tested can be selected based on load conditions of the Network nodes at a given time. The location of the node can also be based on (not limited too) the availability of UEs for testing in a given location.

6.2.3.2 	Details

The solution requires the ability to indicate that a particular network slice insance is meant for testing purpose. To achieve this the following attrbiutes should be added in the NetworkSlice IOC.

| Attributes | Description |
| --- | --- |
| operationalState | Existing parameter |
| administrativeState | Existing parameter |
| serviceProfileList | Existing parameter |
| networkSliceRef | Existing parameter |
| networkSliceSubnetRef | Existing parameter |
| testSliceInfo | This attribute provided the information related with the test slice. |
| >> Managed Function Information (List) | This parameter is a list of ManagedElement that needs to be upgraded to test the new feature |
|  | This parameter is a list of software versions for the ManagedElement to be upgraded to test the new feature |
| >>Test duration | The slice can be deactivated after completion of this duration. - Total testing time (e.g. 200 hours). - Minimum Continuous testing period (e.g. 24 hours). |
| >> Pre-emption indicator | This parameter is a Boolean value indicating if the slice can be pre-empted and deactivated during high load scenarios and when there is resource constraint. |
| >> Procedures to be tested | This could be list of 3gpp procedures (e.g. rrc connection setup, handover etc.). This can be used to decide the geographical location where the test slices need to be deployed and when it needs to be deployed. |

Once the test network slice is instantiated, the UE(s) can be assigned to this slice. The UEs selected for testing can be:

- Based on UE capability

- 	Carrier aggregation, Dual connectivity capability.

- Based on UE characteristics

- 	Based on User Mobility (which can be determined based on UEmobilityLevel).

- 	Based on UE RRC_State (which can be determined based on the UE history information stored with the operator)- Percentage of available UE

The testing location of the new node can be critical in terms of minimizing the service experience and maintain better use experience. Selection of deployment location of the test software in the field can be based on

- the availability of radio and cloud resources;

- the availability of UE selected; and

- any other operator defined restrictions.

End of modified section