3GPP TSG-SA5 Meeting #139-e 	S5-215095

- e-meeting, October 11 – 19 2021                                                          (revision of S5-214213)

- Source:	Huawei, China Telecom, China Mobile, China Unicom

- Title:	Add concrete RadioNetworkExpectation

- Document for:	Approval

Agenda Item:	6.4.10

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP draft TS 28.312: “Management and orchestration; Intent driven management services for mobile networks v0.5.0”.

# 3	Rationale

This contribution proposes to add concrete IntentExpectation for radio network which can be used for both network deployment scenario and radio network optimization scenario.

# 4	Detailed proposal

It proposes to make the following changes to TS 28.312[1].

1st Change

##### 6.2.1.2.2	IntentExpectation

###### 6.2.1.2.2.1	Definition

Following Content for MnS’s expectation on a radio network is used as example to discuss the concrete model for IntentExpectation:

Editor’s Note: following content needs to be revisited based on further discussion for intent model, how to model IntentExpectation is FFS.

Following described the information/attributes for MnS Consumer’ s expectation to deliver/ensure on a radio network,

-	ManagedObject, it describes the managed object (i.e. RadioNetwork) expressed by MnS’s expectation on a radio network.

-	ExpectedAreas, it describes a list of coverage areas for radio network. This can be implemented by multiple choices, e.g. CoverageAreaPolygon, TrackingAreaCode.

-	RANCoverageTarget, it describes the properties of coverage quality for the radio network, which includes:

-	targetWeakRSRPRatioInfo, it describes the list of target Weak RSRP Ratio for the radio network. Each TargetWeakRSRPRatioInfo includes WeakRSRPRatio and WeakRSRPThreshold.

-	targetLowSINRRatioInfos, it describes the list of target low SINR Ratio for the radio network. Each TargetLowSINRRatioInfo includes LowSINRThreshold and LowSINRRatio.

-	RANCapacityTarget, it describes the properties of network capacity for the radio network, which includes：

-	targetMaximumUENumber, it describes the maximum number of should be supported by the radio network

-	targetActivityFactor, it describes the maximum number of should be supported by the radio network

-	RANUEThptTarget, it describes the properties of the RAN UE throughput for the radio network, which includes:

-	targetAveULRANUEThpt, it describes the average UL RAN UE throughput for the radio network

-	targetAveDLRANUEthpt, it describes the average UL RAN UE throughput for the radio network

-	targetLowULRANUEThptRatioInfos, it describes the list of target low uplink RAN UE throughput ratio for the network, which includes LowULRANUEThptRatio and LowULRANUEThptThreshold

-	targetLowDLRANUEThptRatioInfos, it describes the list of target low downlink RAN UE throughput ratio for the radio network, which includes LowDLRANUEThptThreshold and LowDLRANUEThptRatio

-	RANConstraint, it describes the properties of a set of constraints for the radio network, which includes:

-	radioConstraints, it describes the properties of a set of radio constraints for the radio network, which includes cellIdRanges, gNBIdRanges, tACRanges, pLMNIdList and nRPCIRanges.

-	frequencyConstraints, it describes the properties of a set of frequency constraints for radio network, which includes arfcnUL/DL and bSChannelBwUL/DL

-	transportConstraints, it describes the properties of a set of transport constraints to be used in the radio network, including ngcLocalIpAddressList, nguRemoteIpAddressList, logicInterfaceId and nextHopInfo.

End of changes