3GPP TSG-SA5 Meeting #139-e 	S5-215124

- e-meeting, 11 - 20 October 2021

- Source:	Huawei,China Mobile,China Unicom,AsiaInfo

- Title:	Update generic MnS requirements and solutions for network optimization

- Document for:	Approval

Agenda Item:	6.4.9

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP 	draft TS 28.100: "Management and orchestration; Levels of autonomous network 0.6.0".

# 3	Rationale

This contribution proposes to add more solution for generic MnS requirements for network optimization.

# 4	Detailed proposal

It proposes to make the following changes to TS 28.100[1].

1st Change

### 7.1.4 Generic MnS requirements and solutions

##### 7.1.4.1 MnS requirements to support autonomous network level 1

REQ-ANL-NetOpt-Level_1-MnS-1 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network adjustment solution.

This is implemented by provisioning MnS defined in TS 28.532 [4] with 5G Network Resource model defined in TS 28.541[5].

REQ-ANL-NetOpt-Level_1-MnS-2 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network related information collection rules.

This is implemented by provisioning MnS defind in TS 28.532 [4] with PM control NRM fragment and Trace control NRM fragment defin in TS 28.622 [6].

REQ-ANL-NetOpt-Level_1-MnS-3 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the network related information (including network performance data, network configuration data and environment data).

This is implemented by file data reporting MnS and streaming data report MnS defined in TS 28.532 [4] with measurements defined in TS 28.552 [7], KPI defined in TS 28.554[8], MDT data defined in TS 32.422 [9].

##### 7.1.4.2 Additional MnS requirements to support autonomous network level 2

REQ-ANL-NetOpt-Level_2-MnS-1 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network issue identification rule.

For the network issues which can be detected based on threshold, this can be implemented by generic provisioning MnS defined in TS 28.532 [4] with Threshold monitoring control NRM fragment defined in TS 28.622 [6].

REQ-ANL-NetOpt-Level_2-MnS-2 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the network issue.

For the network issues which can be detected based on threshold, this can be  implemented by notifyThresholdCrossing of performance assurance MnS defined in TS 28.532 [4].

REQ-ANL-NetOpt-Level_2-MnS-3 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network issue demarcation rule.

For the network issues which can be detected based on threshold,this can be implemented by generic provisioning MnS (e.g, createMOI) defined in TS 28.532 [4] with Threshold monitoring control NRM fragment defined in TS 28.622 [6].

REQ-ANL-NetOpt-Level_2-MnS-4 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the network issue demarcation result.

For the network issues which can be detected based on threshold, this can be implemented by notifyThresholdCrossing of performance assurance MnS defined in TS 28.532 [4].

REQ-ANL-NetOpt-Level_2-MnS-5 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network issue root cause analysis rule.

REQ-ANL-NetOpt-Level_2-MnS-6 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the network issue root cause analysis result.

For the network issues which can be detected based on threshold, this can be implemented by performance assurance MnS defined in TS 28.532 [4].

##### 7.1.4.3 Additional MnS requirements to support autonomous network level 3

REQ-ANL-NetOpt-Level_3-MnS-1 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network adjustment policies.

This can be  implemented by using generic policy MnS defined in TS 28.556 [X] to specify the network adjustment policy.

REQ-ANL-NetOpt-Level_3-MnS-2 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the recommended network adjustment solution.

REQ-ANL-NetOpt-Level_3-MnS-3 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network adjustment decision policies.

This can be implemented by using generic policy MnS defined in TS 28.556 [X] to specify the network adjustment decision policies.

REQ-ANL-NetOpt-Level_3-MnS-4 The 3GPP management system shall have the capability to obtain the network performance deterioration prediction information.

##### 7.1.4.4 Additional MnS requirements to support autonomous network level 4

REQ-ANL-NetOpt-Level_4-MnS-1 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network optimization intent.

REQ-ANL-NetOpt-Level_4-MnS-2 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network optimization intent translation policies.

REQ-ANL-NetOpt-Level_4-MnS-3 The 3GPP management system shall have the capability allowing its authorized consumer to specify the network optimization evaluation policies.

REQ-ANL-NetOpt-Level_4-MnS-4 The 3GPP management system shall have the capability allowing its authorized consumer to obtain the network optimization fulfilment information.

End of changes