- 3GPP TSG-SA5 Meeting #139-e 	S5-215151

- e-meeting, 11 - 20 October 2021

- Source:	China Mobile, HUAWEI?

- Title:	Add alarm related incident analysis use case

- Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	3GPP TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	3GPP TS 28.104-000 “Management and orchestration; Management Data Analytics”

# 3	Rationale

In 5G system, millions of alarms are generated due to the more complex network with high density of network functions and end users. Huge amount of alarms brings difficulties in network operation and maintenance. Therefore, the alarms and deteriorated performance measurements of same root cause should be correlated and analysed to relieve the stress of manually alarm handling.

This contribution is proposed to add alarm incident analysis capability of MDAS in [2].

# 4	Detailed proposal

It is proposed to apply following changes in [2].

1st Modified Section

## X.Y MDA Capabilities

## X.Y.Z	Alarm related Incident analysis

#### X.Y.Z.1	Description

The MDA capability is about the alarm related incident analysis.

#### X.Y.Z.2	Use cases

Due to the complexity of 5G network system, a large number of alarms are posted to the 3GPP management system every day. These alarms come from network elements, infrastructures, systems of different levels, such as network slices, network slice subnets, NFs (network functions) (that could be running as Virtual NFs (VNFs) or as Physical NFs (PNFs)). These alarms may also come from a specific management domain (e.g., CN or AN).

There is a possibility that a root cause could trigger several alarms from different network sources. The reasons could be 1) there are several topological relations between different network elements; 2) there are several logical relations between different generated alarms; 3) an incident could be identified by numbers of alarms, performance measurements and etc. Therefore, it is desirable for 3GPP management system to do collection, correlation, filter, analysis and recognization to determine fault type, the time, severity, affected objects of the fault and other fault related information. Some ML models and algorithms may be used to group or filter the correlated alarms and indicate the root cause.

To improve the efficiency of network operation and maintenance, MDA can provide the area specific analysis and provide the output based on multiple kinds of data (performance measurement data, configuration data, network topology information, etc.) for alarm demarcation and root cause analysis.

If necessary, MDA could provide recovery mechanism options.

#### X.Y.Z.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ALARM_INC_MDA-1 | 3GPP management system shall have the capability to provide the alarm related incident analysis of demarcation and root cause based on collected data. | Alarm Incident Analysis |
| REQ-ALARM_INC_MDA-2 | 3GPP management system shall have the capability to correlate the related alarms with root causes. | Alarm Incident Analysis |
| REQ-ALARM_INC_MDA-3 | 3GPP management system shall have the capability to provide the area specific alarm related incident analysis. | Alarm Incident Analysis |

End of Modified Sections