3GPP TSG SA WG5 Meeting 139-e		S5-215158

- Electronic meeting, online, 11 - 20 October 2021

- Source:	Huawei

- Title:	Add E2E latency analysis

- Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

This contribution is for approval.

# 2	References

[1]	TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	3GPP TS 28.104-000 “Management and orchestration; Management Data Analytics”

# 3	Rationale

E2E latency is an important parameter for 5G services such as URLLC. User data packets should be successfullydelivered within certain time constraints to satisfy the end users requirements. Latency could be impacted by the network capability and network configurations, e.g. configuration of service priority, RAN capacity, network load, number of re-transmissions, Wireless channel environment and the processing time of the network functions, etc. And the affected objects, the root cause of the too high latency and solutions to solve the latency issue need to be discussed.

This contribution is proposed to provide the E2E latency analysis capability of MDA in [2].

# 4	Detailed proposal

It is proposed to apply following changes in [2].

First of changes

-

7	MDA scenarios

- 7.X	E2E latency analysis(UC- E2ELAT_MDA)

- 7.X.1	Description

This management capability allows the MDAS producer to analyse E2E latency related issue and provides the corresponding analytics report to MDAS consumer.

- 7.X.2	Use cases

E2E latency is an important parameter for URLLC services. User data packets should be successfully delivered within certain time constraints to satisfy the end users requirements. Latency could be impacted by the network capability and network configurations e.g. configuration of service priority, RAN capacity, network load, wireless channel environment and the processing time of the network functions, etc.. These factors may be the root cause if the latency requirements cannot be achieved. Packet transmission latency may dynamically change if these factors change. The latency requirement should be assured even if some of the network conditions may degrade. It is important for the MDAS producer to be able to classify and identify root cause of the latency issues to support SLS assurance.

- 7.X.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-E2ELAT_MDA-CON-1 | 3GPP management system should have the capability to identify the type of the E2E latency issue, e.g., RAN latency issue, CN latency issue, TN latency issue, UE latency issue，service provider latency issue | (UC- E2ELAT_MDA) E2E latency analytics |
| REQ-E2ELAT_MDA-CON-2 | 3GPP management system should have the capability to provide the severity level (e.g., critical, medium, not important) of the E2E latency issue | (UC- E2ELAT_MDA) E2E latency analytics |
| REQ-E2ELAT_MDA-CON-3 | 3GPP management system should have the capability to provide the root cause analysis of the E2E latency issue | (UC- E2ELAT_MDA) E2E latency analytics |

- MnS component type C

- report

End of changes