- 3GPP TSG SA WG5 Meeting #139-e	S5-215191

- October 11 – 20, 2021, e-Meeting	revision of S5-2abcde

- Source:	Intel

- Title:	pCR 28.538 add procedures for EAS LCM

- Document for:	Approval

Agenda Item:	6.4.21

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TS 28.538 “Management and orchestration; Edge Computing Management;”

# 3	Rationale

This contribution proposes to add the use case and requirements for EAS LCM to TS 28.538 [1].

# 4	Detailed proposal

1st modified section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 23.558 “Architecture for enabling Edge Applications”.

[y]	ETSI GS NFV-IFA 013 V3.4.1 “Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Os-Ma-nfvo reference point -Interface and Information Model Specification”.

[z]	ETSI GS NFV-IFA 011 V3.3.1 “Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; VNF Descriptor and Packaging Specification”.

[w]	3GPP TS 28.532 “Management and orchestration; Generic management services”.

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

Next modified section

# 7	Procedural Flows

# 7.x	Lifecycle management

### 7.x.1	Description

The clause contains procedures associated with lifecycle management.

### 7.x.2	EAS lifecycle management

#### 7.x.2.1	EAS VNF instantiation

Figure 7.x.2.1-1 depicts a procedure that describes how an ASP can consume provisioning MnS to instantiate the EAS. It is assumed that both ASP and ECSP consumers have subscribed to the producer of provisioning MnS to receive notifications.

Figure 7.x.2.1-1: EAS instantiation

1. ASP consumes the provisioning MnS with createMOI operation (see clause 11.1.1.1. in TS 28.532 [w]) for EASRequirements IOC to request ECSP provisioning MnS producer to start the EAS VNF instantiation, where the EASRequirements IOC contains the deployment requirements, including (but not limited to) the following attributes:

- the service areas (i.e., geographical, or topological) where the UEs can access the edge computing service (see clause 7.3.3 in TS 28.558 [x]).

- Software image information (e.g., software image location, minimum RAM, disk requirements) (see clause 7.1.6.5 in in ETSI NFV IFA-011 [z]).

-  QoS requirements (e.g., bandwidth, end-to-end latency).

2. ECSP provisioning MnS producer returns an output parameter to the ASP indication the instantiation operation is in progress.

3. ECSP provisioning MnS producer analyses the deployment requirements to determine which EDN and how many EAS instance(s) should be instantiated to satisfy the deployment requirements, and downloads the EAS VNF software image from the software image location.

. ECSP provisioning MnS producer invokes the InstantiateNsRequest operation (see clause 7.3.3 in ETSI GS NFV-IFA 013 [y]) to request NFVO via the Os-Ma-nfvo interface to instantiate a NS instance including the EAS VNF instance.

5. NFVO sends a notification to ECSP provisioning MnS producer indicating the result of instantiation procedure (see clause 7.3.3.4 of ETSI GS NFV-IFA 013 [y]).

6. If the VNF instantiation has been successful, then

6.1. ECSP provisioning MnS producer creates the MOI for EASFunction IOC.

6.2. ECSP provisioning MnS producer sends a notification notifyMOICreation to ASP to indicate that the EASFunction MOI has been created.

Otherwise (VNF instantiation has failed)

6.3. ECSP provisioning MnS producer sends a notification notifyMOICreation to ASP to indicate that the EASFunction MOI has not been created due to failure.

7. If all VNF instance(s) have been successfully instantiated, then

7.1. ECSP provisioning MnS producer creates the MOI for EASRequirements IOC.

7.2. ECSP provisioning MnS producer sends a notification notifyMOICreation to ASP to indicate that the EASRequirements MOI has been created.

If not all VNF instance(s) have not been instantiated, then

7.3. ECSP provisioning MnS producer creates the MOI for EASRequirements IOC.

7.4. ECSP provisioning MnS producer sends a notification notifyMOICreation to ASP to indicate that the EASRequirements MOI has been partially created.

Otherwise (no VNF instance has been successfully instantiated)

7.5 ECSP provisioning MnS producer sends a notification notifyMOICreation to ASP to indicate that the EASRequirements MOI has not been created due to failure.

#### 7.x.2.2	EAS VNF termination

Figure 7.x.1.2-1 depicts a procedure that describes how an ASP can consume provisioning MnS to terminate the EAS VNF. It is assumed that both ASP and ECSP consumers have subscribed to the producer of provisioning MnS to receive notifications.

Figure 7.x.2.2-1: EAS VNF termination

1. ASP consumes the provisioning MnS with deleteMOI operation for EASFunction MOI to request ECSP provisioning MnS producer to start the EAS VNF termination.

2. ECSP provisioning MnS producer returns an output parameter to the ASP indication the termination operation is in progress.

3. ECSP provisioning MnS producer invokes the TerminateNsRequest operation (see clause 7.3.7 in ETSI GS NFV-IFA 013 [y]) to request NFVO via the Os-Ma-nfvo interface to terminate EAS VNF instance.

4. NFVO sends the NS Lifecycle Change notification to ECSP provisioning MnS producer indicating the result of termination procedure (see clause 7.3.7.4 of ETSI GS NFV-IFA 013 [y]).

5. If the VNF termination has been successful then:

5.1. ECSP provisioning MnS producer deletes the MOI for EASRequirements IOC.

5.2. ECSP provisioning MnS producer sends a notification notifyMOIDeletion to ASP to indicate the EASRequirements MOI has been deleted.

Otherwise (VNF termination has failed):

5.3. ECSP provisioning MnS producer sends a notification notifyMOIDeletion to ASP to indicate the EASRequirements MOI has not been deleted due to failure.

End of modified section