3GPP TSG-SA5 Meeting #139-e 	S5-215205rev3

- e-meeting, 11 - 20 October 2021

- Source:	Huawei

- Title:	pCR 28.104 Add KPI anomaly analysis use case

- Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	SP-210132 Enhancements of Management Data Analytics Service

[2]	3GPP TS 28.104-000 Management and orchestration; Management Data Analytics (MDA)

[3]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

# 3	Rationale

KPI(s) are of great importance for network operators to monitor the key performance of the network. For 5G and beyond, a large amount of KPIs are defined. The correlations between different KPIs are complicated and it is hard to monitor the KPI(s) manually. Besides, how to assign each KPI threshold is a big challenge for network operators.

According to the approved WI [1], the objectives include defining the use case of SLS analysis, of which KPI anomaly analysis is a typical scenario. This contribution is proposed to define KPI anomaly analysis use case and requirements for MDAS in [2].

# 4	Detailed proposal

It is proposed to add the following chapter in TS 28.104[2].

1st modified section

# 2	References

[x]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

2nd modified section

# 7	MDA scenarios

## 7.2	MDA capabilities

### 7.2.X	KPI anomaly analysis (UC-KPI_ANOMALY)

#### 7.2.X.1	Discription

3GPP management system shall have the capability to provide KPI anomaly related issue analysis.

#### 7.2.X.2	Use cases

KPI(s) are of great importance for network operators to monitor the key performance of the network. For 5G and beyond, a large amount of KPIs are defined in TS 28.554 [y]. The correlations between different KPIs are complicated and it is hard to monitor the KPI(s) manually. Also, how to assign each KPI threshold is a big challenge for network operators, since each KPI threshold should not be a fixed value considering many factors such as network capacity, service types, end user's experiences, etc. In addition, the criteria to determine whether a KPI is anomalous also depends on a variety of requirements.

The MDAS can be utilized to correlate and analyse the KPI related management data. The MDAS producer provides a KPI anomaly analysis report which contains the information of detection and/or prediction of KPI(s), including the anomalous KPI(s) and the corresponding root cause(s). Anomalous KPI may be caused by multiple causes. The detection and prediction of single KPI anomaly and in relation with multiple correlated KPIs anomaly may be involved.

#### 7.2.X.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-KPI_ANOMALY-CON-1 | 3GPP management system shall have the capability to provide the KPI anomaly intelligent analysis to improve the efficiency and accuracy of KPI monitoring. | (UC-KPI_ANOMALY) KPI anomaly analysis |
| REQ- KPI_ANOMALY -CON-2 | 3GPP management system shall have the capability to identify the anomalous KPI and the related managed object instances. | (UC-KPI_ANOMALY) KPI anomaly analysis |
| REQ- KPI_ANOMALY -CON-3 | 3GPP management system shall have the capability to analyze the root cause of the KPI anomaly to assist the rectification of anomalous KPI related faults. | (UC-KPI_ANOMALY) KPI anomaly analysis |

3rd modified section

## 8.4	Data definitions per MDA capability

### 8.4.X	SLS analytics

#### 8.4.X.1	KPI Anomaly Analysis

##### 8.4.X.1.1	MDA type

The MDA type for KPI anomaly analysis is: SLSAnalysis.KPIAnomalyAnalysis.

##### 8.4.X.1.2	Enabling data

##### 8.4.X.1.3	Analytics output

The specific information elements of the analytics output (MDA report) for KPI anomaly analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.X.1.3-1.

Table 8.4.X.1.3-1: Analytics output for KPI Anomaly Analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| Anomalous KPI Name | The name of the KPI which is identified or predicted as anomalous, the KPI name refers to the definition in TS 28.554[x]; | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Anomalous KPI Value | The value of the KPI which is identified or predicted as anomalous. | M | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Managed Objects of anomalous KPI | The object instances where the anomalous KPI is applicable, e.g., SubNetwork Instance, NetworkSlice Instance | M | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

Editor’s note: KPIs to be analyzed are specified in the MDAreportcontrol IOC.

## MnS component type C

End of modified section