3GPP TSG-SA5 Meeting #139-e 	S5-215208rev4

- e-meeting, 11 - 20 October 2021

- Source:	Huawei

- Title:	Add MDA analysis report reporting related service component

- Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

This contribution is for approval.

# 2	References

[1]	3GPP TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2] 	3GPP TS 28.104-000 Management and orchestration; Management Data Analytics (MDA)

# 3	Rationale

This pCR is proposed to add MDA analysis report reporting related service component in TS 28.104[2].

# 4	Detailed proposal

It is proposed to add the following chapter in TS 28.104[2].

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.500: "Telecommunication management; Self-Organizing Networks (SON); Concepts and requirements".

[3]	3GPP TS28.535: “Management services for communication service assurance; Requirements”.

[4]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[5]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[6]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[7]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[8]	3GPP TS 28.405: "Telecommunication management, Quality of Experience (QoE) measurement collection; Control and configuration".

[9]	3GPP TS 28.406: "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport".

[10]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[11]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[y]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[z]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

Next Change

# 9	Information model definitions for MDA

## 9.1       Imported and associated information entities

### 9.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [y], IOC, ManagedFunction | ManagedFunction |
| TS 28.622 [y], IOC, ManagedElement | ManagedElement |
| TS 28.622 [y], IOC, SubNetwork | SubNetwork |

### 9.1.2	Associated information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [y], IOC, ManagedElement | ManagedElement |
| TS 28.622 [y], IOC, SubNetwork | SubNetwork |

## 9.2       Class diagram

### 9.2.1       Relationships

This clause provides the relationships of relevant classes in UML.

Figure 9.2.1-1: NRM fragment for MDA Analytics control

Note1: The Subnetwork can be RAN Subnetwork or CN Subnetwork or Cross-domain Subnetwork, depends on the detailed use case.

Note2: MDAReportControl can be namecontained by Subnetwork.

Editor’s notes: whether MDAReportControl can be contained in other entity (e.g. managedElement and MnF) is FFS.

### 9.2.2       Inheritance

Figure 9.2.2-1: Inheritance Hierarchy

## 9.3       Class definitions

### 9.3.1          ClassName <<MDAAnalyticsControl>>

#### 9.3.1.1        Definition

This <<IOC>> MDAAnalyticsControl represents the MDA analysis report request Control and Configuration parameters of MDA analysis service capabilities.

To request a desired MDA analysis report, a MDAS consumer has to create MDAAnalyticsControl object instance on the MDAS producer.

#### 9.3.1.2        Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| analyticsType | M | T | T | F | T |

#### 9.3.1.3        Attribute constraints

None.

#### 9.3.1.4        Notifications

The common notifications defined in clause 4.5 as defined in TS 28.541[z] are valid for this IOC, without exceptions or additions.

## 9.4       Attribute definitions

### 9.4.1       Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| analyticsType | It indicates the type of MDA capability for analysis. AllowedValues: Editor's Note: the allowed value is FFS which depends on the detailed analytic use case. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |

### 9.4.2       Constraints

## 9.5.1       Alarm notifications

9.5.2       Configuration notifications

9.5.3       Threshold Crossing notifications

Next Change

# 10	MDA related service components

Editor’s note: Whether putting section 8 (MDA capability data definitions) and section 9 (Information model definitions for MDA) inside section 10 (MDA related service components) is to be evaluated for better readability when we have related content in this specification.

## 10.1	MDA MnS Service components

### 10.1.1	General

TBD

### 10.1.2	MDA report request and control

#### 10.1.2.1	Service components

##### 10.1.2.1.1	Service component A

See clause 11.1.1 of TS 28.532 [x] for operations related to MDA request control.

##### 10.1.2.1.2	Service component B

See MDAAnalyticsControl IOC defined in clause 9.

##### 8

##### 8

## MnS component type B

End of changes