3GPP TSG-SA5 Meeting #139-e	S5-215212

- Online, , 11th Oct 2021 - 20th Oct 2021

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0586 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR 28.541 Add C-SON CCO NRM model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSON_5G |  |  |  |  |  | Date: |  |  | 2021-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To support CCO use case in TS 28.313 [57], this contribution proposes to add CCCOFunction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add CCCOFunction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support CCO use case in TS 28.313 [57]. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.1, 4.2.1.2, 4.3.X (new), 4.3.Y (new), 4.3.Z (new), 4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Change

#### 4.2.1.1	Relationships

Figure 4.2.1.1-X: NRM fragment for CCO Management

2nd Change

#### 4.2.1.2	Inheritance

Figure 4.2.1.2-1: Inheritance Hierarchy

3rd Change

### 4.3.X	CCCOFunction

#### 4.3.X.1	Definition

This IOC contains attributes to support the C-SON function of Capacity and Coverage optimization (See clause 7.2.3 in TS 28.313 [57]).

NOTE: in the case where multiple CCCOFunction MOIs exist at different levels of the containment tree, the CCCOFunction MOI at the lower level overrides the CCCOFunction MOIs at higher level(s) of the same containment tree.

#### 4.3.X.2	Attributes

The CCCOFunction IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cCCOPolicy | M | T | T | F | T |
| cCCOSwitch | M | T | T | F | T |

#### 4.3.X.3	Attribute constraints

None.

#### 4.3.X.4	Notifications

The common notifications defined in sub clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.Y	CCCOPolicy <<dataType>>

#### 4.3.Y.1	Definition

This data type represents the attributes of radio coverage policy content. It consists of condition and the corresponding actions applied for C-SON function of radio Capacity and Coverage optimization.

#### 4.3.Y.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cCCOPolicyCondition | M | T | T | F | T |
| cCCOPolicyAction | M | T | T | F | T |

#### 4.3.Y.3	Attribute constraints

None.

#### 4.3.Y.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.Z	CCCOPolicyAction <<dataType>>

#### 4.3.Z.1	Definition

This data type represents the information of parameters to be adjusted to optimize radio coverage.

#### 4.3.Z.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| downlinkTransmitPowerInfo | O | T | T | F | T |
| antennaTiltInfo | O | T | T | F | T |
| antennaAzimuthInfo | O | T | T | F | T |
| digitalTiltInfo | O | T | T | F | T |
| digitalAzimuthInfo | O | T | T | F | T |
| coverageShapeList | O | T | T | F | T |

#### 4.3.Z.3	Attribute constraints

None.

#### 4.3.Z.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.A	ParameterInfo <<dataType>>

#### 4.3.A.1	Definition

This data type represents the adjustment range for parameters.

#### 4.3.A.2	Attributes

The data type includes the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| maxValue | M | T | T | F | T |
| minValue | M | T | T | F | T |

Editor’s NOTE: New attributes are FFS.

#### 4.3.A.3	Attribute constraints

None.

#### 4.3.A.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

4th Change

## 4.4	Attribute definitions

### 4.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| administrativeState | It indicates the administrative state of the NRCellDU. It describes the permission to use or prohibition against using the cell, imposed through the OAM services. allowedValues: LOCKED, SHUTTING DOWN, UNLOCKED. The meaning of these values is as defined in ITU-T Recommendation X.731 [18]. See Annex A for Relation between the "Pre-operation state of the gNB-DU Cell" and administrative state relevant in case of 2-split and 3-split deployment scenarios. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: LOCKED isNullable: False |
| cCCOPolicy | It indicates condition and the corresponding actions applied for the C-SON function of radio Capacity and Coverage optimization. allowedValues: N/A | type: CCCOPolicy multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cCCOPolicyCondition | It indicates the condition which will trigger the actions to optimize radio coverage. For example, allowedValues for the condition can be “weak coverage”, “pilot pollution”, “overshoot coverage”. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cCCOPolicyAction | It indicates information of the parameters which can be adjusted to optimize radio coverage. allowedValues: N/A | type: CCCOPolicyAction multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| downlinkTransmitPowerInfo | It indicates adjustment range (including maximum value, minimum value) of downlinkTransmitPower to optimize radio coverage. allowedValues: N/A | type: ParameterInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| antennaTiltInfo | It indicates adjustment range (including maximum value, minimum value) of antennaTilt to optimize radio coverage. allowedValues: N/A | type: ParameterInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| antennaAzimuthInfo | It indicates adjustment range (including maximum value, minimum value) of antennaAzimuth to optimize radio coverage. allowedValues: N/A | type: ParameterInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| digitalTiltInfo | It indicates adjustment range (including maximum value, minimum value) of digitalTilt to optimize radio coverage. allowedValues: N/A | type: ParameterInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| digitalAzimuthInfo | It indicates adjustment range (including maximum value, minimum value) of digitalAzimuth to optimize radio coverage. allowedValues: N/A | type: ParameterInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| coverageShapeList | It indicates the coverageShape which can be selected to optimize radio coverage. allowedValues: 0 : 65535 | type: Integer multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| maxValue | It indicates the maximum value of the parameter. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| minValue | It indicates the minimum value of the parameter. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cCCOSwitch | This attribute determines whether the centralized SON CCO Function is enabled or disabled. allowedValues: TRUE,FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of changes