3GPP TSG-SA5 Meeting #139-e 	S5-215217

- e-meeting, 11 - 20 October 2021

- Source:	Huawei

- Title:	pCR 28.104 MDA assisted Energy Saving

- Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

This contribution is for approval.

# 2	References

[1]	3GPP TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	S5-212369 New WID Enhancements of Management Data Analytics Service

[x]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[y]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[z]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)"

[w]	3GPP TS 28.104 “Management and orchestration; Management Data Analytics”

# 3	Rationale

In the new WI of ‘Enhancements of Management Data Analytics Service’ MDA assisted Energy Saving is one of the MDA capabilities. This contribution proposes the description and requirements of the use case of the MDA assisted Energy Saving.

# 4	Detailed proposal

It is proposed to add MDA assisted Energy Saving as one of the MDA capabilities in TS 28.104 [1].

1st modified section

## 7.2	MDA capabilities

### 7.2.x	MDA assisted Energy Saving (UC-ES_MDA)

#### 7.2.x.1	Description

3GPP management system shall be able to provide energy saving related issues related issue analysis.

#### 7.2.x.2	Use cases

Energy saving is a critical issue for the 5G operators. Operators are aiming at decreasing power consumption in 5G networks to lower their operational expense with energy saving management solutions. In this use case, energy saving is achieved by activating the energy saving mode of the NR capacity booster cell, and the cell energy saving activation decision making is typically based on the load information of the related cells, the energy saving policies set by operators and the energy saving instructions provided by MDA producer.

To achieve an optimized balance between the energy consumed and the network performance, MDA can be used to assist the MDA consumer to make energy saving decisions. The MDA producer can analyse the energy saving related performance measurements (e.g. PDCP data volume of cells, power consumption, etc.) to provide the analytics results of current network energy efficiency. The MDAS producer can analyze historical load related information to predict the load variation of cells at some future time. The prediction result of these information can then be used by operators to make energy-saving policies. The MDA producer may also provide some energy saving related recommendation in the MDA report.

#### 7.2.x.3		Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ES_MDA-CON-1 | 3GPP management system shall have the capability to identify the energy efficiency issue including high energy consumption, low energy efficiency. | (UC-ES_MDA) MDA assisted Energy Saving |
| REQ-ES_MDA-CON-2 | 3GPP management system shall have the capability to identify the cell or location area of where the indicated energy saving issue exists. | (UC-ES_MDA) MDA assisted Energy Saving |
| REQ-ES_MDA-CON-3 | 3GPP management system shall have the capability to utilize the predictions information of the traffic load trends to assist achieving energy saving. | (UC-ES_MDA) MDA assisted Energy Saving |
| REQ-ES_MDA-CON-4 | 3GPP management system shall have the capability to identify different traffic scenarios, e.g., residential district, commercial area, suburban, to conduct energy saving analysis. | (UC-ES_MDA) MDA assisted Energy Saving |

2nd modified section

## 8.Y	MDA reporting management service

### 8.Y.3	MnS component type C

| Attribute name | Support qualifier | Properties |
| --- | --- | --- |
| MDA Assisted Energy Saving Analysis report | O |  |

#### 8.Y.3.1	MDA assisted energy saving analysis

Table 8.x.2-1: MDA assisted energy saving analysis report

| Information | Description | Support qualifier | Properties |
| --- | --- | --- | --- |
| Location | The geographical area or list of cells where the energy saving issue exists，e.g., too high energy consumption exists. | M | FFS |
| Trends of the traffic load | Predictions related to the trends of traffic load (e.g., PRB utilization rate) which could be used as references for making decision on energy saving. | M | FFS |
| RootCause | Indicates the root cause of energy saving issue. | O | FFS |

3rd modified section

# 9	Workflow for MDA management

## 9.X	MDA assisted energy saving Analysis

### 9.X.1	General

The MDA management services as defined in 8.2 and 8.3 are used to expose MDA assisted energy saving analytics from the MDAS producer to the MDAS consumer.

The MDAS producer shall be able to provide the following analytics (see clause 8.Y.3.1), which can be included in the MDA report:

-	Location

-	Trends of the traffic load

-	Root Cause

# Annex B (informative): Analytics Input data example

## B.1	Analytics Input data example

### B.1.X  MDA assisted energy saving Analytics inputs

End of modification