3GPP TSG-SA5 Meeting #139-e 	S5-215217rev3

- e-meeting, 11 - 20 October 2021

- Source:	Huawei, AsiaInfo, China Telecom

- Title:	pCR 28.104 MDA assisted Energy Saving

- Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

This contribution is for approval.

# 2	References

[1]	3GPP TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	S5-212369 New WID Enhancements of Management Data Analytics Service

[x]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[y]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[z]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)"

[w]	3GPP TS 28.104 “Management and orchestration; Management Data Analytics”

# 3	Rationale

In the new WI of ‘Enhancements of Management Data Analytics Service’ MDA assisted Energy Saving is one of the MDA capabilities. This contribution proposes the description and requirements of the use case of the MDA assisted Energy Saving.

# 4	Detailed proposal

It is proposed to add MDA assisted Energy Saving as one of the MDA capabilities in TS 28.104 [1].

1st modified section

## 7.2	MDA capabilities

### 7.2.x	MDA assisted Energy Saving (UC-ES_MDA)

#### 7.2.x.1	Description

3GPP management system shall have the capability to provide energy saving related issues related issue analysis.

#### 7.2.x.2	Use cases

Energy saving is a critical issue for the 5G operators. Operators are aiming at decreasing power consumption in 5G networks to lower their operational expense with energy saving management solutions. In this use case, energy saving is achieved by activating the energy saving mode of the NR capacity booster cell, and the cell energy saving activation decision making is typically based on the load information of the related cells, the energy saving policies set by operators and the energy saving instructions provided by MDA producer.

To achieve an optimized balance between the energy consumed and the network performance, MDA can be used to assist the MDA consumer to make energy saving decisions.

The MDA producer can analyse the energy saving related performance measurements (e.g. PDCP data volume of cells, power consumption, etc.) to provide the analytics results of current network energy efficiency. The MDAS producer can analyze historical load related information to predict the load variation of cells at some future time.

The prediction result of these information can then be used by operators to make energy-saving policies. The MDA producer may provide a traffic prediction model for making energy saving decisions by introducing machine learning algorithms. The MDA producer may also provide some energy saving related recommendation in the MDA report.

After the recommendations have been taken to make energy saving execution, the MDA producer may analyze network performance data, alarms, and user experience data (including QoE data) in real time. These analyses may affect energy-saving decisions. The MDA producer should optimize the recommendations to guarantee the user experience at the same level.

#### 7.2.x.3		Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ES_MDA-CON-1 | 3GPP management system shall have the capability to identify the energy efficiency issue including high energy consumption, low energy efficiency. | (UC-ES_MDA) MDA assisted Energy Saving |
| REQ-ES_MDA-CON-2 | 3GPP management system shall have the capability to identify the cell or location area of where the indicated energy efficiency issue exists. | (UC-ES_MDA) MDA assisted Energy Saving |
| REQ-ES_MDA-CON-3 | 3GPP management system shall have the capability to utilize the predictions information of the traffic load trends to assist achieving energy saving. | (UC-ES_MDA) MDA assisted Energy Saving |
| REQ-ES_MDA-CON-4 | 3GPP management system shall have the capability to identify different traffic scenarios, e.g., residential district, commercial area, suburban, to conduct energy saving analysis. | (UC-ES_MDA) MDA assisted Energy Saving |

2nd modified section

## 8.4	Data definitions per MDA capability

### 8.4.X	Energy efficiency related issues

#### 8.4.X.1	MDA assisted energy saving

##### 8.4.X.1.1	MDA type

The MDA type for MDA assisted energy saving is: Energyefficiency.EnergySavingAnalysis.

##### 8.4.X.1.2	Enabling data

##### 8.4.X.1.3	Analytics output

The specific information elements of the analytics output (MDA report) for MDA assisted energy saving, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.1.1.3-1.

Table 8.4.X.1.3-1: Analytics output for MDA assisted energy saving

| Information | Description | Support qualifier | Properties |
| --- | --- | --- | --- |
| Location | The list of cells where the energy efficiency issue exists. | M | FFS |
| Trends of the traffic load | Predictions related to the trends of traffic load (e.g., PRB utilization rate) which could be used as references for making decision on energy saving. | M | FFS |
| RootCause | Indicates the root cause of energy efficiency issue. | O | FFS |
| Current Energy Saving State of the cells | The information of the current energy saving state of the cells. The allowed value is one of the enumerated values: notEnergySavingState, EnergySavingState. | O | FFS |
| ML model information | The information of the optimal ML model for energy saving related information prediction. The ML model information can be used by MDAS consumer to retrieve the ML model. The ML model information can be ML model file address. | O | FFS |

Editor Note: How to format the energy saving recommendations may need discussion. The energy saving for UPF and energy saving recommendations can be added later.

End of modification