- 3GPP TSG-SA5 Meeting #139e	S5-215267

- E-meeting, 11 - 20 October 2021

- Source:	Huawei, Samsung

- Title:	pCR 28.538 add ECSFunction IOC

- Document for:	Approval

- Agenda Item:	6.4.21

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	TR 28.814 Management and orchestration; Study on enhancements of edge computing management

[2]	3GPP TS 28.538-020 “Management and orchestration; Edge Computing Management”

# 3	Rationale

It is suggested to add ECSFunction IOC for Edge Computing management.

# 4	Detailed proposal

It is proposed to apply following changes in [2].

1st modified section

## 6.x	Class definitions

Editor’s Note:  The definition of IOCs is not complete. It is expected additional attributes, as needed

### 6.x.1	ECSFunction

#### 6.x.1.1	Definition

This IOC represents the ECS functionality for supporting Edge Computing. For more information about the ECS, see 3GPP TS 23.558 [y].

#### 6.x.1.2	Attributes

The ECSFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[z]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ecsAddress | M | T | T | F | T |
| providerIdentifier | O | T | T | F | T |
| eDNConnectionInfo | M | T | T | F | T |

#### 6.x.1.3	Attribute constraints

None

6.x.2		EDNConnectionInfo <<datatype>>

6.x.2.1 		Definition

This datatype represent the EDN connection information.

6.x.2.2		Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dNN | M | T | T | F | T |
| eDNServiceArea | M | T | T | F | T |

- 6.3.3.3	Attribute constraints

None

2nd modified section

## 6.y	Attribute definitions

### 6.y.1	Attribute properties

The following table defines the attributes that are present in several Information Object Classes (IOCs) of the present document.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| ecsAddress | One or more URLs and/or IP Address(es) of ECS(s) (See TS 23.558 [y]). allowedValues: N/A | type: String multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| providerIdentifier | The identifier of the ECSP that provides the ECS(See TS 23.558 [y]). allowedValues: N/A | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| eDNConnectionInfo | It defines the set of information needed to connect to an EDN. | type: EDNConnectionInfo multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| eDNServiceArea | This parameter defines the service location for the EDN. | type: ServingLocation multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |

3rd modified section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[y]	3GPP TS 23.558 “Architecture for enabling Edge Applications”.

[z]	3GPP TS 28.622 “Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”.

[a]	3GPP TS 28.532 “Management and orchestration; Generic management services”.

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

4th modified section

# 7	Procedural Flows

### 7.x.a	ECS lifecycle management

#### 7.x.a.1	ECS instantiation

Figure 7.x.a.1-1 shows that the PLMN operator or ECSP as the consumer requests the ECS instantiation via the provisioning MnS.

1.   Provisioning MnS Producer receives a request (this will use createMOI operation defined in 3GPP TS 28.532 [a]) with ECS related requirements. The following are the list of requirements, which can be provided with the request as part of attributeListIn parameter of createMOI operation.

a.   ecsAddress: the URLs and/or IP Address(es) of ECS.

b.   providerIdentifier: Identifying the ECSP that provides the ECS.

2.   Provisioning MnS Producer producer returns a response indicating that the instantiation operation is in progress

3.   The NF instance creation procedure as described in 7.10 of [a] is reused to instantiate the ECS VNF instance with the requirements captured in the ECSFunction IOC. The request is to create the MOI for ECSFunction IOC.

4.  In case of ECS VNF instantiation failure, a Notification to indicate the creation of ECSFunction instance has failed.

5.   In case of ECS VNF instantiation success, the producer creates the MOI (Managed Object Instance) for ECSFunction IOC. The MOI shall contain attributes as defined in ECSFunction IOC.

6.  In case of ECS VNF instantiation success, a Notification to indicate the  ECSFunction instance has been created.

7.   The producer configures the new created MOI with corresponding configuration information as per the information model definition in clause 6.x.1.