3GPP TSG-SA5 Meeting #139-e	S5-215350rev1

- Online, , 11th Oct 2021 - 20th Oct 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.158 | CR | 0020 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR 32.158 Add more examples on how to use provisioning operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, REST_SS |  |  |  |  |  | Date: |  |  | 2021-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The annex lacks examples for some provisioning features. However, examples help to increase understandability and thus increases interoperability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Some mor examples are added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of examples may lead to reduced interoperability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.1, A.2.1, A.2.2, A.2.3, A.3.3, A.7.1, A.7.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modification

# A.1	Example information model

The following JSON instance document is used for the examples in this chapter.

{ "SubNetwork": { "id": "SN1", "attributes": { "userLabel": "Berlin NW", "userDefinedNetworkType": "5G", "plmn-id": { "mcc": 456, "mnc": 789 } }, "ManagedElement": [ { "id": "ME1", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "TV Tower" }, "XyzFunction": [ { "id": "XYZF1", "attributes": { "attrA": "xyz", "attrB": 551 } }, { "id": "XYZF2", "attributes": { "attrA": "abc", "attrB": 552 } } ] }, { "id": "ME2", "attributes": { "userLabel": "Berlin NW 2", "vendorName": "Company XY", "location": "Grunewald" } } ], "PerfMetricJob":[ { "id": "J1", "attributes": { "granularityPeriod": "5", "perfMetrics": ["Metric1", "Metric2"], "objectInstances": ["Obj1", "Obj2"] } } ] } }

The corresponding JSON schema is

{ "SubNetwork": { "type": "object", "properties": { "id": { "type": "string" }, "attributes": { "type": "object", "properties": { "userLabel": "string", "userDefinedNetworkType": "string", "plmn-id": { "type": "object", "properties": { "mcc": "integer", "mnc": "integer" } } } }, "ManagedElement": { "type": "array", "items": { "type": "object", "properties": { "id": { "type": "string" }, "attributes": { "type": "object", "properties": { "userLabel": { "type": "string" }, "vendorName": { "type": "string" }, "location": { "type": "string" } } }, "XyzFunction": { "type": "array", "items": { "type": "object", "properties": { "id": { "type": "string" }, "attributes": { "type": "object", "properties": { "attributeA": { "type": "string" }, "attributeB": { "type": "integer" } } } } } } } } }, "PerfMetricJob": { "items": { "type": "object", "properties": { "id": { "type": "string" }, "attributes": { "type": "object", "properties": { "granularityPeriod": { "type": "string" }, "perfMetrics": { "type": "array", "items": { "type": "string" } } }, "objectInstances": { "type": "array", "items": { "type": "string" } } } } } } } } }

The corresponding XML instance document is provided below as well. It may be helpful when evaluating XPath expressions.

<?xml version="1.0" encoding="UTF-8" ?> <SubNetwork> <id>SN1</id> <attributes> <userLabel>Berlin NW</userLabel> <userDefinedNetworkType>5G</userDefinedNetworkType> <plmn-id> <mcc>456</mcc> <mnc>789</mnc> </plmn-id> </attributes> <ManagedElement> <id>ME1</id> <attributes> <userLabel>Berlin NW 1</userLabel> <vendorName>Company XY</vendorName> <location>TV Tower</location> </attributes> <XyzFunction> <id>XYZF1</id> <attributes> <attrA>xyz</attrA> <attrB>551</attrB> </attributes> </XyzFunction> <XyzFunction> <id>XYZF2</id> <attributes> <attrA>abc</attrA> <attrB>552</attrB> </attributes> </XyzFunction> </ManagedElement> <ManagedElement> <id>ME2</id> <attributes> <userLabel>Berlin NW 2</userLabel> <vendorName>Company XY</vendorName> <location>Grunewald</location> </attributes> </ManagedElement> <PerfMetricJob> <id>J1</id> <attributes> <granularityPeriod>5</granularityPeriod> <perfMetrics>Metric1</perfMetrics> <perfMetrics>Metric2</perfMetrics> <objectInstances>Obj1</objectInstances> <objectInstances>Obj2</objectInstances> </attributes> </PerfMetricJob> </SubNetwork>

NOTE:	the following examples do not follow the URI structure specified in clause 4.4 for simplicity reasons. The "data" object in responses is omitted as well.

# A.2	Retrieval of resources

## A.2.1	Retrieval of a single complete resource with HTTP GET

To retrieve a complete "YxzFunction" resource the MnS Consumer might send the following request.

GET /SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF1 HTTP/1.1 Host: example.org Accept: application/json

The response includes the resource representation

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "id": "XYZF1", "attributes": { "attrA": "xyz", "attrB": 551 } }

and might include a key ("XyzFunction") specifying the class name of the returned resource

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "XyzFunction": [ { "id": "XYZF1", "attributes": { "attrA": "xyz", "attrB": 551 } } ] }

In the example above "XyzFunction" is of type array to align with the JSON schema of "XyzFunction" defined in clause A.1. "XyzFunction" may also be an object, since the schema specifing the message body is not required to be identical to the schema specifying the resources contained by a resource.

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "XyzFunction": { "id": "XYZF1", "attributes": { "attrA": "xyz", "attrB": 551 } } }

When using a "data" object the response might look like

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "data": { "XyzFunction": { "id": "XYZF1", "attributes": { "attrA": "xyz", "attrB": 551 } } } }

The exact syntax of the response body is specified by the JSON schema included in the concrete API definition.

## A.2.2	Attribute and attribute field selection on a single resource

To retrieve only the "userLabel" attribute and the "mnc" attribute field of the "plmn-id" attribute of the "SubNetwork", the MnS Consumer might send:

GET /SubNetwork=SN1?attributes=userLabel&fields=/attributes/plmn-id/mcc HTTP/1.1 Host: example.org Accept: application/json

Alternatively one might send as well:

GET /SubNetwork=SN1?fields=/attributes/userLabel,/attributes/plmn-id/mcc HTTP/1.1 Host: example.org Accept: application/json

The response contains only the selected attribute "userLabel" and the selected attribute field "mnc":

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "id": "SN1", "attributes": { "userLabel": "Berlin NW", "plmn-id": { "mnc": 789 } } }

In this example, the MnS Consumer retrieves the "userLabel" and "vendorName" of the "ManagedElement" whose "id" is equal to "ME1":

GET /SubNetwork=SN1/ManagedElement=ME1?attributes=userLabel,vendorName HTTP/1.1 Host: example.org Accept: application/json

The MnS Producer might respond as follows:

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "id": "ME1", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY" } }

The following request selects all attributes:

GET /SubNetwork=SN1/ManagedElement=ME1?fields=/attributes HTTP/1.1 Host: example.org Accept: application/json

It is thus identical to:

GET /SubNetwork=SN1/ManagedElement=ME1 HTTP/1.1 Host: example.org Accept: application/json

Both requests return the complete resource representation with all attributes:

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "id": "ME1", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "TV Tower" } }

The following request returns the first item of of the "perfMetrics" attribute, which is of type array:

GET /SubNetwork=SN1/ManagedElement=ME1/PerfMetricJob=J1?fields=attributes/perfMetrics/0 HTTP/1.1 Host: example.org Accept: application/json

Note indices start with "0" in JSON Pointer. The response may look like:

{ "SubNetwork": { "id": "SN1", "PerfMetricJob":[ { "id": "J1", "attributes": { "perfMetrics": ["Metric1"] } } ] } }

## A.2.3	Retrieval of multiple complete resources using scoping and filtering

The following example selects the "SubNetwork" as base object at scope level "0" and all objects at scope level "1":

GET /SubNetwork=SN1?scopeType=BASE_SUBTREE&scopeLevel=1 HTTP/1.1 Host: example.org Accept: application/json

All objects at scope level "1" are included in the response irrespective of their object class. When using the hierarchical response construction method, the response looks as follows:

{ "SubNetwork": { "id": "SN1", "attributes": { "userLabel": "Berlin NW", "userDefinedNetworkType": "5G", "plmn-id": { "mcc": 456, "mnc": 789 } }, "ManagedElement": [ { "id": "ME1", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "TV Tower" } }, { "id": "ME2", "attributes": { "userLabel": "Berlin NW 2", "vendorName": "Company XY", "location": "Grunewald" } } ], "PerfMetricJob":[ { "id": "J1", "attributes": { "granularityPeriod": "5", "perfMetrics": ["Metric1", "Metric2"], "objectInstances": ["Obj1", "Obj2"] } } ] } }

The response constructed with the flat response construction method may look like:

{ "result": [ { "id": "SN1", "objectClass": "SubNetwork", "objectInstance": "SubNetwork=SN1", "attributes": { "userLabel": "Berlin NW", "userDefinedNetworkType": "5G", "plmn-id": { "mcc": 456, "mnc": 789 } } }, { "id": "ME1", "objectClass": "ManagedElement", "objectInstance": "SubNetwork=SN1,ManagedElement=ME1", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "TV Tower" } }, { "id": "ME2", "objectClass": "ManagedElement", "objectInstance": "SubNetwork=SN1,ManagedElement=ME2", "attributes": { "userLabel": "Berlin NW 2", "vendorName": "Company XY", "location": "Grunewald" } }, { "id": "J1", "ojectClass": "PerfMetricJob", "objectInstance": "SubNetwork=SN1,PerfMetricJob=J1", "attributes": { "granularityPeriod": "5", "perfMetrics": ["Metric1","Metric2"], "objectInstances": ["Obj1","Obj2"] } } ] }

When only objects at scope level "1" shall be returned, the request might look like:

GET /SubNetwork=SN1?scopeType=BASE_NTH_LEVEL&scopeLevel=1 HTTP/1.1 Host: example.org Accept: application/json

The response does not include the attributes of "SubNetwork" any more, only the "id" is included:

{ "SubNetwork": { "id": "SN1", "ManagedElement": [ { "id": "ME1", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "TV Tower" } }, { "id": "ME2", "attributes": { "userLabel": "Berlin NW 2", "vendorName": "Company XY", "location": "Grunewald" } } ], "PerfMetricJob":[ { "id": "J1", "attributes": { "granularityPeriod": "5", "perfMetrics": ["Metric1", "Metric2"], "objectInstances": ["Obj1", "Obj2"] } } ] } }

Similarly, for reading all objects on scope level "2", the MnS Consumer may send:

GET /SubNetwork=SN1?scopeType=BASE_NTH_LEVEL&scopeLevel=2 HTTP/1.1 Host: example.org Accept: application/json

The response includes only "XyzFunction" objects:

{ "SubNetwork": { "id": "SN1", "ManagedElement": [ { "id": "ME1", "XyzFunction": [ { "id": "XYZF1", "attributes": { "attrA": "xyz", "attrB": 551 } }, { "id": "XYZF2", "attributes": { "attrA": "abc", "attrB": 552 } } ] } ] } }

The "PerfMetricJob" is not included altogether, not even with the "id" only. This is because this node is no necessary path component to the scoped objects on the second level.

The following example selects all objects on scope level "1", that have a "location" attribute whose value is equal to "Grunewald":

GET /SubNetwork=SN1?\ scopeType=BASE_NTH_LEVEL&scopeLevel=1\ filter=/*/*[attributes[location="Grunewald"]] HTTP/1.1 Host: example.org Accept: application/json

The response includes one "ManagedElement" object only:

{ "SubNetwork": { "id": "SN1", "ManagedElement": [ { "id": "ME2", "attributes": { "userLabel": "Berlin NW 2", "vendorName": "Company XY", "location": "Grunewald" } } ] } }

The input document to the XPath filter is a document whose root node is the object identified by the path component of the URI and that includes the object representations of the scoped objects. In this example the root node is the "SubNetwork" without the "attributes" node. The input document includes all scoped objects on the scope level "1". These are the two "ManagedElement" objects and the "PerfMetricJob" object:

{ "SubNetwork": { "id": "SN1", "ManagedElement": [ { "id": "ME1", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "TV Tower" } }, { "id": "ME2", "attributes": { "userLabel": "Berlin NW 2", "vendorName": "Company XY", "location": "Grunewald" } } ], "PerfMetricJob":[ { "id": "J1", "attributes": { "granularityPeriod": "5", "perfMetrics": ["Metric1", "Metric2"], "objectInstances": ["Obj1", "Obj2"] } } ] } }

An implementation may be based on available XPath tools. In that case the JSON document may have to be converted to a XML document:

<SubNetwork> <id>SN1</id> <ManagedElement> <id>ME1</id> <attributes> <userLabel>Berlin NW 1</userLabel> <vendorName>Company XY</vendorName> <location>TV Tower</location> </attributes> </ManagedElement> <ManagedElement> <id>ME2</id> <attributes> <userLabel>Berlin NW 2</userLabel> <vendorName>Company XY</vendorName> <location>Grunewald</location> </attributes> </ManagedElement> <PerfMetricJob> <id>J1</id> <attributes> <granularityPeriod>5</granularityPeriod> <perfMetrics>Metric1</perfMetrics> <perfMetrics>Metric2</perfMetrics> <objectInstances>Obj1</objectInstances> <objectInstances>Obj2</objectInstances> </attributes> </PerfMetricJob> </SubNetwork>

In this example the complete "ManageElement" object is the result of applying the XPath expression:

<ManagedElement> <id>ME2</id> <attributes> <userLabel>Berlin NW 2</userLabel> <vendorName>Company XY</vendorName> <location>Grunewald</location> </attributes> </ManagedElement>

XPath predicates allow to specify also ranges. The following example selects objects on scope level "2" that have an attribute with name "attrB" whose value is equal to or greater than 552 and less than 562.

GET /SubNetwork=SN1?\ scopeType=BASE_NTH_LEVEL&scopeLevel=2\ filter=/*/*/*[attributes[attrB>=552 and attrB<562]] HTTP/1.1 Host: example.org Accept: application/json

The response includes one "XyzFunction" object only:

{ "SubNetwork": { "id": "SN1", "ManagedElement": [ { "id": "ME1", "XyzFunction": [ { "id": "XYZF2", "attributes": { "attrA": "abc", "attrB": 552 } } ] } ] } }

An identical response is returned with the underyling information model when using the following requests:

GET /SubNetwork=SN1?\ scopeType=BASE_ALL\ filter=//*[attributes[attrB>=552 and attrB<562]] HTTP/1.1 Host: example.org Accept: application/json

or

GET /SubNetwork=SN1?\ scopeType=BASE_SUBTREE&scopeLevel=2\ filter=//*[attributes[attrB>=552 and attrB<562]] HTTP/1.1 Host: example.org Accept: application/json

or

GET /SubNetwork=SN1?\ scopeType=BASE_ALL\ filter=//XyzFunction[attributes[attrB>=552 and attrB<562]] HTTP/1.1 Host: example.org Accept: application/json

This example returns the containment tree only:

GET /SubNetwork=SN1?scopeType=BASE_ALL&attributes= HTTP/1.1 Host: example.org Accept: application/json

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "SubNetwork": { "id": "SN1", "ManagedElement": [ { "id": "ME1", "XyzFunction": [ { "id": "XYZF1" }, { "id": "XYZF2" } ] }, { "id": "ME2" } ] } }

# A.3	Creation of resources

## A.3.1	Creation of a resource with HTTP PUT

In this example a new "XyzFunction" resource is created. The target URI specifies the location of the new resource. The "id" of the new resource is "XYZF1" and created by the MnS Consumer.

PUT /SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF1 HTTP/1.1 Host: example.org Content-Type: application/json { "XyzFunction": [ { "id": "XYZF1", "attributes": { "attrA": "xyz", "attrB": 551 } } ] }

The response contains the location header and the complete representation of the new resource.

HTTP/1.1 201 Created Date: Tue, 06 Aug 2019 16:50:26 GMT Location: http://example.org/ SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF1 Content-Type: application/json { "XyzFunction": [ { "id": "XYZF1", "attributes": { "attrA": "xyz", "attrB": 551 } } ] }

## A.3.2	Creation of a resource with HTTP POST

When creating a new resource with POST the target URI identifies the parent resource of the new resource to be created. The identifier of the new resource is created by the MnS Producer, hence the "id" is equal to "null" in the POST request. If the "id" carries a value, then the MnS Producer may consider that as a non-binding recommendation by the MnS Consumer.

POST /SubNetwork=SN1/ManagedElement=ME1 HTTP/1.1 Host: example.org Content-Type: application/json { "XyzFunction": [ { "id": "null", "attributes": { "attrA": "xyz", "attrB": 551 } } ] }

HTTP/1.1 201 Created Date: Tue, 06 Aug 2019 16:50:26 GMT Location: http://example.org/ SubNetwork=SN1/ManagedElement=ME1/XyzFunction=123e4567-e89b Content-Type: application/json { "XyzFunction": [ { "id": "123e4567-e89b", "attributes": { "attrA": "xyz", "attrB": 551 } } ] }

## A.3.3	Creation of a resource with JSON Patch

This example shows the creation of a resource with JSON Patch. The target URI identifies the resource to be created. The "path" property of the patch is empty ("").

PATCH /SubNetwork=SN1/ManagedElement=ME1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "add", "path": "", "value": { "id": "ME1", "class": "ManagedElement", "attributes": { "userLabel": " Berlin NW 1", "vendorName": "Company XY", "location": "TV Tower" } } } ]

# A.4	Deletion of  resources

## A.4.1	Deletion of a resource with HTTP DELETE

The following example deletes an instance of "ManagedElement".

DELETE /SubNetwork=SN1/ManagedElement=ME2 HTTP/1.1 Host: example.org

HTTP/1.1 204 No Content Date: Tue, 06 Aug 2019 16:50:26 GMT

## A.4.2	Deletion of multiple resources with HTTP DELETE

This example deletes both "XyzFunction" resources.

DELETE /SubNetwork=SN1?scopeType= BASE_NTH_LEVEL&scopeLevel=2 HTTP/1.1 Host: example.org

HTTP/1.1 204 No Content Date: Tue, 06 Aug 2019 16:50:26 GMT

## A.4.3	Deletion of a resource with JSON Patch

The following example deletes an instance of "ManagedElement". The target URI identifies the resource to be deleted. The "path" property of the patch is empty ("").

PATCH /SubNetwork=SN1/ManagedElement=ME1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "remove", "path": "" } ]

# A.5	Complete update of a resource

The following example updates a "XyzFunction" resource. Only the "attrA" attribute is updated with a new value. The "attrB" attribute is set to the old value, but still the "attrB" attribute needs to be present.

PUT /SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF1 HTTP/1.1 Host: example.org Content-Type: application/merge-patch+json { "XyzFunction": [ { "id": "XYZF1", "attributes": { "attrA": "newValue", "attrB": 551 } } ] }

# A.6	Partial update of a resource

## A.6.1	Partial update of a resource with JSON Merge Patch

The first example shows how the attribute "attrA" of the "XyzFunction with the "id" equal to "YXZF1" is changed from "xyz" to "def" using JSON Merge Patch.

PATCH /SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF1 HTTP/1.1 Host: example.org Content-Type: application/merge-patch+json { "XyzFunction": { "id": "XYZF1", "attributes": { "attrA": "def" } } }

In the second example the "mcc" attribute field of the "plmnId" attribute is updated to "654". The employed patch method is again JSON Merge Patch.

PATCH /SubNetwork=SN1 HTTP/1.1 Host: example.org Content-Type: application/merge-patch+json { "SubNetwork": { "id": "SN1", "attributes": { "plmn-Id": { "mcc": 654 } } } }

Note that the value of "SubNetwork" needs to be a JSON object when using standard JSON Merge Patch. JSON arrays are not allowed. This needs to be taken into account when specifying the JSON schema for the PATCH method request body.

## A.6.2	Partial update of a resource with 3GPP JSON Merge Patch

In these examples the same changes as in clause A.6.1 are requested, but 3GPP JSON Merge Patch is used. The value of "XyzFunction" can be an array in this case.

PATCH /SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF1 HTTP/1.1 Host: example.org Content-Type: application/3gpp-merge-patch+json { "XyzFunction": [ { "id": "XYZF1", "attributes": { "attrA": "def" } } ] }

PATCH /SubNetwork=SN1 HTTP/1.1 Host: example.org Content-Type: application/3gpp-merge-patch+json { "SubNetwork": { "id": "SN1", "attributes": { "plmn-Id": { "mcc": 654 } } } }

## A.6.3	Partial update of a resource with JSON Patch

When JSON Patch is used to request the changes described in clause A.6.1, the MnS consumer may send

PATCH /SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "replace", "path": "/attributes/attrA", "value": 654 } ]

and

PATCH /SubNetwork=SN1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "replace", "path": "/attributes/plmn-Id/mcc", "value": 654 } ]

## A.6.4	Partial update of a resource with 3GPP JSON Patch

When 3GPP JSON Patch is used to request the changes described in clause A.6.1 the MnS consumer may send the following

PATCH /SubNetwork=SN1 HTTP/1.1 Host: example.org Content-Type: application/3gpp-json-patch+json [ { "op": "replace", "path": "/ManagedElement=ME1/XyzFunction=XYZF1#attributes/attrA", "value": 654 } ]

and

PATCH /SubNetwork=SN1 HTTP/1.1 Host: example.org Content-Type: application/3gpp-json-patch+json [ { "op": "replace", "path": "#attributes/plmn-Id/mcc", "value": 654 } ]

In the first example the target URI of the HTTP PATCH method identifies the document root.

The value of "path" in the second example is just the URI fragment component beginning with "#".

# A.7	Manipulating multiple resources

## A.7.1	Manipulating multiple resources with 3GPP JSON Merge Patch

In this example the "userLabel" attribute and the "mcc" attribute field of the "subNetwork" resource is updated. A new "XyzFunction" resource with id "XYZF3"is created as well as a new "ManagedElement" resource with id "ME3".

PATCH /SubNetwork=SN1 HTTP/1.1 Host: example.org Content-Type: application/3gpp-merge-patch+json { "SubNetwork": { "id": "SN1", "attributes": { "userLabel": "Berlin NW-1", "plmn-id": { "mcc": 456 } }, "ManagedElement": [ { "id": "ME1", "XyzFunction": [ { "id": "XYZF3", "attributes": { "attrA": "fgh", "attrB": 555 } } ] }, { "id": "ME3", "attributes": { "userLabel": " Berlin NW 3", "vendorName": "Company XY", "location": "Spandau" } } ] } }

In the following example a "XYzFunction" resource is deleted.

PATCH /SubNetwork=SN1 HTTP/1.1 Host: example.org Content-Type: application/3gpp-merge-patch+json { "SubNetwork": { "id": "SN1", "ManagedElement": [ { "id": "ME1", "XyzFunction": [ { "id": "XYZF2", "attributes": null } ] } ] } }

## A.7.2	Manipulating multiple resources with 3GPP JSON PATCH

The same resource modifications as in the previous chapter expressed using 3GPP JSON Patch are given by

PATCH /SubNetwork=SN1 HTTP/1.1 Host: example.org Content-Type: application/3gpp-json-patch+json [ { "op": "replace", "path": "/SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF1#/attributes/userLabel", "value": "Berlin NW-1" }, { "op": "replace", "path": "/SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF1#/attributes/plmn-id/mcc", "value": 654 }, { "op": "add", "path": "/SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF3", "value": { "id": "XYZF3", "attributes": { "attrA": "fgh", "attrB": 555 } } }, { "op": "add", "path": "/SubNetwork=SN1/ManagedElement=ME3", "value": { "id": "ME3", "attributes": { "userLabel": " Berlin NW 3", "vendorName": "Company XY", "location": "Spandau" } } } ]

End of modifications