3GPP TSG SA WG5 Meeting 139-e		S5-215371rev2

- e-meeting, 11th - 20th Oct. 2021

- Source:	AsiaInfo, Alibaba Group

- Title:	Possible solution to support eMnS discovery service

- Document for:	Approval

Agenda Item:	6.5.4

# 1	Decision/action requested

The group is asked to discuss and approve the proposal..

# 2	References

[1]	3GPP TR 28.824: " Study on network slice management capability exposure " V0.2.0

# 3	Rationale

The use cases, issues and potential requirements for eMnS discovery service was discussed and agreed in TR 28.824[1]. This pCR is to propose possible solutions to support eMnS discovery service.

# 4	Detailed proposal

This contribution adds possible solutions for the use case – eMnS discovery service and the use case – eMnS support to discovery systems.

Start of 1st Change

## 7.X Possible solutions for eMnS discovery service

This clause provides potential solutions for the use case of eMnS discovery service (see clause 5.3). It is assumed that the MnS discovery service has been deployed.

To enable communication between eMnS consumers and eMnS producers, eMnS consumers need a mechanism to discover eMnS producers that are available in the 3GPP management system.

When the operator decides to expose a management service (eMnS), the operator must decide which internal MnS(s) should be used as the basis of the eMnS, which internal MnS operations should be abstracted/filtered, and which internal MnS data should be abstracted/filtered. As part of this decision, the operator may use the MnS Discovery Service and the Data Discovery Service to collect information. The operator designs and implements an EGMF which is capable of exposing the eMnS. The EGMF publishes the eMnS data to the appropriate discovery service/system(e.g. eMnS discovery service producer).

The eMnS discovery service consumer sends a request to appropriate discovery service (e.g. eMnS discovery service producer) to obtain the eMnS data. The request may optionally request that the eMnS data is filtered by exposure levels.

End of Change