- 3GPP TSG-SA5 Meeting #139e	S5-215380rev4

- 11 - 20 October 2021, E-meeting

- Source:	Intel

- Title:	pCR 28.104 Add ML support for MDA

- Document for:	Approval

- Agenda Item:	6.4.18

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	TS 28.104 “Management and orchestration; Management Data Analytics”

# 3	Rationale

MDA may be supported by ML capabilities.

MDAS producer may or may not disclose the use of ML for MDA. When MDAS producer disclose the use of ML or MDA, the ML model training needs to be supported.

This pCR is to add specific aspects on ML support for MDA in draft TS 28.104 [1].

# 4	Detailed proposal

1st Modified Section

# 7.X	Machine Learning (ML) for MDA

Editor’s note: it is TBD that whether the MLMT MnS is to be defined in this TS or in a generic TS.

### 7.X.1	Description

#### 7.X.1.1	MDA with ML support

MDA may be supported by ML capabilities.

When MDA is supported by ML capabilities, one example of MDA MnS deployment scenario is  illustrated in Figure 7.X.1.1-1.

Figure 7.X.1.1-1: Deployment example of MDA with ML support

1. 	The Management Function (MDA MnS producer) collects the analytics inputs for MDA.

2.	The MDA MnS consumer requests the MDA reports and controls the MDA reporting from the MDA MnS producer.

3.	The Management Function (MDA MnS producer) processes the analytics inputs for MDA based on the inference of the (trained) ML model, and generates analytics/inference output.

NOTE: How MDA uses the ML model for inference is out of scope of the present specification.

4.	The Management Function (MDA MnS producer) generates the MDA report(s) based on the analytics/inference output, and provides the MDA report(s) to the MDA MnS consumer.

#### 7.X.1.2	ML model training

In operational environment before the ML model is deployed to conduct the inference, it needs to be trained (e,g., by an external entity).

The ML model is trained by the ML Model Training (MLMT) MnS producer, and the training can be triggered by the request(s) from one or more MLMT MnS consumer(s), or initiated by the MLMT MnS producer (e.g., as result of model evalution).

7.X.2	Use cases

#### 7.X.2.1	ML model training triggered by consumer

The ML model training capabilities are provided by an MLMT MnS producer to one or more consumer(s).

Figure 7.X.1-1: ML model training triggered by MLMT MnS consumer

The ML model may be triggered by the request(s) from one or more MLMT MnS consumer(s).

To trigger an ML model training, the MLMT MnS consumer requests the MLMT MnS producer to train the ML model. The consumer may provide the data source(s) that contain(s) the training data which are considered as inputs candidates for training.

The MLMT MnS producer provides a response to the consumer indicating whether the request was accepted.

If the request is accepted, the MLMT MnS producer decides when to start the ML model training with consideration of the request(s) from the consumer(s). Once the training is decided, the producer

-	selects the training data, with consideration of the consumer provided candidate training data. Since the training data directly influences the algorithm and performance of the trained model, the MLMT MnS producer may examine the consumer provided training data and select none, some or all of them. In addition, the MLMT MnS producer may select some other training data that are available.

-	trains the ML model using the selected training data , and

-	provides the training result (including the location of the trained model, etc) to the MLMT MnS consumer (s).

### 7.X.3	Requirements for ML model training for MDA

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-MDA_ML-FUN-1 | The MLMT MnS producer shall have a capability allowing the consumer to request ML model training. | Clause ML model training triggered by consumer (clause 7.X.2.1) |
| REQ-MDA_ML-FUN-2 | The MLMT MnS producer shall have a capability allowing the consumer to specify the data sources containing the candidate training data for ML model training. | ML model training triggered by consumer (clause 7.X.2.1) |
| REQ-MDA_ML-FUN-3 | The MLMT MnS producer shall have a capability to provide the training result (including the location of the trained model) to the consumer. | ML model training triggered by consumer (clause 7.X.2.1) |

End of Modified Sections