- 23GPP TSG-SA5 Meeting #139e	S5-215381rev2

- 11 - 20 October 2021, E-meeting

- Source:	Intel, NEC, China Telecom, HUAWEI

- Title:	pCR 28.104 Add common information elements of analytics outputs

- Document for:	Approval

- Agenda Item:	6.4.18

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]     S5-214143 add structure of TS 28.104 Management Data Analytics (MDA).

# 3	Rationale

There are some information elements that are common for all analytics outputs, i.e., these common information elements form a subset of all analytic outputs.

It would be more organized and clearer if the common information elements can be defined commonly in a central place.

This pCR is to add common information of analytics outputs to a dedicated clause in draft TS 28.104 aligning with the structure in S5-214143 [1].

# 4	Detailed proposal

1st Modified Section

## 8.3	Common information elements of analytics outputs

There are some information elements that are common for all analytics outputs, i.e., these common information elements form a subset of all analytics outputs.

### 8.3.1	Common information element definitions

The common information elements of the analytics outputs are defined in table 8.3.1-1.

Table 7.3-1: Common information elements of analytics outputs

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| MDAType | It indicates the type of analytics conducted by MDA. The allowed values are the MDA type names defined for each MDA capability respectively, in format of “CategoryName.CapabilityName”. | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AnalyticsId | The identifier of the analytics output. | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| RelatedMOIs | The MOIs which the analytics output is for. | M | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AnalyticsOutputGenerationTime | It indicates the time when the analytics output is generated. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AnalyticsStartTime | It indicates the start time of the information that are used for analytics. | O | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AnalyticsEndTime | It indicates the end time of the information that are used for analytics. | O | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of Modified Sections