- 3GPP TSG-SA5 Meeting #139e	S5-215388rev2

- 11 – 20 October 2021, E-meeting

- Source:	Intel

- Title:	pCR Add solutions on charging for 5GS usage based on monitored QoS

- Document for:	Approval

- Agenda Item:	7.5.1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.815 “Study on charging aspects of edge computing”

# 3	Rationale

The following UCs and their corresponding requirements have been captured in TR 28.815 [1].

- 7.1.1.4	Use case #1d: MNO charging subscriber for 5GS based on monitored QoS

- 7.1.1.5	Use case #1e: MNO charging ECSP/ASP for 5GS based on monitored QoS

This pCR is to add the corresponding possible solutions.

# 4	Detailed proposal

1st Modified Section

##### 7.1.4.1.2	Possible solutions for MNO charging subscriber for 5GS usage for edge computing based on monitored QoS

The possible solutions to support the potential requirements REQ-CH_EC_5GS-03 and Key Issue #1c could be based on the 5G data connectivity converged charging architecture defined in TS 32.255 [6], with the extension of including monitored QoS result for URLLC service on Edge Computing in the usage report, as described below.

Solution #1g: Including the monitored QoS result in the usage report

Figure 7.1.4.1.2-1: Subscriber charging for 5GS usage for edge computing based on monitored QoS

The CHF of MNO is configured with quota and reporting threshold for subscriber charging for usage of 5GS capabilities supporting edge computing.

1)	UE starts using an edge application: Triggers is are according to TS 32.255 [6] and TS 32.290 [5].

2)	Charging Data Request [Initial, Quota Requested]: the SMF sends the request to the CHF to reserve a number of units if determined.

3)	Account, Rating, Reservation Control: the CHF checks if corresponding funds can be reserved on the edge application’s account balance, for each Rating Group. If the account has sufficient funds, the CHF rates the request and performs the corresponding reservation for each Rating Group.

4)	 Open CDR: based on policies, the CHF opens a CDR related to the service for edge application.

5)	Charging Data Response [Initial, Quota Granted]: the CHF grants the reserved number of units to SMF for each Rating Group.

6)	Granted Units Supervision: The SMF monitors the consumption of the granted units and the delivered QoS if the monitored QoS result for URLLC service on Edge Computing is requested for charging.

NOTE: how to request the monitored QoS results for charging is not addressed in the present document.

7)	UE edge application usage ongoing: the SMF continues to deliver the service.

8)	Quota management Trigger: A Trigger associated to Quota management is met. Units determination is performed when applicable.

9)	Charging Data Request [Update, Quota Requested, Usage Reporting]: the SMF sends the request to the CHF, to be granted with more unit for the service to continue, and also for reporting the used units. For the usage reporting, the monitored QoS result (e2e packet delay in 5GS) is included.

10)	Account, Rating, Reservation Control: same as step 4, with the option to also deduct the funds corresponding to the usage on the account balance.

11)	Update CDR: based on policies, the CHF updates the CDR with charging data related to the service.

12)	Charging Data Response [Update, Quota Granted]: The CHF grants quota to SMF for the service, with the reserved number of units for each Rating Group.

13)	Service delivery ongoing: the  NF (CTF) continues to deliver the service.

14)	UE stops using the edge application: the SMF is requested to end the service delivery and does this.

15)	Charging Data Request [Termination]: the SMF sends the request to the CHF, for charging data related to the service termination with the final consumed units for each Rating Group.

16)	Account, Rating Control: the CHF performs the service termination process which involve using the reported charging data to rate the usage and deduct the funds corresponding to the usage on the account balance.

17)	 Close CDR: based on policies, the CHF closes the CDR with charging data related to the service termination and the last reported units for each Rating Group.

18)	Charging Data Response [Termination]: The CHF informs the SMF on the result of the request.

Next Modified Section

##### 7.1.4.2.2	Possible solutions for inter-provider charging for 5GS capabilities usage based on monitored QoS

Solution #1j: Aggregation of SMF reported usage based on monitored QoS by CHF

In this solution, the charging mechanism between SMF and CHF for 5GS capabilities usage supporting edge computing based on monitored QoS is the same as Solution #1g, in addition, the CHF aggregates the reported units with monitored QoS results from SMF(s) at a pre-configured interval for each edge application and/or EDN, and reports the aggregated usage to Billing domain via CGF.

Solution #1k: Aggregation of SMF reported usage based on monitored QoS by Billing domain

In this solution, the charging mechanism between SMF and CHF for 5GS capabilities usage supporting edge computing based on monitored QoS is the same as Solution #1g, and CHF reports the usage with monitored QoS results in the CDRs per UE. It would be the billing domain that then aggregates the usage based on the received CDRs and edge application indication in these CDRs.

Next Modified Section

#### 7.1.5.2	Evaluation of possible solutions for MNO charging subscriber for 5GS usage for edge computing based on monitored QoS

This clause evaluates the following solution for MNO charging subscriber for 5GS usage for edge computing based on monitored QoS:

Solution #1g: Including the monitored QoS result in the usage report.

This solution enables the SMF to monitor the QoS for charging purpose by the PCC rule or an explicit request from CHF, and include the monitored QoS result in the usage report. This solution is feasible for charging subscriber for 5GS usage for edge computing based on monitored QoS.

Next Modified Section

#### 7.1.5.4	Evaluation of possible solutions for inter-provider charging for 5GS capabilities usage based on monitored QoS

This clause evaluates the following solutions for inter-provider charging for 5GS capabilities usage:

Solution #1j: Aggregation of SMF reported usage based on monitored QoS by CHF.

This solution enables the CHF to collect the 5GS usage supporting edge computing based on the monitored QoS per UE from SMF and aggregate the usage for each edge application. This solution can support PEC of the inter-provider charging for 5GS capabilities usage based on monitored QoS supporting each edge application, and save the bandwidth from CHF to Billing domain for transferring the usage for every UE.

Solution #1k: Aggregation of SMF reported usage based on monitored QoS by Billing domain.

This solution requires the CHF to report the 5GS usage supporting each edge application based on monitored QoS per UE, which has no additional changes on charging interfaces to the solution #1g for MNO charging subscriber for 5GS usage based on monitored QoS.

Next Modified Section

### 7.1.6	Conclusion

#### 7.1.6.2	Conclusion of possible solutions for MNO charging subscriber for 5GS usage for edge computing based on monitored QoS

Based on the evaluation in clause 7.1.5.2, the Solution #1g is feasible for MNO charging subscriber for 5GS usage for edge computing based on monitored QoS.

Next Modified Section

#### 7.1.6.4	Conclusion of possible solutions for inter-provider charging for 5GS capabilities usage based on monitored QoS

Based on the evaluation in clause 7.1.5.4, the Solution #1k is the most feasible and preferrable solution for inter-provider charging for 5GS capabilities usage based on monitored QoS.

End of Modified Sections