| 3GPP TR 28.815 V1.3.0 (2021-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Charging management; Study on charging aspects of edge computing (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

1	Scope	9

2	References	9

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Symbols	10

3.3	Abbreviations	10

4	Business roles	10

5	Concepts and overview	11

6	Architecture considerations	11

6.1	5GS and edge application enabling architecture assumptions	11

6.1.1	Edge Computing enabling architectures	11

6.1.2	5GS architecture for Edge Computing	11

6.1.3	Architecture for enabling edge applications	12

6.2	Potential converged charging architectures for edge computing	12

6.2.1	General	12

7	Charging scenarios and key issues	13

7.1	5GS capability usage based charging for edge computing	13

7.1.1	Use cases	13

7.1.1.1	Use case #1a: MNO charging subscriber for 5GS capabilities usage for accessing edge application	13

7.1.1.2	Use case #1b: MNO charging ECSP for 5GS capabilities usage	13

7.1.1.3	Use case #1c: MNO charging ASP for 5GS capabilities usage	14

7.1.1.4	Use case #1d: MNO charging subscriber for 5GS based on monitored QoS	14

7.1.1.5	Use case #1e: MNO charging ECSP/ASP for 5GS based on monitored QoS	14

7.1.2	Potential charging requirements	14

7.1.2.1	Potential requirements for subscriber based charging	14

7.1.2.2	Potential requirements for inter-provider based charging	15

7.1.3	Key issues	15

7.1.4	Possible solutions	15

7.1.4.1	Possible solutions for subscriber based charging	15

7.1.4.1.1	Possible solutions for MNO charging subscriber for 5GS usage for accessing edge application	15

7.1.4.1.2	Possible solutions for MNO charging subscriber for 5GS usage for edge computing based on monitored QoS	16

7.1.4.2	Possible solutions for inter-provider based charging	18

7.1.4.2.1	Possible solutions for inter-provider charging for 5GS capabilities usage	18

7.1.4.2.2	Possible solutions for inter-provider charging for 5GS capabilities usage based on monitored QoS	20

7.1.4.3	Possible solutions for edge application access charging via EAS	21

7.1.5	Evaluation	23

7.1.5.1	Evaluation of possible solutions for MNO charging subscriber for 5GS usage for accessing edge application	23

7.1.5.2	Evaluation of possible solutions for MNO charging subscriber for 5GS usage for edge computing based on monitored QoS	23

7.1.5.3	Evaluation of possible solutions for inter-provider charging for 5GS capabilities usage	24

7.1.5.4	Evaluation of possible solutions for inter-provider charging for 5GS capabilities usage based on monitored QoS	24

7.1.6	Conclusion	24

7.1.6.1	Conclusion of possible solutions for MNO charging subscriber for 5GS usage for accessing edge application	24

7.1.6.2	Conclusion of possible solutions for MNO charging subscriber for 5GS usage for edge computing based on monitored QoS	24

7.1.6.3	Conclusion of possible solutions for inter-provider charging for 5GS capabilities usage	24

7.1.6.4	Conclusion of possible solutions for inter-provider charging for 5GS capabilities usage based on monitored QoS	25

7.2	Edge enabling infrastructure resource charging	25

7.2.1	Use cases	25

7.2.1.1	Use case #2a: Edge enabling infrastructure resource usage charging	25

7.2.1.2	Use case #2b: Edge enabling infrastructure resource performance charging	25

7.2.2	Potential charging requirements	25

7.2.3	Key issues	25

7.2.4	Possible solutions	26

7.2.4.1	Potential charging architectures	26

7.2.4.2	Possible solutions for edge enabling infrastructure resource usage charging	27

7.2.4.3	Possible solutions for edge enabling infrastructure resource performance charging	30

7.2.5	Evaluation	30

7.2.6	Conclusion	30

7.3	Charging for edge application server deployment	31

7.3.1	Use case	31

7.3.1.1	Use case #3a: Edge enabling application server deployment charging	31

7.3.2	Potential charging requirements	31

7.3.3	Key issues	31

7.3.4	Possible solutions	31

7.3.4.1	Potential charging architectures	31

7.3.4.2	Possible solutions for EAS deployment charging	33

7.3.5	Evaluation	35

7.3.6	Conclusion	35

7.4	Charging for edge enabling services usage	36

7.4.1	Use case	36

7.4.1.1	Use case #4a: ECSP charging ASP for indirect 3GPP Core network capabilities usage	36

7.4.1.2	Use case #4b: ECSP charging ASP for edge enabling services provided by ECSP	36

7.4.2	Potential charging requirements	36

7.4.3	Key issues	37

7.4.4	Possible solutions	37

7.4.4.1	Potential charging architectures	37

7.4.4.2	Possible solutions for EAS registration charging	38

7.4.4.3	Possible solutions for EAS discovery charging	39

7.4.4.4	Possible solutions for EAS application context transfer charging	40

7.4.4.5	Possible solutions for UE location obtaining charging	42

7.4.4.5.1	One-time UE location request	42

7.4.4.5.2	UE location subscription and notification	43

7.4.4.6	Possible solutions for Application Client information subscription and notification charging	45

7.4.4.7	Possible solutions for ACR management events subscription charging	47

7.4.4.8	Possible solutions for charging for Session with QoS	48

7.4.4.9	Possible solutions for aggregated charging	51

7.4.5	Evaluation	53

7.4.5.1	Evaluation of possible solutions for EAS registration charging	53

7.4.5.2	Evaluation of possible solutions for EAS discovery charging	53

7.4.5.3	Evaluation of possible solutions for EAS application context transfer charging	53

7.4.5.4	Evaluation of possible solutions for UE location obtaining charging	53

7.4.5.5	Evaluation of possible solutions for Application Client information subscription and notification charging	53

7.4.5.6	Evaluation of possible solutions for ACR management events subscription charging	54

7.4.5.7	Evaluation of possible solutions for Session with QoS charging	54

7.4.6	Conclusion	54

7.4.6.1	Conclusion of possible solutions for EAS registration charging	54

7.4.6.2	Conclusion of possible solutions for EAS discovery charging	54

7.4.6.3	Conclusion of possible solutions for EAS application context transfer charging	54

7.4.6.4	Conclusion of possible solutions for UE location obtaining charging	54

7.4.6.5	Conclusion of possible solutions for Application Client information subscription and notification charging	54

7.4.6.6	Conclusion of possible solutions for ACR management events subscription charging	54

7.4.6.7	Conclusion of possible solutions for Session with QoS charging	55

8	Conclusions and recommendations	55

Annex A (informative): Change history	56

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document studies the charging aspects of Edge Computing based on:

-	architecture, procedures and information flows for enabling Edge Applications over 3GPP network specified in TS 23.558 [4].

-	capabilities for 5GS to support edge computing specified in TS 23.501 [3], TS 23.502 [9] and TS 23.503 [10] incorporating enhancements from TR 23.748 [11] conclusions.

The ETSI ISG MEC framework is also considered.

The charging aspects encompasses:

-	End user charging when accessing edge applications enabled over 3GPP networks.

-	Inter-Provider charging by considering the different business roles (i.e. ASP, ECSP and MNO).

The investigation includes different charging scenarios with potential business requirements, alternative solutions with potential impact on charging architecture, charging functions and charging procedures.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[3]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[4]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[5]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[6]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G Data connectivity domain charging; stage 2".

[7]	3GPP TS 32.256: "Telecommunication management; 5G connection and mobility domain charging; Stage 2".

[8]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[9]	3GPP TS 23.502: "Procedure for the 5G System (5GS); Stage 2".

[10]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[11]	3GPP TR 23.748: "Study on enhancement of support for Edge Computing in 5G Core network (5GC)".

[12]	3GPP TR 28.814: "Study on enhancements of edge computing management".

[13]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[14]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[15]	3GPP TR 28.550: "Management and orchestration; Performance assurance".

[16]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[17]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[18]	3GPP TS 28.522: "Telecommunication management; Performance Management (PM) for mobile networks that include virtualized, network functions; Stage 2".

[19]	ETSI GS NFV-IFA 008 (V3.4.1): "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AC	Application Client

ACR	Application Context Relocation

ASP	Application Service Provider

CEF	Charging Enablement Function

CHF	Charging Function

EAS	Edge Application Server

ECS	Edge Configuration Server

ECSP	Edge Computing Service Provider

EDN	Edge Data Network

EES	Edge Enabler Server

MNO	Mobile Network Operator

MnS	Management Service

# 4	Business roles

Edge computing in 5G environment involves the services or capabilities provided by multiple service providers and entities in the form of following business roles:

-	ASP providing edge application service to the subscribers;

-	ECSP providing edge data network environment (e.g. infrastructure and platform) and edge enabling service (e.g. EAS deployment, registration and discovery) to ASP;

-	MNO providing 5GS capabilities supporting edge computing.

In the deployments, there could be scenarios where one or more of the business roles are supported by a single enterprise, i.e., one enterprise may play a single business role or multiple business roles, for instance the enterprise may only play the MNO role, or play both MNO and ESCP roles. The present document does not impose any restrictions to the possible deployment scenarios.

# 5	Concepts and overview

There could be various business models in terms of charging between the service providers and the served consumer(s) for edge computing in 5G environment in manifold business modes such as B2C, B2B and B2B2X, for example (but not limited to):

-	Subscriber charging

-	MNO charges the subscribers for usage of 5GS capabilities supporting edge computing.

-	Service-provider charging

-	MNO charges the industry consumer (e.g., ECSP) for 5GS capabilities provided to support edge computing in total instead of for each individual subscriber;

-	ECSP charges the ASP for the edge enabling service.

The solutions described in the present document aim to flexibly support the charging scenarios for edge computing in 5G environment without restriction to any specific business model.

# 6	Architecture considerations

## 6.1	5GS and edge application enabling architecture assumptions

### 6.1.1	Edge Computing enabling architectures

For enabling edge applications, the baseline architecture is per clause 6.1.3 of the present document; for supporting edge computing in 5GS, the architecture defined in TS 23.501 [3] per clause 6.1.2 of the present document applies.

### 6.1.2	5GS architecture for Edge Computing

The following architecture figure shows 5GS architecture (non-roaming) as defined in TS 23.501 [3] for supporting Edge Computing.

The support of Edge Computing by using this architecture is specified in clause 5.13 of TS 23.501 [3].

Figure 6.1.2-1: 5GS architecture (non-roaming)

### 6.1.3	Architecture for enabling edge applications

The Figure 6.1.3-1 shows the architecture for enabling edge applications as defined in TS 23.558 [4].

Figure 6.1.3-1: Architecture for enabling edge applications

The Edge Data Network is a local Data Network. Edge Application Server(s) and the Edge Enabler Server are contained within the EDN. The Edge Configuration Server provides configurations related to the EES, including details of the Edge Data Network hosting the EES. The UE contains Application Client(s) and the Edge Enabler Client. The Edge Application Server(s), the Edge Enabler Server and the Edge Configuration Server may interact with the 3GPP Core Network.

## 6.2	Potential converged charging architectures for edge computing

### 6.2.1	General

The converged charging architectures for edge computing aime to support charging for the following services or capabilities:

-	Edge enabling service;

-	5GS capabilities supporting edge computing;

The architectures for converged charging for edge enabling service and 5GS capabilities supporting edge computing are depicted for the possible solution(s) in the corresponding sections in clause 7.

The architectures for converged charging for 5GS are defined in other TSs (such as TS 32.240 [2], TS 32.255 [6] and TS 32.256 [7]) and are referenced by the present document where it applies.

Details on the interfaces and functions can be found in TS 32.240 [2] for the general architecture components. Nchf is described in TS 32.290 [5].

# 7	Charging scenarios and key issues

## 7.1	5GS capability usage based charging for edge computing

### 7.1.1	Use cases

#### 7.1.1.1	Use case #1a: MNO charging subscriber for 5GS capabilities usage for accessing edge application

This use case focuses on MNO and subscriber business roles.

A UE has a subscription with the MNO which allows usage of 5GS for accessing edge applications.

5GS provides the capabilities to support edge computing, and the MNO may charge these 5GS capabilities in the following ways:

- 	Charge the individual subscribers for using the 5G capabilities supporting edge computing for accessing edge applications.

The subscriber charging could be based on data volume or in-session (PDU session) time that the UE has been connected to the edge applications, and potentially with consideration of the following factors:

-	User’s location;

-	Access RAT type;

-	Subscribed QoS information;

-	Authorized QoS information.

#### 7.1.1.2	Use case #1b: MNO charging ECSP for 5GS capabilities usage

This use case focuses on MNO and ECSP business roles.

The ECSP has an agreement to use the 5GS capabilities of the MNO.

-	Charge the ECSP for the 5GS capabilities provided for edge computing.

The inter-provider charging by MNO could be based on the total data volume transferred in the PLMN for the EDN (Edge Data Network).

#### 7.1.1.3	Use case #1c: MNO charging ASP for 5GS capabilities usage

This use case focuses on MNO and ASP business roles.

The ASP has an agreement to use the 5GS capabilities of the MNO.

-	Charge the ASP for the 5GS capabilities provided for edge computing.

The inter-provider charging by MNO could be based on the total data volume transferred in the PLMN for the edge application.

#### 7.1.1.4	Use case #1d: MNO charging subscriber for 5GS based on monitored QoS

5GS offers the capabilities of QoS control and delivery to support edge applications with sensitive performance requirements, especially on the delay (for the URLLC service).

The actual QoS delivered by the 5GS for supporting edge computing may matter to the charging of the 5GS capabilities provided to the subscribers. For example, for two subscribes who have consumed the same amount of data in the same PLMN for the same edge application, if they have been offered different QoS, they may need to be charged differently based on the monitored QoS for them respectively.

#### 7.1.1.5	Use case #1e: MNO charging ECSP/ASP for 5GS based on monitored QoS

5GS offers the capabilities of QoS control and delivery to support edge applications with sensitive performance requirements, especially on the delay (for the URLLC service).

The actual QoS delivered by the 5GS for supporting edge computing may matter to the charging of the 5GS capabilities provided to the ECSP/ASP. For example, for two ECSPs can if they have been offered different QoS, be charged differently based on the monitored QoS.

### 7.1.2	Potential charging requirements

#### 7.1.2.1	Potential requirements for subscriber based charging

REQ-CH_EC_5GS-01: The SMF should support the following extensions to the charging requirements defined in TS 32.255 [6]:

-	The SMF should support converged charging for each edge application, i.e., charging per rating group per PDU session with information about edge application.

-	The charging information of PDU session, service flow or QoS flow provided by SMF should be able to indicate or be correlated with the edge application that the user accesses to.

-	The SMF should support converged charging and charging information reporting for each edge application optionally based on the following criteria:

-	User's location;

-	Access RAT type;

-	Subscribed QoS information;

-	Authorized QoS information.

REQ-CH_EC_5GS-02:	The 5G system should support collecting charging information per UE on usage of 5GS capabilities based on edge application.

REQ-CH_EC_5GS-03: The 5G system should support converged charging and charging information reporting for each edge application optionally based on the following criteria:

-	Monitored QoS in terms of packet delay (see TS 23.501 [3], clause 5.33.3).

#### 7.1.2.2	Potential requirements for inter-provider based charging

REQ-CH_EC_5GS_SP-01: The charging mechanism for 5G system should support collecting charging information on usage of 5GS capabilities per UE EDN (edge data network).

REQ-CH_EC_5GS_SP-02: The charging mechanism for 5G system should support collecting charging information on usage of 5GS capabilities per edge application.

REQ-CH_EC_5GS_SP-03: The charging mechanism for 5GS should support converged inter-provider charging for the 5GS capabilities based on performance measurements or KPIs of average UL/DL packet delay of the UEs in 5GS accessing the edge application.

### 7.1.3	Key issues

The following key issues are identified:

-	Key Issue #1a: The charging information reported by SMF needs to contain the parameter for correlation between 5GS and edge application that the user has accessed to.

-	Key Issue #1b: The charging mechanism for 5GS (including the management system) needs to support converged inter-Provider charging for the 5G capabilities provided to support edge computing to its consumer based on the total UL/DL data volume transferred for the edge application.

-	Key Issue #1c: SMF needs to support converged charging and charging information reporting for each edge application optionally based on monitored QoS in terms of packet delay.

-	Key Issue #1d: How to support converged inter-provider charging for the 5GS capabilities based on performance measurements and/or KPIs related to average UL/DL packet delay between PSA UPF and UE.

### 7.1.4	Possible solutions

#### 7.1.4.1	Possible solutions for subscriber based charging

##### 7.1.4.1.1	Possible solutions for MNO charging subscriber for 5GS usage for accessing edge application

The possible solutions to support the potential requirements REQ-CH_EC_5GS-01, REQ-CH_EC_5GS-02 and Key Issue #1a could be the following enhancements or extensions to the PDU session charging solutions defined in TS 32.255 [6]:

Solution #1a: Reuse the “AF Charging Identifier” (see TS 32.255 [6] and TS 29.512 [8]) or “afChargId” (see TS 29.512 [8])  parameter for correlating the Charging Data Request, charging information and CHF record data with the edge application for PDU session charging.

The “AF Charging Identifier” or “afChargId” parameter is provided to SMF in the PCC rule for one or more service flows and could be contained in the “PDU Container Information” that is provided in the Charging Data Request sent by SMF and CHF record data.

Solution #1b: Reuse the “Network Slice Instance Identifier” parameter (see TS 32.255 [6]) for correlating the Charging Data Request, charging information and CHF record data with the edge application for PDU session charging.

The “Network Slice Instance Identifier” parameter is contained in the “PDU Session Charging Information” and needs to be provided in the Charging Data Request sent by SMF and CHF record data.

The assumption for this solution is that each edge application is associated with a dedicated network slice.

Solution #1c: Use the “Application Identifier”, “flowDescription” or “ethFlowDescription”  parameter (see “appId” in TS 29.512 [8]) for correlating the Charging Data Request, charging information and CHF record data with the edge application for PDU session charging.

The “Application Identifier”, “flowDescription” or “ethFlowDescription” parameter is provided to SMF in the PCC rule for one or more service flows. Including either of these parameters in the Charging Data Request sent by SMF and CHF record data would enable the CHF to correlate them with the edge application.

Solution #1d: Use the “Rating Group” parameter for correlating the Charging Data Request, charging information and CHF record data with the edge application for PDU session charging.

The “Rating Group” is provided to SMF in the PCC rule for one or more service flows.

Solution #1e: Use the “Service Identifier” parameter for correlating the Charging Data Request, charging information and CHF record data with the edge application for PDU session charging.

The “Service Identifier” is provided to SMF in the PCC rule for one or more service flows.

Solution #1f: Use the “Sponsor Identity”  parameter for correlating the Charging Data Request, charging information and CHF record data with the edge application for PDU session charging.

The “Sponsor Identity”   are provided to SMF in the PCC rule for one or more service flows.

##### 7.1.4.1.2	Possible solutions for MNO charging subscriber for 5GS usage for edge computing based on monitored QoS

The possible solutions to support the potential requirements REQ-CH_EC_5GS-03 and Key Issue #1c could be based on the 5G data connectivity converged charging architecture defined in TS 32.255 [6], with the extension of including monitored QoS result for URLLC service on Edge Computing in the usage report, as described below.

Solution #1g: Including the monitored QoS result in the usage report

Figure 7.1.4.1.2-1: Subscriber charging for 5GS usage for edge computing based on monitored QoS

The CHF of MNO is configured with quota and reporting threshold for subscriber charging for usage of 5GS capabilities supporting edge computing.

1)	UE starts using an edge application: Triggers is are according to TS 32.255 [6] and TS 32.290 [5].

2)	Charging Data Request [Initial, Quota Requested]: the SMF sends the request to the CHF to reserve a number of units if determined.

3)	Account, Rating, Reservation Control: the CHF checks if corresponding funds can be reserved on the edge application’s account balance, for each Rating Group. If the account has sufficient funds, the CHF rates the request and performs the corresponding reservation for each Rating Group.

4)	 Open CDR: based on policies, the CHF opens a CDR related to the service for edge application.

5)	Charging Data Response [Initial, Quota Granted]: the CHF grants the reserved number of units to SMF for each Rating Group.

6)	Granted Units Supervision: The SMF monitors the consumption of the granted units and the delivered QoS if the monitored QoS result for URLLC service on Edge Computing is requested for charging.

NOTE: how to request the monitored QoS results for charging is not addressed in the present document.

7)	UE edge application usage ongoing: the SMF continues to deliver the service.

8)	Quota management Trigger: A Trigger associated to Quota management is met. Units determination is performed when applicable.

9)	Charging Data Request [Update, Quota Requested, Usage Reporting]: the SMF sends the request to the CHF, to be granted with more unit for the service to continue, and also for reporting the used units. For the usage reporting, the monitored QoS result (e2e packet delay in 5GS) is included.

10)	Account, Rating, Reservation Control: same as step 4, with the option to also deduct the funds corresponding to the usage on the account balance.

11)	Update CDR: based on policies, the CHF updates the CDR with charging data related to the service.

12)	Charging Data Response [Update, Quota Granted]: The CHF grants quota to SMF for the service, with the reserved number of units for each Rating Group.

13)	Service delivery ongoing: the  NF (CTF) continues to deliver the service.

14)	UE stops using the edge application: the SMF is requested to end the service delivery and does this.

15)	Charging Data Request [Termination]: the SMF sends the request to the CHF, for charging data related to the service termination with the final consumed units for each Rating Group.

16)	Account, Rating Control: the CHF performs the service termination process which involve using the reported charging data to rate the usage and deduct the funds corresponding to the usage on the account balance.

17)	 Close CDR: based on policies, the CHF closes the CDR with charging data related to the service termination and the last reported units for each Rating Group.

18)	Charging Data Response [Termination]: The CHF informs the SMF on the result of the request.

#### 7.1.4.2	Possible solutions for inter-provider based charging

##### 7.1.4.2.1	Possible solutions for inter-provider charging for 5GS capabilities usage

The possible solutions to support the inter-provider charging related potential requirements REQ-CH_EC_5GS_SP-01, REQ-CH_EC_5GS_SP-02 and Key Issue #1b are based on the 5G data connectivity converged charging architecture defined in TS 32.255 [6], with the extension of CHF to support inter-provider charging for 5GS supporting edge computing per edge application and/or per EDN.

Figure 7.1.4.2-1: 5G data connectivity converged charging architecture defined in TS 32.255 [6]

Solution #1h: Aggregation of SMF reported usage by CHF

In this solution, the total data volume transferred for the edge application in the 5GS is obtained by CHF from each individual UE usage reported by SMF.

Figure 7.1.4.2-2: 5GS Inter-provider charging for edge computing (aggregation of SMF reported usage)

The CHF of MNO is configured with quota and reporting threshold for 5GS inter-provider charging.

1)	UE starts using an edge application: Triggers is are according to TS 32.255 [6] and TS 32.290 [5].

2)	Charging Data Request [Initial, Quota Requested]: the SMF sends the request to the CHF to reserve a number of units if determined in step 2.

3)	Account, Rating, Reservation Control: the CHF rates the request, checks if corresponding funds can be reserved on the edge application’s account balance. If the account has sufficient funds, the CHF performs the corresponding reservation.

4)	 Open CDR: based on policies, the CHF opens a CDR related to the service for edge application.

5)	Charging Data Response [Initial, Quota Granted]: the CHF grants the reserved number of units to SMF.

6)	Granted Units Supervision: The SMF monitors the consumption of the granted units.

7)	UE edge application usage ongoing: the SMF continues to deliver the service.

8)	Quota management Trigger: A Trigger associated to Quota management is met. Units determination is performed when applicable.

9)	Charging Data Request [Update, Quota Requested]: the SMF sends the request to the CHF, to be granted with more unit for the service to continue, and also for reporting the used units.

10)	Account, Rating, Reservation Control: same as step 4, with the option to also deduct the funds corresponding to the usage on the account balance.

11)	Update CDR: based on policies, the CHF updates the CDR with charging data related to the service.

12)	Charging Data Response [Update, Quota Granted]: The CHF grants quota to SMF for the service, with the reserved number of units.

13)	Service delivery ongoing: the  NF (CTF) continues to deliver the service.

14)	UE stops using the edge application: the SMF is requested to end the service delivery and does this.

15)	Charging Data Request [Termination]: the SMF sends the request to the CHF, for charging data related to the service termination with the final consumed units.

16)	Account, Rating Control: the CHF performs the service termination process which involve using the reported charging data to rate the usage and deduct the funds corresponding to the usage on the account balance.

17)	 Close CDR: based on policies, the CHF closes the CDR with charging data related to the service termination and the last reported units.

18)	Charging Data Response [Termination]: The CHF informs the SMF on the result of the request.

The CHF aggregates the reported units from SMF(s) at a pre-configured interval for each edge application for all UEs, and reports the aggregated usage to Billing domain via CGF.

Solution #1i: Aggregation of SMF reported usage by Billing domain

In this solution, the total data volume transferred for the edge application in 5GS is obtained by CHF as in solution #2a but only reported per UE, it would be the billing domain that then aggregates the usage based on the received CDRs and edge application indication in these CDRs. This also means that the account is on UE and not on edge application.

##### 7.1.4.2.2	Possible solutions for inter-provider charging for 5GS capabilities usage based on monitored QoS

Solution #1j: Aggregation of SMF reported usage based on monitored QoS by CHF

In this solution, the charging mechanism between SMF and CHF for 5GS capabilities usage supporting edge computing based on monitored QoS is the same as Solution #1g, in addition, the CHF aggregates the reported units with monitored QoS results from SMF(s) at a pre-configured interval for each edge application and/or EDN, and reports the aggregated usage to Billing domain via CGF.

Solution #1k: Aggregation of SMF reported usage based on monitored QoS by Billing domain

In this solution, the charging mechanism between SMF and CHF for 5GS capabilities usage supporting edge computing based on monitored QoS is the same as Solution #1g, and CHF reports the usage with monitored QoS results in the CDRs per UE. It would be the billing domain that then aggregates the usage based on the received CDRs and edge application indication in these CDRs.

#### 7.1.4.3	Possible solutions for edge application access charging via EAS

The possible solutions to support the subscriber based charging related potential requirements REQ-CH_EC_5GS-02, and Key Issue #1b could be the following architecture.

The EAS is implemented by the MNO.

Figure 7.1.4.3-1: EAS based solution

The solution with CTF embedded in the EAS is illustrated in Figure 7.1.4.3-2.

Figure 7.1.4.3-2: duration based charging for 5GS capabilities (with EAS supporting CTF)

The CHF of MNO is configured with quota and reporting threshold for duration charging.

The operations is the same with the 7.1.4.2-2, with EAS instead of the SMF.

### 7.1.5	Evaluation

#### 7.1.5.1	Evaluation of possible solutions for MNO charging subscriber for 5GS usage for accessing edge application

This clause evaluates the following solutions for correlating the 5GS charging related messages with edge application:

Solution #1a:

This solution reuses the “AF Charging Identifier” (see TS 32.255 [6] and TS 29.512 [8]) or “afChargId” (see TS 29.512 [8])  parameter.

The “AF Charging Identifier” or “afChargId” parameter initially comes from AF, and 3GPP cannot control and guarantee its uniqueness and its relationship with edge application. Therefore, this solution needs offline coordination between PLMN operator and ECSP/ASP to ensure the correctness of the correlation between this parameter and edge application.

Solution #1b:

This solution reuses the “Network Slice Instance Identifier” parameter (see TS 32.255 [6]).

This solution works on an assumption that each edge application is associated with a dedicated network slice. It makes impossible for a PLMN to support multiple edge applications by one network slice, which in some cases may be needed. Therefore, this solution is restrictive.

Solution #1c:

This solution uses the “Application Identifier”, “flowDescription” or “ethFlowDescription”  parameter (see “appId” in TS 29.512 [8]).

These parameters reference to the application or service flow detection filter configured at the UPF, and can be mapped to edge application without restrictive assumption.

Solution #1d:

This solution uses the “Rating Group” parameter.

This solution works on an assumption that each edge application is associated with a dedicated Rating Group. It makes impossible for a PLMN to support multiple applications by one Rating Group, which in some cases may be needed. Therefore, this solution is restrictive.

Solution #1e:

This solution uses the “Service Identifier” parameter.

This solution requires that the “Service Identifier” parameter is able to identify the instance of the edge application, but not only the identity of the service (e.g., edge computing service).

Solution #1f: Use the “Sponsor Identity”  parameter.

This solution works in the scenario that the edge computing related connectivity and usage are sponsored (by a service provider, e.g., ASP). However, this is not the only scenario for supporting edge computing in 5GS, and there could be also the non-sponsored case. Another perspective is that, to make this solution work, this parameter needs to be extended to be able to identify the application but not only the service provider.

#### 7.1.5.2	Evaluation of possible solutions for MNO charging subscriber for 5GS usage for edge computing based on monitored QoS

This clause evaluates the following solution for MNO charging subscriber for 5GS usage for edge computing based on monitored QoS:

Solution #1g: Including the monitored QoS result in the usage report.

This solution enables the SMF to monitor the QoS for charging purpose by the PCC rule or an explicit request from CHF, and include the monitored QoS result in the usage report. This solution is feasible for charging subscriber for 5GS usage for edge computing based on monitored QoS.

#### 7.1.5.3	Evaluation of possible solutions for inter-provider charging for 5GS capabilities usage

This clause evaluates the following solutions for inter-provider charging for 5GS capabilities usage:

Solution #1h: Aggregation of SMF reported usage by CHF.

This solution enables the CHF to collect the 5GS usage supporting edge computing per UE from SMF and aggregate the usage for each edge application. This solution can support PEC of the inter-provider charging for 5GS capabilities usage supporting each edge application, and save the bandwidth from CHF to Billing domain for transferring the usage for every UE.

Solution #1i: Aggregation of SMF reported usage by Billing domain.

This solution requires the CHF to report the 5GS usage supporting each edge application per UE, which can be supported by existing 5GS usage charging solutions.

#### 7.1.5.4	Evaluation of possible solutions for inter-provider charging for 5GS capabilities usage based on monitored QoS

This clause evaluates the following solutions for inter-provider charging for 5GS capabilities usage:

Solution #1j: Aggregation of SMF reported usage based on monitored QoS by CHF.

This solution enables the CHF to collect the 5GS usage supporting edge computing based on the monitored QoS per UE from SMF and aggregate the usage for each edge application. This solution can support PEC of the inter-provider charging for 5GS capabilities usage based on monitored QoS supporting each edge application, and save the bandwidth from CHF to Billing domain for transferring the usage for every UE.

Solution #1k: Aggregation of SMF reported usage based on monitored QoS by Billing domain.

This solution requires the CHF to report the 5GS usage supporting each edge application based on monitored QoS per UE, which has no additional changes on charging interfaces to the solution #1g for MNO charging subscriber for 5GS usage based on monitored QoS.

### 7.1.6	Conclusion

#### 7.1.6.1	Conclusion of possible solutions for MNO charging subscriber for 5GS usage for accessing edge application

Based on the evaluation in clause 7.1.5.1, the Solution #1c is the most feasible and preferrable solution for MNO charging subscriber for 5GS usage for accessing edge application, which is supported by existing charging capability.

#### 7.1.6.2	Conclusion of possible solutions for MNO charging subscriber for 5GS usage for edge computing based on monitored QoS

Based on the evaluation in clause 7.1.5.2, the Solution #1g is feasible for MNO charging subscriber for 5GS usage for edge computing based on monitored QoS.

#### 7.1.6.3	Conclusion of possible solutions for inter-provider charging for 5GS capabilities usage

Based on the evaluation in clause 7.1.5.3, the Solution #1i, which can be supported by existing 5GS usage charging solutions, is the most feasible and preferrable solution for inter-provider charging for 5GS capabilities usage.

#### 7.1.6.4	Conclusion of possible solutions for inter-provider charging for 5GS capabilities usage based on monitored QoS

Based on the evaluation in clause 7.1.5.4, the Solution #1k is the most feasible and preferrable solution for inter-provider charging for 5GS capabilities usage based on monitored QoS.

## 7.2	Edge enabling infrastructure resource charging

### 7.2.1	Use cases

#### 7.2.1.1	Use case #2a: Edge enabling infrastructure resource usage charging

ECSP provides the following infrastructure resources to ASP to enable the EAS to be running in the EDN:

-	EDN environment and infrastructure resources (e.g., virtual CPU, virtual memory, virtual disk and virtual network resources);

ECSP may charge the ASP for the infrastructure resources usage to enable and support the EAS(s). This inter-provider charging may be based on the infrastructure resources actually used by the EAS(s).

#### 7.2.1.2	Use case #2b: Edge enabling infrastructure resource performance charging

ECSP provides the EDN environment and infrastructure resources and capabilities (e.g. virtual CPU, virtual memory, virtual disk and virtual network resources) to ASP to enable the EAS to be running in the EDN. To do this it may have performance assurance of EES and ECS.

ECSP may charge the ASP based on the performance of the infrastructure resources provided to enable and support the EAS(s).

### 7.2.2	Potential charging requirements

REQ-CH_EC_EEIR-01: The charging mechanism for ECSP should support converged charging based on the usage of infrastructure resources supporting the EAS(s).

REQ-CH_EC_EEIR-02: The charging mechanism for ECSP should support collecting charging information related to usage of infrastructure resource supporting each EAS.

REQ-CH_EC_EEIR03: The charging mechanism for 5GS (including the management system) should support converged charging for the edge enabling infrastructure resource  based on performance.

### 7.2.3	Key issues

The following key issues are identified:

-	Key Issue #2a: How does the charging mechanism for ECSP collect the information about the charged infrastructure resources as described in REQ-CH_EC_EEIR-02.

-	Key Issue #2b: What infrastructure resources should be charged as described in REQ-CH_EC_EEIR-01 and REQ-CH_EC_EEIR-02.

-	Key Issue #2c: How to support the charging for edge enabling infrastructure resource based on performance as described in REQ-CH_EC_EEIR-03.

### 7.2.4	Possible solutions

#### 7.2.4.1	Potential charging architectures

The possible solutions for charging for edge enabling infrastructure resources are based on the following potential converged charging architectures. In these architectures, it is assumed that all the entities are owned by the same enterprise.

Figure 7.2.4.1-1: Converged charging architecture for edge enabling infrastructure resource with MnS producer supporting CTF

Figure 7.2.4.1-2: Converged charging architecture for edge enabling infrastructure resource with MnS producer

NOTE: the MnSs for managing edge computing are described in TR 28.814 [12].

#### 7.2.4.2	Possible solutions for edge enabling infrastructure resource usage charging

The charging for edge enabling infrastructure resources could be supported with CTF embedded in the MnS producer or with CEF to enable the charging for MnS producer.

The solutions are only applicable to the case that the EAS is implemented as VNF.

Solution #2a: Edge enabling infrastructure resource usage charging - CTF embedded in the MnS producer

The solution with CTF embedded in the MnS producer is illustrated in Figure 7.2.4.2-1.

Figure 7.2.4.2-1: Edge enabling infrastructure resource usage (with MnS producer supporting CTF)

This solution uses performance data to report the usage of edge enabling infrastructure resources.

1)	Generate performance data for resource usage for EAS: performance assurance MnS producer generates performance data (measurements or KPI) about usage of edge enabling infrastructure resource supporting the EAS based on the mapping of VR usage measurements received from ETSI NFV MANO system as specified in TS 28.522 [18]. The performance data about edge enabling infrastructure resource could be related to data volume transferred for the EAS, virtual CPU usage of the EAS, virtual memory usage of the EAS, virtual disk usage of the EAS, or the virtual storage of the EAS.

1ch-a)	Charging Data Request [Event]: performance assurance MnS Producer generates charging data related to the collected resource usage and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

1ch-b)	Create CDR: the CHF stores received information and creates a CDR related to the event.

1ch-c)	Charging Data Response [Event]: The CHF informs the performance assurance MnS Producer on the result of the request.

2)	Report the CDR to BD: The CHF reports the CDR to BD (via CGF).

Solution #2b: Edge enabling infrastructure resource usage charging – with CEF enabling charging for MnS producer

The solution with CEF enabling charging for MnS producer is illustrated in Figure 7.2.4.2-2.

Figure 7.2.4.2-2: Edge enabling infrastructure resource usage (with CEF enabling charging for MnS producer)

This solution uses performance data to report the usage of edge enabling infrastructure resources.

1)	Create measurement job: The CEF creates measurement job to collect the performance data related to VR resource usage for EAS to performance assurance MnS producer (see TS 28.550 [15]).

1a)	Subscribe to performance data file notifications: If file reporting method is chosen for the measurement job, the CEF subscribes to the performance data file notifications.

2)	Generate performance data for resource usage for EAS: performance assurance MnS producer generates the performance data (measurements or KPI) about usage of edge enabling infrastructure resource supporting the EAS based on the mapping of the VR usage measurements received from ETSI NFV MANO system as specified in TS 28.522 [18], the MnS producer reports the performance data to the CEF. The performance data about edge enabling infrastructure resource could be related to data volume transferred for the EAS, virtual CPU usage of the EAS, virtual memory usage of the EAS, virtual disk usage of the EAS, or the virtual storage of the EAS.

3)	Performance data report to CEF: the performance assurance MnS producer reports the performance data to the CEF according the reporting method selected by the CEF for the measurement job.

If the file data reporting method is selected:

3a) 	The performance data are reported by a notifyFileReady notification (see TS 28.532 [17]);

3b) 	CEF fetches the file containing the performance data.

If the streaming data reporting method is selected:

3c) and 3d)	 The performance assurance MnS producer establishes the streaming connection with the CEF if the connection has not been established (see TS 28.532 [17]);

3e) 	The performance data are reported by the reportStreamData operation (see TS 28.532 [17]).

3ch-a)		Charging Data Request [Event]: The CEF generates charging data related to the collected resource usage and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

3ch-b)		Create CDR: the CHF stores received information and creates a CDR related to the event.

3ch-c)		Charging Data Response [Event]: The CHF informs the CEF on the result of the request.

4)	Report the CDR to BD: The CHF reports the CDR to BD (via CGF).

For both solutions, the reported usage may be used by BSS for interactions with OAM. For example if the limit of the authorized usage is reached, the BSS may request OAM to disable the operational state of the EAS (e.g., by consuming the provisioning MnS via modifyMOIAttributes operation) and OAM may request ETSI NFV MANO to stop the corresponding VNF/VNFC instances (e.g., via OperateVnfRequest operation, see ETSI GS NFV-IFA 008 [19]). The interactions between BSS and OAM are out of scope of the present document.

#### 7.2.4.3	Possible solutions for edge enabling infrastructure resource performance charging

The charging for edge computing performance could be supported with CTF embedded in the MnS producer or with CEF to enable the charging for MnS producer.

Solution #2c: Edge enabling infrastructure resource performance charging – CTF embedded in the MnS producer.

The procedure is the same as the solution #2a as illustrated in Figure 7.2.4.2-1, except the performance measurements in this solution are related to edge enabling infrastructure resource performance.

Solution #2d: Edge enabling infrastructure resource performance charging – with CEF enabling charging for MnS producer.

The procedure is the same as the solution #2b as illustrated in Figure 7.2.4.2-2, except the performance measurements in this solution are related to edge enabling infrastructure resource performance.

### 7.2.5	Evaluation

Solution #2a and Solution #2b support edge enabling infrastructure resource usage charging when EAS is implemented as VNF for two scenarios respectively - CTF embedded in the MnS producer and with CEF enabling charging for MnS producer.

Since there is no need for quota management (requesting and granting units), both solutions are equally appropriate. The solution #2b with CEF enabling charging for MnS producer has one advantage and that is that it doesn’t require the MnS Producer to implement more interfaces like Nchf or Nnrf.

Solution #2c and Solution #2d support edge enabling infrastructure resource performance charging when EAS is implemented as VNF for two scenarios respectively - CTF embedded in the MnS producer and with CEF enabling charging for MnS producer.

Since there is no need for quota management (requesting and granting units), both solutions are equally appropriate. The solution #2d with CEF enabling charging for MnS producer has one advantage and that is that it doesn’t require the MnS Producer to implement more interfaces like Nchf or Nnrf.

### 7.2.6	Conclusion

Based on the evaluation in clause 7.2.5,

-  	Solution #2b is the feasible and preferable solution for edge enabling infrastructure resource usage charging when EAS is implemented as VNF; and

-  	Solution #2d is the feasible and preferable solution for edge enabling infrastructure resource performance charging when EAS is implemented as VNF.

## 7.3	Charging for edge application server deployment

### 7.3.1	Use case

#### 7.3.1.1	Use case #3a: Edge enabling application server deployment charging

ECSP provides the EDN environment and infrastructure resources and capabilities (e.g. virtual CPU, memory, disk and network resources) to ASP to enable the EAS to be running in the EDN.

ECSP provides edge enabling services to ASP to enable the EAS to be running in the EDN:

-	EAS deployment (e.g., EAS instantiation, update, and termination);

ECSP may charge the ASP for the infrastructure resources and services provided to enable and support the EAS(s). This inter-provider charging may be based on the infrastructure resources allocated to the EAS(s).

### 7.3.2	Potential charging requirements

REQ-CH_EC_EA-01: The charging mechanism for ECSP should support converged charging for the following EAS deployment related procedures:

-	EAS instantiation;

-	EAS update;

-	EAS termination.

### 7.3.3	Key issues

The following key issues are identified:

-	Key Issue #3a: How does the charging mechanism for ECSP support converged charging for the EAS deployment related procedures as depicted in REQ-CH_EC_EA-01.

### 7.3.4	Possible solutions

#### 7.3.4.1	Potential charging architectures

The possible solutions for charging for edge enabling infrastructure resources and services are based on the following potential converged charging architectures. In these architectures, it is assumed that the MnS producer, CHF, CEF and Billing Domain are owned by the same ECSP.

Figure 7.3.4.1-1: Converged charging architecture for edge application server deployment with MnS producer supporting CTF

Figure 7.3.4.1-2: Converged charging architecture for edge application server deployment with MnS producer not supporting CTF

NOTE: the MnSs for managing edge computing are described in TR 28.814 [12].

#### 7.3.4.2	Possible solutions for EAS deployment charging

The charging for EAS deployment could be supported with CTF embedded in the MnS producer or with CEF to enable the charging for MnS producer.

Solution #3a: EAS deployment charging - CTF embedded in the MnS producer

The solution with CTF embedded in the MnS producer is illustrated in Figure 7.3.4.2-1.

Figure 7.3.4.2-1: Charging for EAS deployment (with MnS producer supporting CTF) - PEC

1)	Chargeable event trigger configuration: The MnS Producer (CTF) is configured for which events there is a need to have charging interaction.

2)	EAS deployment request: The MnS consumer sends the EAS deployment request (i.e., instantiation, update, or termination) to the MnS producer.

3)	EAS deployment process: the MnS producer processes and executes the EAS deployment request according to the request.

4)	EAS deployment response: The MnS consumer sends the EAS deployment request to the MnS producer.

4ch-a)	Charging Data Request [Event]: The MnS Producer generates charging data related to the EAS deployment request and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

4ch-b)	Create CDR: the CHF stores received information and creates a CDR related to the event.

4ch-c)	Charging Data Response [Event]: The CHF informs the MnS Producer on the result of the request.

Solution #3b: EAS deployment charging - with CEF enabling charging for MnS producer

The solution with CEF enabling charging for MnS producer is illustrated in Figure 7.3.4.2 -2.

Figure 7.3.4.2-2: Charging for EAS deployment (with CEF enabling charging for MnS producer) - PEC

1)	CEF consumes the MnS: The CEF consumes the MnS (see TS 28.622 [16]) and subscribes to notifications for the EAS deployment reporting.

2)	EAS deployment request: The MnS consumer sends the EAS deployment request (i.e., instantiation, update, or termination) to the MnS producer.

3)	EAS deployment process: the MnS producer processes and executes the EAS deployment request according to the request.

4)	EAS deployment response: The MnS producer sends the EAS deployment request to the MnS consumer.

5)	EAS deployment notification: The MnS producer sends the EAS deployment notification to the CEF. That means the CEF needs to subscribe to the EAS deployment notification from the MnS producer beforehand.

5ch-a)	Charging Data Request [Event]: The CEF generates charging data related to the EAS deployment notification and sends the charging data request.

5ch-b)	Create CDR: the CHF stores received information and creates a CDR related to the event.

5ch-c)	Charging Data Response [Event]: The CHF informs the CEF on the result of the request.

### 7.3.5	Evaluation

Solution #3a and Solution #3b support EAS deployment charging for two scenarios respectively - CTF embedded in the MnS producer and with CEF enabling charging for MnS producer.

Since there is no need for quota management (requesting and granting units), both solutions are equally appropriate. The solution #3b with CEF enabling charging for MnS producer has one advantage and that is that it doesn’t require the MnS Producer to implement more interfaces like Nchf or Nnrf.

### 7.3.6	Conclusion

Based on the evaluation in clause 7.3.5, the Solution #3b is the feasible and preferable solution for EAS deployment charging.

## 7.4	Charging for edge enabling services usage

### 7.4.1	Use case

#### 7.4.1.1	Use case #4a: ECSP charging ASP for indirect 3GPP Core network capabilities usage

ECSP provides edge enabling services to ASP to enable the EAS to be running in the EDN. Some edge enabling services may be provided to ASP indirectly via 3GPP Core network capabilities. The edge enabling services include following 3GPP Core network capabilities exposed to the EAS as described in TS 23.558 [4]:

-	Obtaining UE location (see TS 23.558 [4]);

-	ACR management events subscription (see TS 23.558 [4]);

-	Session with QoS (see TS 23.558 [4]).

#### 7.4.1.2	Use case #4b: ECSP charging ASP for edge enabling services provided by ECSP

ECSP provides edge enabling services to ASP to enable the EAS to be running in the EDN. Some edge enabling services are provided directly by ECSP. The edge enabling services include following EES capabilities exposed to the EAS as described in TS 23.558 [4]:

-	EAS registration (see TS 23.558 [4]);

-	EAS discovery (see TS 23.558 [4]);

-	Support to Service Continuity (see TS 23.558 [4]);

-	Application Client information subscription/notification (see TS 23.558 [4]).

### 7.4.2	Potential charging requirements

REQ-CH_EC_ES-01: The charging mechanism for ECSP should support converged charging for the following EAS signalling procedures:

-	EAS registration;

-	EAS discovery;

-	EAS application context relocation,

REQ-CH_EC_ES-02: The charging mechanism for ECSP should support converged charging for the following EAS service consumption:

-	Support to Service Continuity;

-	Obtaining UE location;

-	ACR management events subscription;

-	Application Client information subscription/notification;

-	Session with QoS.

### 7.4.3	Key issues

The following key issues are identified:

-	Key Issue #4a: How does the charging mechanism for ECSP support converged charging for the EAS signalling procedures as depicted in REQ-CH_EC_ES-01.

-	Key Issue #4b: How does the charging mechanism for ECSP support converged charging for the EAS service consumption as depicted in REQ-CH_EC_ES-02.

### 7.4.4	Possible solutions

#### 7.4.4.1	Potential charging architectures

The solutions for edge enabling services usage charging are based on the following potential converged charging architectures. In these architectures, it is assumed that the EES, CHF and Billing Domain are owned by the same ECSP.

Figure 7.4.4.1-1: Converged charging architecture for edge enabling infrastructure resources and services with EES supporting CTF

#### 7.4.4.2	Possible solutions for EAS registration charging

Solution #4a: The solution for EAS registration charging based on IEC is illustrated in the Figure 7.4.4.2-1.

Figure 7.4.4.2-1: Charging for EAS registration - IEC

1.	The EAS sends the Edge Application Server Registration request (see TS 23.558 [4]) to the EES.

1ch-a)	Charging Data Request [Event]: The CEF generates charging data related to the Edge Application Server Registration request and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

1ch-b)	Create CDR: the CHF stores received information and creates a CDR related to the event.

1ch-c)	Charging Data Response [Event]: The CHF informs the CEF on the result of the request.

2.	The EES sends Edge Application Server Registration response (see TS 23.558 [4]) to the EAS.

Solution #4b: The solution for EAS registration charging based on PEC is illustrated in the Figure 7.4.4.2-2.

Figure 7.4.4.2-2: Charging for EAS registration - PEC

1.	The EAS sends the Edge Application Server Registration request (see TS 23.558 [4]) to the EES.

2.	The EES sends Edge Application Server Registration response (see TS 23.558 [4]) to the EAS.

2ch-a)	Charging Data Request [Event]: The CEF generates charging data related to the Edge Application Server Registration and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

2ch-b)	Create CDR: the CHF stores received information and creates a CDR related to the event.

2ch-c)	Charging Data Response [Event]: The CHF informs the CEF on the result of the request.

#### 7.4.4.3	Possible solutions for EAS discovery charging

Solution #4c: The solution for EAS discovery charging based on IEC is illustrated in the Figure 7.4.4.3-1.

Figure 7.4.4.3-1: Charging for EAS discovery - IEC

1.	The EEC sends the Edge Application Server discovery request (see TS 23.558 [4]) to the EES.

1ch-a)	Charging Data Request [Event]: The CEF generates charging data related to the Edge Application Server discovery request and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

1ch-b)	Create CDR: the CHF stores received information and creates a CDR related to the event.

1ch-c)	Charging Data Response [Event]: The CHF informs the CEF on the result of the request.

2.	The EES sends Edge Application Server discovery response (see TS 23.558 [4]) to the EEC.

Solution #4d: The solution for EAS discovery charging based on PEC is illustrated in the Figure 7.4.4.3-2.

Figure 7.4.4.3-2: Charging for EAS discovery - PEC

1.	The EEC sends the Edge Application Server discovery request (see TS 23.558 [4]) to the EES.

2.	The EES sends Edge Application Server discovery response (see TS 23.558 [4]) to the EEC.

2ch-a)	Charging Data Request [Event]: The CEF generates charging data related to the Edge Application Server discovery and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

2ch-b)	Create CDR: the CHF stores received information and creates a CDR related to the event.

2ch-c)	Charging Data Response [Event]: The CHF informs the CEF on the result of the request.

#### 7.4.4.4	Possible solutions for EAS application context transfer charging

Solution #4e: The solution for EAS application context transfer charging based on IEC is illustrated in the Figure 7.4.4.4-1.

Figure 7.4.4.4-1: Charging for EAS application context transfer - IEC

1.	The EEC sends the Application Context Relocation request (see TS 23.558 [4]) to the S-EES.

1ch-a)	Charging Data Request [Event]: The CEF generates charging data related to the Application Context Relocation request and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

1ch-b)	Create CDR: the CHF stores received information and creates a CDR related to the event.

1ch-c)	Charging Data Response [Event]: The CHF informs the CEF on the result of the request.

2.	The S-EES sends Application Context Relocation Notify (see TS 23.558 [4]) to the S-EAS.

3.	The S-EAS sends Application Context Relocation Complete (see TS 23.558 [4]) to the S-EES.

4.	The S-EES sends Application Context Relocation Complete (see TS 23.558 [4]) to the EEC.

Solution #4f: The solution for EAS application context transfer charging based on PEC is illustrated in the Figure 7.4.4.4-2.

Figure 7.4.4.4-2: Charging for EAS application context transfer - PEC

1.	The EEC sends the Application Context Relocation request (see TS 23.558 [4]) to the S-EES.

2.	The S-EES sends Application Context Relocation Notify (see TS 23.558 [4]) to the S-EAS.

3.	The S-EAS sends Application Context Relocation Complete (see TS 23.558 [4]) to the S-EES.

3ch-a)	Charging Data Request [Event]: The CEF generates charging data related to the Application Context Relocation and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

3ch-b)	Create CDR: the CHF stores received information and creates a CDR related to the event.

3ch-c)	Charging Data Response [Event]: The CHF informs the CEF on the result of the request.

4.	The S-EES sends Application Context Relocation Complete (see TS 23.558 [4]) to the EEC.

Solution #4g: The solution for EAS application context transfer charging based on ECUR is illustrated in the Figure 7.4.4.4-3.

Figure 7.4.4.4-3: Charging for EAS application context transfer - ECUR

1.	The EEC sends the Application Context Relocation request (see TS 23.558 [4]) to the S-EES.

1ch-a)	Charging Data Request (initial, quota request): The S-EES generates charging data related to the Application Context Relocation request and sends the charging data request to the CHF for quota request and CDR opening purpose.

1ch-b)	Create CDR: the CHF stores received information, conducts the account, rating and reservation control and opens a CDR related to the Application Context Relocation procedure.

1ch-c)	Charging Data Response (initial, quota granted): The CHF informs the S-EES on the result of the request.

2.	If the quota request was granted, the S-EES sends Application Context Relocation Notify (see TS 23.558 [4]) to the S-EAS.

3.	The S-EAS sends Application Context Relocation Complete (see TS 23.558 [4]) to the S-EES.

4.	The S-EES sends Application Context Relocation Complete (see TS 23.558 [4]) to the EEC.

4ch-a)	Charging Data Request (termination, used quota): The S-EES generates charging data related to the Application Context Relocation Complete and sends the charging data request to the CHF for CDR closing purpose.

4ch-b)	Close CDR: the CHF stores received information conducts the account and rating control and closes the CDR related to the Application Context Relocation procedure.

4ch-c)	Charging Data Request (termination): The CHF informs the S-EES on the result of the request.

#### 7.4.4.5	Possible solutions for UE location obtaining charging

##### 7.4.4.5.1	One-time UE location request

Solution #4h: The solution for one-time UE location request charging based on IEC is illustrated in the Figure 7.4.4.5.1-1.

Figure 7.4.4.5.1-1: Charging for one-time UE location request - IEC

1.	The EAS sends the UE location request (see TS 23.558 [4]) to the EES.

1ch-a)	Charging Data Request [Event]: The EES generates charging data related to UE location request and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

1ch-b)	Create CDR: the CHF stores received information and creates a CDR related to the event.

1ch-c)	Charging Data Response [Event]: The CHF informs the EES on the result of the request.

2.	The EES checks the UE location with 3GPP network (see TS 23.558 [4]).

3.	The EES sends the UE location response (see TS 23.558 [4]) to the EAS.

Solution #4i: The solution for one-time UE location request charging based on PEC is illustrated in the Figure 7.4.4.5.1-2.

Figure 7.4.4.5.1-2: Charging for one-time UE location request - PEC

1.	The EAS sends the UE location request (see TS 23.558 [4]) to the EES.

2.	The EES checks the UE location with 3GPP network (see TS 23.558 [4]).

3.	The EES sends the UE location response (see TS 23.558 [4]) to the EAS.

3ch-a)	Charging Data Request [Event]: The EES generates charging data related to UE location request and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

3ch-b)	Create CDR: the CHF stores received information and creates a CDR related to the event.

3ch-c)	Charging Data Response [Event]: The CHF informs the EES on the result of the request.

##### 7.4.4.5.2	UE location subscription and notification

Solution #4j: The solution for UE location subscription charging based on IEC is illustrated in the Figure 7.4.4.5.2-1.

Figure 7.4.4.5.2-1: Charging for UE location subscription - IEC

1.	The EAS sends the UE location subscribe request (see TS 23.558 [4]) to the EES.

1ch-a)	Charging Data Request [Event]: The EES generates charging data related to UE location subscribe request and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

1ch-b)	Create CDR: the CHF stores received information and creates a CDR related to the event.

1ch-c)	Charging Data Response [Event]: The CHF informs the EES on the result of the request.

2.	The EES subscribes to UE location from 3GPP Core Network (see TS 23.558 [4]).

3.	The EES may subscribe to UE expected behaviour analytics (UE mobility) from 3GPP Core Network (see TS 23.558 [4]).

4.	The EES sends the UE location subscribe response (see TS 23.558 [4]) to the EAS.

Solution #4k: The solution for UE location subscription charging based on PEC is illustrated in the Figure 7.4.4.5.2-2.

Figure 7.4.4.5.2-2: Charging for UE location subscription - PEC

1.	The EAS sends the UE location subscribe request (see TS 23.558 [4]) to the EES.

2.	The EES subscribes to UE location from 3GPP Core Network (see TS 23.558 [4]).

3.	The EES may subscribe to UE expected behaviour analytics (UE mobility) from 3GPP Core Network (see TS 23.558 [4]).

4.	The EES sends the UE location subscribe response (see TS 23.558 [4]) to the EAS.

4ch-a)	Charging Data Request [Event]: The EES generates charging data related to UE location subscription and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

4ch-b)	Create CDR: the CHF stores received information and creates a CDR related to the event.

4ch-c)	Charging Data Response [Event]: The CHF informs the EES on the result of the request.

Solution #4l: The solution for UE location notification charging based on PEC is illustrated in the Figure 7.4.4.5.2-3.

Figure 7.4.4.5.2-3: Charging for UE location notification - PEC

1.	The EES detects the UE location (see TS 23.558 [4]).

2.	The EES sends the UE location notification (see TS 23.558 [4]) to the EAS.

2ch-a)	Charging Data Request [Event]: The EES generates charging data related to UE location notification and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

2ch-b)	Create CDR: the CHF stores received information and creates a CDR related to the event.

2ch-c)	Charging Data Response [Event]: The CHF informs the EES on the result of the request.

#### 7.4.4.6	Possible solutions for Application Client information subscription and notification charging

Solution #4m: The solution for Application Client (AC) information subscription charging based on IEC is illustrated in the Figure 7.4.4.6-1.

Figure 7.4.4.6-1: Charging for AC information subscription - IEC

1.	The EAS sends the AC information subscription request (see TS 23.558 [4]) to the EES.

1ch-a)	Charging Data Request [Event]: The EES generates charging data related to AC information subscription request and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

1ch-b)	Create CDR: the CHF stores received information and creates a CDR related to the event.

1ch-c)	Charging Data Response [Event]: The CHF informs the EES on the result of the request.

2.	The EES processes the request (see TS 23.558 [4]).

3.	The EES sends the AC information subscription response to EAS (see TS 23.558 [4]).

Solution #4n: The solution for AC information subscription charging based on PEC is illustrated in the Figure 7.4.4.6-2.

Figure 7.4.4.6-2: Charging for AC information subscription - PEC

1.	The EAS sends the AC information subscription 	request (see TS 23.558 [4]) to the EES.

2.	The EES processes the request (see TS 23.558 [4]).

3.	The EES sends the AC information subscription response to EAS (see TS 23.558 [4]).

3ch-a)	Charging Data Request [Event]: The EES generates charging data related to AC information subscription and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

3ch-b)	Create CDR: the CHF stores received information and creates a CDR related to the event.

3ch-c)	Charging Data Response [Event]: The CHF informs the EES on the result of the request.

Solution #4o: The solution for AC information notification charging based on PEC is illustrated in the Figure 7.4.4.6-3.

Figure 7.4.4.6-3: Charging for AC information notification - PEC

1.	The EEC triggers for AC updates with EES (see TS 23.558 [4]).

2.	The EES sends the AC information notification (see TS 23.558 [4]) to the EAS.

2ch-a)	Charging Data Request [Event]: The EES generates charging data related to AC information notification and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

2ch-b)	Create CDR: the CHF stores received information and creates a CDR related to the event.

2ch-c)	Charging Data Response [Event]: The CHF informs the EES on the result of the request.7.4.4.7	Possible solutions for ACR management events subscription

#### 7.4.4.7	Possible solutions for ACR management events subscription charging

Solution #4p: The solution for ACR management events subscription charging based on IEC is illustrated in the Figure 7.4.4.7-1.

Figure 7.4.4.7-1: Charging for ACR management event subscription - IEC

1.	The EAS sends the ACR management event subscribe request (see TS 23.558 [4]) to the EES.

1ch-a)	Charging Data Request [Event]: The EES generates charging data related to ACR management event subscribe request and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

1ch-b)	Create CDR: the CHF stores received information and creates a CDR related to the event.

1ch-c)	Charging Data Response [Event]: The CHF informs the EES on the result of the request.

2.	The EES checks the user plane path management event of the requesting UE with 3GPP Core Network (see TS 23.558 [4]).

3.	The EES may subscribe to UE expected behaviour analytics (UE mobility) from 3GPP Core Network (see TS 23.558 [4]).

4.	The EES sends the ACR management event subscribe response (see TS 23.558 [4]) to the EAS.

Solution #4q: The solution for ACR management event subscription charging based on PEC is illustrated in the Figure 7.4.4.7-2.

Figure 7.4.4.7-2: Charging for ACR management events subscription - PEC

1.	The EAS sends the ACR management event subscribe request (see TS 23.558 [4]) to the EES.

2.	The EES checks the user plane path management event of the requesting UE with 3GPP Core Network (see TS 23.558 [4]).

3.	The EES may subscribe to UE expected behaviour analytics (UE mobility) from 3GPP Core Network (see TS 23.558 [4]).

4.	The EES sends the ACR management event subscribe response (see TS 23.558 [4]) to the EAS.

4ch-a)	Charging Data Request [Event]: The EES generates charging data related to ACR management event subscription and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

4ch-b)	Create CDR: the CHF stores received information and creates a CDR related to the event.

4ch-c)	Charging Data Response [Event]: The CHF informs the EES on the result of the request.

#### 7.4.4.8	Possible solutions for charging for Session with QoS

Solution #4r: for key issue #4b on charging for creation of Session with QoS based on IEC is illustrated in the Figure 7.4.4.8-1.

Figure 7.4.4.8-1: Charging for creation of Session with QoS - IEC

1.	The EAS sends the Session with QoS create request (see TS 23.558 [4]) to the EES.

1ch-a)	Charging Data Request [Event]: The EES generates charging data related to Session with QoS create request and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

1ch-b)	 Create CDR: the CHF stores received information and creates a CDR related to the event.

1ch-c)	Charging Data Response [Event]: The CHF informs the EES on the result of the request.

2.	The EES subscribes to PDU session status monitoring and request data session with specific QoS from 3GPP 5G Core Network (see TS 23.558 [4]).

3.	The EES sends Session with QoS create response (see TS 23.558 [4]) to the EAS.

Solution #4s: for key issue #4b on charging for creation of Session with QoS based on PEC is illustrated in the Figure 7.4.4.8-2.

Figure 7.4.4.8-2: Charging for creation of Session with QoS - PEC

1.	The EAS sends the Session with QoS create request (see TS 23.558 [4]) to the EES.

2.	The EES subscribes to PDU session status monitoring and request data session with specific QoS from 3GPP 5G Core Network (see TS 23.558 [4]).

3.	The EES sends Session with QoS create response (see TS 23.558 [4]) to the EAS.

3ch-a)	Charging Data Request [Event]: The EES generates charging data related to Session with QoS creation and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

3ch-b)	 Create CDR: the CHF stores received information and creates a CDR related to the event.

3ch-c)	Charging Data Response [Event]: The CHF informs the EES on the result of the request.

Solution #4t: for key issue #4b on charging for update of Session with QoS based on IEC is illustrated in the Figure 7.4.4.8-3.

Figure 7.4.4.8-3: Charging for update of Session with QoS - IEC

1.	The EAS sends the Session with QoS update request (see TS 23.558 [4]) to the EES.

1ch-a)	Charging Data Request [Event]: The EES generates charging data related to Session with QoS update request and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

1ch-b)	 Create CDR: the CHF stores received information and creates a CDR related to the event.

1ch-c)	Charging Data Response [Event]: The CHF informs the EES on the result of the request.

2.	The EES updates data session with specific QoS with 3GPP 5G Core Network (see TS 23.558 [4]).

3.	The EES sends Session with QoS update response (see TS 23.558 [4]) to the EAS.

Solution #4u: for key issue #4b on charging for update of Session with QoS based on PEC is illustrated in the Figure 7.4.4.8-4.

Figure 7.4.4.8-4: Charging for update of Session with QoS - PEC

1.	The EAS sends the Session with QoS update request (see TS 23.558 [4]) to the EES.

2.	The EES updates data session with specific QoS with 3GPP 5G Core Network (see TS 23.558 [4]).

3.	The EES sends Session with QoS update response (see TS 23.558 [4]) to the EAS.

3ch-a)	Charging Data Request [Event]: The EES generates charging data related to Session with QoS update and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

3ch-b)	 Create CDR: the CHF stores received information and creates a CDR related to the event.

3ch-c)	Charging Data Response [Event]: The CHF informs the EES on the result of the request.

Solution #4v: for key issue #4b on charging for Session with QoS notification based on PEC is illustrated in the Figure 7.4.4.x-5.

Figure 7.4.4.8-5: Charging for Session with QoS notification - PEC

1.	The EES detects User Plane event for Session with QoS from 3GPP 5G Core Network (see TS 23.558 [4]).

2.	The EES sends Session with QoS notification to EAS (see TS 23.558 [4]).

2ch-a)	Charging Data Request [Event]: The EES generates charging data related to Session with QoS notification and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

2ch-b)	 Create CDR: the CHF stores received information and creates a CDR related to the event.

2ch-c)	Charging Data Response [Event]: The CHF informs the EES on the result of the request.

#### 7.4.4.9	Possible solutions for aggregated charging

The possible solution to support the key issue #4x are based on the following potential converged charging architecture.

Figure 7.4.4.9-1: Converged charging architecture with ECS supporting CEF

In this architecture, the CEF could be integrated with the ECS as depicted in Figure 7.4.4.9-1, the ECS:

-	subscribes notification from the EES to receive required information for charging.

-	aggregate the information for charging received from the EES.

-	generates charging event when the trigger conditions are met.

Alternatively, the CEF could be deployed as standalone functionality and the CEF:

-	subscribes notification from the EES to receive required information for charging.

-	aggregate the information for charging received from the EES.

-	generates charging event when the trigger conditions are met.

Editor’ note: The architecture figure should follow up the decision of the common CEF issues.

The possible solutions to support key issue #4x is illustrated in the Figure 7.4.4.9-2. In this solution, the CEF is integrated with the ECS.

Figure 7.4.4.9-2: Aggregated charging for edge enabling services

- The ECS subscribes notification from the EES to receive required information for charging. The subscription request may include EAS ID and a reporting rule of how a certain measurement shall be reported (e.g., when detecting a service/event had been invoked by the EAS for N times).

- The EAS invokes edge enabling service as defined in clause 7.4.1.

- The EES sends notification to the ECS when the reporting rule is met.

- The ECS aggregates the information for charging (e.g., amount of event) reported by one or multiple EES.

5.ch-a) Charging Data Request [Event]: The ECS generates charging information related to the collected information and sends the charging data request to the CHF when the triggers condition is met.

5.ch-b) Create CDR: The CHF stores received charging information, creates a CDR related to the event.

5.ch-c) Charging Data Response [Event]:  The CHF sends the response to the ECS on the result of the request.

### 7.4.5	Evaluation

#### 7.4.5.1	Evaluation of possible solutions for EAS registration charging

Solution #4a and Solution #4b support EAS registration charging based on IEC and PEC, where the EES interacts with CHF to deduct the ASP’s account balance before or after finishing the EAS registration respectively and both are valid options.

#### 7.4.5.2	Evaluation of possible solutions for EAS discovery charging

Solution #4c and Solution #4d support EAS discovery charging based on IEC and PEC, where the EES interacts with CHF to deduct the ASP’s account balance before or after finishing the EAS registration respectively and both are valid options.

#### 7.4.5.3	Evaluation of possible solutions for EAS application context transfer charging

Solution #4e, Solution #4f and Solution #4g support EAS application context transfer charging based on IEC, PEC and ECUR respectively. Due to the EAS application context transfer involves multiple steps (step 1 to step 4 in Solution #4e, Solution #4f and Solution #4g), the final EAS application context transfer may fail (at step 4) even if the Application Context Relocation Request is accepted by the EAS (at step 2), therefore the Solution #4e based on IEC where CHF charges the Application Context Relocation Request (for step 1) without knowing whether the EAS application context transfer finally succeed or fail (in step 4) is not optimal. The Solution #4f based on PEC supports charging after finishing the EAS application context transfer, and Solution #4g based on ECUR supports quota management and usage reporting for EAS application context transfer, both are valid charging options.

#### 7.4.5.4	Evaluation of possible solutions for UE location obtaining charging

There are various scenarios related to charging for re-exposing the 5GC APIs by EES:

1) 	ECSP and 5GC MNO belong to different enterprises, so the APIs exposed by 5GC and re-exposed by EES need to be charged respectively by 5GC MNO and ESCP;

2) 	ESCP and 5GC MNO belong to the same enterprise, however the APIs exposed by 5GC is not duplicatedly re-exposed by EES to the same consumer (ASP), therefore the APIs exposed by 5GC and re-exposed by EES need to be charged respectively by 5GC MNO and ESCP.

3)	ESCP and 5GC MNO belong to the same enterprise, however the same API is duplicatedly exposed and re-exposed by 5GC and EES to the same consumer (ASP). whether this is a valid scenario and whether an optimized solution is needed to avoid duplicated charging is not addressed in the present technical report.

Solution #4h and Solution #4i support one-time UE location request charging based on IEC and PEC, where the EES interacts with CHF to deduct the ASP’s account balance before or after finishing UE location obtaining respectively and both are valid options for scenarios 1) and 2) described above.

Solution #4j and Solution #4k support UE location subscription charging based on IEC and PEC, where the EES interacts with CHF to deduct the ASP’s account balance before or after finishing UE location subscription respectively and both are valid options for scenarios 1) and 2) described above.

Solution #4l supports UE location notification charging based on PEC, which is a valid option for scenarios 1) and 2) described above.

#### 7.4.5.5	Evaluation of possible solutions for Application Client information subscription and notification charging

Solution #4m and Solution #4n support AC information subscription charging based on IEC and PEC, where the EES interacts with CHF to deduct the ASP’s account balance before or after finishing AC information subscription respectively and both are valid options.

Solution #4o supports AC information notification charging based on PEC, which is a valid option.

#### 7.4.5.6	Evaluation of possible solutions for ACR management events subscription charging

Solution #4p and Solution #4q support ACR management events subscription charging based on IEC and PEC, where the EES interacts with CHF to deduct the ASP’s account balance before or after finishing ACR management events subscription respectively and both are valid options for scenarios 1) and 2) as described in clause 7.4.5.4.

#### 7.4.5.7	Evaluation of possible solutions for Session with QoS charging

Solution #4r and Solution #4s support charging for creation of Session with QoS based on IEC and PEC, where the EES interacts with CHF to deduct the ASP’s account balance before or after finishing the creation of Session with QoS respectively and both are valid options for scenarios 1) and 2) as described in clause 7.4.5.4.

Solution #4t and Solution #4u support charging for update of Session with QoS based on IEC and PEC, where the EES interacts with CHF to deduct the ASP’s account balance before or after finishing the update of Session with QoS respectively and both are valid options for scenarios 1) and 2) as described in clause 7.4.5.4.

Solution #4v supports charging for Session with QoS notification based on PEC, which is a valid option for scenarios 1) and 2) as described in clause 7.4.5.4.

### 7.4.6	Conclusion

#### 7.4.6.1	Conclusion of possible solutions for EAS registration charging

Based on the evaluation in clause 7.4.5.1, both Solution #4a and Solution #4b are feasible and preferrable solutions for EAS registration charging.

#### 7.4.6.2	Conclusion of possible solutions for EAS discovery charging

Based on the evaluation in clause 7.4.5.2, both Solution #4c and Solution #4d are feasible and preferrable solutions for EAS discovery charging.

#### 7.4.6.3	Conclusion of possible solutions for EAS application context transfer charging

Based on the evaluation in clause 7.4.5.3, the Solution #4f and Solution #4g are feasible and preferrable solutions for EAS application context transfer charging.

#### 7.4.6.4	Conclusion of possible solutions for UE location obtaining charging

Based on the evaluation in clause 7.4.5.4, all the Solution #4h, Solution #4i, Solution #4j, Solution #4k and Solution #4l are feasible and preferrable solutions for UE location obtaining charging for scenarios 1) and 2) as described in clause 7.4.5.4.

#### 7.4.6.5	Conclusion of possible solutions for Application Client information subscription and notification charging

Based on the evaluation in clause 7.4.5.5, all the Solution #4m, Solution #4n, and Solution #4o are feasible and preferrable solutions for AC information subscription and notification charging.

#### 7.4.6.6	Conclusion of possible solutions for ACR management events subscription charging

Based on the evaluation in clause 7.4.5.6, both Solution #4p and Solution #4q are feasible and preferrable solutions for ACR management events subscription charging for scenarios 1) and 2) as described in clause 7.4.5.4.

#### 7.4.6.7	Conclusion of possible solutions for Session with QoS charging

Based on the evaluation in clause 7.4.5.7, Based on the evaluation in clause 7.4.5.4, all the Solution #4r, Solution #4s, Solution #4t, Solution #4u and Solution #4v are feasible and preferrable solutions for Session with QoS charging for scenarios 1) and 2) as described in clause 7.4.5.4.

# 8	Conclusions and recommendations

The present technical report described the business roles (see clause 4), concepts (see clause 5) and architecture considerations (see clause 6) for edge computing charging. This technical report also identified and documented the use cases and key issues, derived the corresponding potential requirements, and developed and evaluated the possible solutions (see clause 7) for edge computing charging. This TR made conclusions on the following aspects:

-	5GS capability usage based charging for edge computing, including subscriber charging and inter-provider charging, in clause 7.1.6;

-	Edge enabling infrastructure resource charging, in clause 7.2.6;

-	Edge application server deployment charging, in clause 7.3.6; and

-	Edge enabling services usage charging, in clause 7.4.6.

To support converged charging for edge computing in normative work in 3GPP Rel-17, it is recommended to specify the requirements, architectures and solutions for the aspects mentioned above based on the conclusions of the corresponding solutions documented in clauses 7.1.6, 7.2.6, 7.3.6 and 7.4.6 of this technical report.

Editor’s note: the conclusions are still yet to be completed and revisited before the TR is submitted for approval.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-08 | SA5#132e | - | - | - | - | Initial skeleton | 0.0.0 |
| 2020-08 | SA5#132e | S5-204644 | - | - | - | pCR Add skeleton of draft TR 28.815 | 0.1.0 |
| 2020-10 | SA5#133e | S5-205461 | - | - | - | pCR Add overview for EC charging | 0.2.0 |
| 2020-10 | SA5#133e | S5-205462 | - | - | - | pCR Add architecture considerations on charging aspects of edge computing | 0.2.0 |
| 2020-10 | SA5#133e | S5-205463 | - | - | - | pCR Add charging scenarios and key issues on 5GS charging for EC | 0.2.0 |
| 2020-11 | SA5#134e | S5-206423 | - | - | - | pCR Add charging scenarios and key issues on charging for edge enabling resources and services | 0.3.0 |
| 2020-11 | SA5#134e | S5-206424 | - | - | - | pCR Add key issue on charging of 5GS capabilities supporting EC based on monitored QoS | 0.3.0 |
| 2020-11 | SA5#134e | S5-206425 | - | - | - | pCR Add charging scenarios and key issues on inter-provider charging for 5GS capabilities supporting EC | 0.3.0 |
| 2020-11 | SA5#134e | S5-206426 | - | - | - | pCR Add possible solutions for end-user charging of 5GS capabilities supporting edge computing | 0.3.0 |
| 2020-11 | SA5#134e | S5-206427 | - | - | - | Refinements on architectures | 0.3.0 |
| 2020-11 | SA5#134e | S5-206428 | - | - | - | Introduce use cases and potential requirements Note: the renumbering of clauses proposed in this pCR were not fully implemented due to contradiction to the numbering in other pCRs. | 0.3.0 |
| 2020-11 | SA5#134e | S5-206429 | - | - | - | Introduction of the scope | 0.3.0 |
| 2021-02 | SA5#135e | S5-211415 | - | - | - | Update UCs and requirements and add inter-provider solutions for 5GS charging for EC | 0.4.0 |
| 2021-02 | SA5#135e | S5-211402 | - | - | - | pCR update charging scenarios and key issues on charging for edge enabling services | 0.4.0 |
| 2021-02 | SA5#135e | S5-211425 | - | - | - | pCR Add possible solutions on charging for edge enabling infrastructure resources and services | 0.4.0 |
| 2021-03 | SA5#136e | S5-212464 | - | - | - | pCR Corrections on solutions for inter-provider 5GS usage charging | 0.5.0 |
| 2021-03 | SA5#136e | S5-212503 | - | - | - | pCR Updating performance based charging | 0.5.0 |
| 2021-03 | SA5#136e | S5-212263 | - | - | - | pCR Updating application server deployment charging | 0.5.0 |
| 2021-03 | SA5#136e | S5-212504 | - | - | - | pCR Updating enabling services charging | 0.5.0 |
| 2021-05 | SA5#137e | S5-213597 | - | - | - | pCR Add solutions on charging for obtaining UE location by EAS from EES | 0.6.0 |
| 2021-05 | SA5#137e | S5-213598 | - | - | - | pCR Add solutions on charging for Application Client information subscription and notification | 0.6.0 |
| 2021-05 | SA5#137e | S5-213599 | - | - | - | pCR Add solutions on charging for ACR management event subscription and notification | 0.6.0 |
| 2021-05 | SA5#137e | S5-213600 | - | - | - | pCR Evaluation of solutions for subscriber based charging for 5GS usage supporting EC | 0.6.0 |
| 2021-05 | SA5#137e | S5-213655 | - | - | - | pCR Update use case of charging for edge enabling services | 0.6.0 |
| 2021-05 | SA5#137e | S5-213601 | - | - | - | pCR Correction of subscription for EAS deployment | 0.6.0 |
| 2021-05 | SA5#137e | S5-213602 | - | - | - | pCR Correction of use cases | 0.6.0 |
| 2021-06 | SA#92e | SP-210395 |  |  |  | Presented for information | 1.0.0 |
| 2021-09 | SA5#138e | S5-214679 | - | - | - | pCR 28.815 Add solutions on charging for 5GS usage based on monitored QoS | 1.1.0 |
| 2021-09 | SA5#138e | S5-214680 | - | - | - | pCR 28.815 Add solutions on charging for Session with QoS | 1.1.0 |
| 2021-09 | SA5#138e | S5-214681 | - | - | - | pCR 28.815 Add evaluation and conclusion for edge enabling infrastructure resource charging | 1.1.0 |
| 2021-09 | SA5#138e | S5-214682 | - | - | - | pCR 28.815 Add evaluation and conclusion for edge application server deployment charging | 1.1.0 |
| 2021-09 | SA5#138e | S5-214683 | - | - | - | pCR 28.815 Add evaluation and conclusion for edge enabling services usage charging | 1.1.0 |
| 2021-09 | SA5#138e | S5-214155 | - | - | - | pCR 28.815 Rapporteur's clean-up | 1.1.0 |
| 2021-09 | SA5#138e | S5-214685 | - | - | - | Rel-17 pCR 28.815 Addition of solution for edge application access charging | 1.1.0 |
| 2021-09 | SA5#138e | S5-214711 | - | - | - | pCR Add ECS charging information aggregation and technical solution | 1.1.0 |
| 2021-09 | SA5#138e | S5-214684 | - | - | - | pCR 28.815 Add conclusions and recommendations | 1.1.0 |
| 2021-09 | SA#93e | SP-210855 |  |  |  | Presented for information | 1.2.0 |
| 2021-10 | SA5#139e | S5-215599 | - | - | - | Add solutions on charging for 5GS usage based on monitored QoS | 1.3.0 |
| 2021-10 | SA5#139e | S5-215593 | - | - | - | Add ECS charging information aggregation and technical solution | 1.3.0 |