3GPP TSG-SA5 Meeting #140-e 	S5-216195rev2

- e-meeting, 15 - 24 November 2021

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | 0040 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR 28.313 Add C-SON CCO control information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSON_5G |  |  |  |  |  | Date: |  |  | 2021-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To better support the use case for CCO in clause 6.2.2.4 provided in TS 28.313, the control information is added here. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add C-SON CCO control information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | C-SON CCO does not work very well. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.4, 7.2.3.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS/TR 28.541 CR 0618 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Change

#### 6.4.2.4	Centralized Capacity and Coverage Optimization (CCO)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To optimize the capacity and coverage of NR cells to insure the efficient network resource usage, and optimal end-user experience and performance. |  |
| Actors and Roles | C-SON function to support CCO. |  |
| Telecom resources | - gNB; - The producer of provisioning MnS |  |
| Assumptions | - PM job control and provisioning have been executed to allow C-SON function to receive performance measurements, MDT, RLF, and RCEF reports. |  |
| Pre-conditions | - 5G NR cells are in operation. |  |
| Begins when | The C-SON function has been configured with control information and enabled. |  |
| Step 1 (M) | The C-SON function collects measurements (e.g. distribution of RSRP, RSRQ, …), MDT, RLF, and RCEF reports to monitor the issues (e.g. coverage holes, capacity deficiency, …) for NR cells or beams. |  |
| Step 2 (M) | The C-SON function analyzes the measurements and MDT, RLF, RCEF reports to determine the actions if needed to optimize the NR cells or beams capacity and coverage according to the coverage optimization control policy i.e. adjusting the adjustable parameters within the specified ranges. |  |
| Step 3 (O) | C-SON function consumes the provisioning MnS to re-configure the CCO control parameters. |  |
| Step 4 (O) | The C-SON function collects measurements to evaluate whether the CCO actions have resolved the issues. |  |
| Step 5 (O) | The C-SON function may re-configure or restore the CCO control parameters, if the issues have not been mitigated. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The capacity and coverage of NR cells have been optimized. |  |
| Traceability | REQ-CCO-FUN-1, REQ-CCO-FUN-2, |  |

2nd Change

##### 7.2.3.2.X	Control information

The parameter is used to control the C-SON CCO function.

Table 7.2.3.2.X-1: CCO control

| Control parameter | Definition | Note |
| --- | --- | --- |
| CCO switch | This attribute allows authorized consumer to enable/disable the CCO functionality. See attribute CCOSwitch in TS 28.541 [13]. | This parameter is used when domain centralized SON CCO is supported. |
| CCO policy | It indicates the range of the parameters which can be adjusted to optimize radio coverage. See attribute CCOPolicy in TS 28.541 [13]. | This parameter is used when domain centralized SON CCO is supported. |

End of Changes