3GPP TSG-SA5 Meeting #140-e 	S5-216149

# e-meeting, 15 - 24 November 2021 														Revision of S5-20xxxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.290 | CR | 0171 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of the Threshold based re-authorization triggers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the case of threshold based re-authorization triggers is indicated, the NF Service Consumer seeks re-authorization for the quota when the quota contents fall below the supplied threshold. The continued service processing about whether the service is allowed or not during the re-authorization progress and how to report the usage when the new quota is granted and the remaining quota is used up are unclear. Clarify that the service will continue until the remaining quota is used up. If the re-authorization request is failed, the remaining quota is not calculated based on the Operator’s policy and the usage of remaining quota is not reported.Unless, the remaining quota should be reported again. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the continued service processing after the NF Service Consumer seek the re-authorizatino. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The threshold based re-authorization triggers mechanism is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.4.2	Threshold based re-authorization triggers

The CHF (NF Service Producer) may optionally include an indication to the NF Service Consumer of the remaining quota threshold that shall trigger a quota re-authorization.

If received quota threshold based re-authorization triggers (i.e. timeQuotaThreshold, volumeQuotaThreshold, unitQuotaThreshold), the NF Service Consumer shall seek re-authorization for the quota when the quota contents fall below the supplied threshold. The NF Service Consumer allows the service to continue whilst the re-authorization is progress, until the remaining part had been used up.

When a new quota is granted, the units used during the re-authorization process are treated as the used part of the granted quota.

When a new quota fails to be granted, the NF Service Consumer stops the service, unless the CHF indicates that the remaining part can be used up, the NF Service Consumer allows the service to continue until the remaining part is used up, and then reports the usage information of remaining quota.

End of change